Specifications text

Article number

233697

Type

MEB-55M2812M0A

Short description

1/2.8" HD Camera, Day&Night, 2592x1944, Infrared 2,7-13,5mm, 12VDC, IP67

Main Features

1/2.8" CMOS Image Sensor

Resolution max. 2592x1944

multi-signal: HD-TVI, AHD, CVI, CVBS

2,7-13,5 mm lens with variable focal length (motorized)

Day/night switching (ICR)

2D+3D DNR (Digital Noise Reduction)

Automatic exposure control (1/30s~1/50,000s)

Lens shading correction

IR LED illumination integrated

For outdoor use degree of protection IP67

12VDC power supply, NDAA compliant

Other Features (Specifications)

| EAN/GTIN | 4260229840320 |
| --- | --- |
| Audio support | No |
| Recording sensor | CMOS |
| Lighting | Infrared |
| Motion detector | No |
| Horizontal angle of view | 110,6 - 32,6° |
| Focal length | 2,7 - 13,5 mm |
| Chip size | 1/2,8" |
| Digital Noise Reduction (DNR) | Yes |
| Dimensions (WxHxD) (extended) | 83,88x87,4x242mm |
| Illumination range | ca. 35m |
| Remark | NDAA compliance under Section 889 |
| Watch (62px/m) | 243 m |
| Aperture Range (F) | F1.8~F3.3 |
| External connections | 12VDC power input,video (BNC) |
| Flickerless function | Yes |
| Identify (250px/m) | 52 m |
| Camera shape | Bullet |
| Scope of delivery | manual,bag of screws |
| Menu languages | Chinese, English, French, Italian, Japanese, Polish, Portuguese, Russian, Spanish |
| Multicast | No |
| Signal-to-noise ratio | 50 dB |
| Signal Format | AHD @ 2MP, AHD @ 2MP, AHD @ 4MP, AHD @ 5MP, Composite (FBAS), CVI @ 2MP, CVI @ 4MP, HD-TVI @ 2MP, HD-TVI @ 2MP, HD-TVI @ 2MP, HD-TVI @ 2MP, HD-TVI @ 4MP, HD-TVI @ 5MP |
| Sunshade canopy | Yes |
| Day/night mode | Automatic,Day,Night |
| Text overlay | available |
| Perceiving (25px/m) | 625 m |
| Integrated wall mount | Yes |
| White balance | Manual,Automatic |
| Backlight compensation | No |
| Housing type | Outside |
| Housing material | Aluminium |
| Heating | Yes |
| Photosensitivity | 0,05 Lux bei F1,2 |
| Low Speed Shutter | No |
| Fan | No |
| Mounting type | Ceiling, Wall |
| Lens Type | Varifocal (motorized) |
| Manufacturer order number | 233697 |
| Protection | IP67 |
| Series | eneo IN-Serie,eneo COAXIZE |
| Control Interfaces | Coaxitron (HD Protocol) |
| Power supply | 12VDC |
| System | True Day&Night |
| Day/night switching | Switchable IR Filter (ICR) |
| Temperature Range (Operating) | -30°C ~ +55°C |
| Vandalism-resistant | No |
| Wide Dynamic Range (WDR) | WDR 120dB |
| Zoomfactor (optical) | 4 |
| Brand | eneo |

Additional Productinformation

Related products

| AK-20 | Camera connection box for Wall and ceiling mounting, IN Series Bullet Cameras |
| --- | --- |
| AK-21 | Camera connection box for Wall and ceiling mounting, IN Series Bullet Cameras |
| AK-24 | Dual camera connection box for selected Bullet or Fix Dome IN-Series cameras |
| MAM-5MM2203M0A | Converter, 4K HD-TVI, HD-CVI, AHD, auf HDMI, Indoor |
| PM-21 | Pole mounting adapter for camera connection box AK-20 and AK-21 |

Recommendations

Services