Specifications text

Article number

234978

Type

ISM-32F0036M00

Short description

1/2,8" Mini Network Modul, 1920x1080, Day&Night, 3DNR, 3,6mm, 12VDC, PoE, Indoor

Main Features

Sony STARVIS 2MP Image Sensor

Resolution max. 1920x1080 (Full HD)

Video Compression Format: H.264/MJPEG

Motion Detection

Built in SD micro slot

Low Speed Shutter

2DNR, 3DNR and WDR support

PoE Cable

Lens 3.6mm (further lenses upon request)

Power Supply 12VDC (1.8W) or PoE (IEEE802.3af)

Operation Temperature -10°C - +50°C

Other Features (Specifications)

| Audio support | No |
| --- | --- |
| Resolution standard | 0 MP |
| Recording sensor | CMOS |
| Lighting | no |
| Motion detector | No |
| Image resolution max. | 1920x1080 |
| Image transfer rate max. | 30 fps |
| Horizontal angle of view | 92° |
| Focal length | 3,6 mm |
| Chip size | 1/2,8" |
| Digital Noise Reduction (DNR) | Yes |
| Dimensions (WxHxD) (extended) | 35x35mm |
| Alarm processing | FTP |
| Resolution (extended) | 1920x1080 (16:9),1280x720 (16:9),800x600 (4:3),704x480 (4:3),640x480 (4:3),320x240 (4:3) |
| Remark | To guarantee you an optimum user experience,we recommend that you only use components of the same brand in a system. If several brands are used in combination,eneo cannot guarantee functionality. If combined use is necessary,we offer a compatibility check |
| Watch (62px/m) | 22 m |
| Browser | Edge: MJPEG, Mozilla: MJPEG, MS Internet Explorer from Vers. 9.0 or higher, Safari: MJPEG |
| Data stream types | 2x H.264,1x MJPEG simultan |
| Detect (125px/m) | 3.6 m |
| External connections | RJ-45,connection cable approx.15cm |
| Flickerless function | Yes |
| Identify (250px/m) | 3.6 m |
| Camera shape | Module |
| Scope of delivery | manual,bag of screws,mounting bracket |
| Menu languages | English |
| Multicast | No |
| Password | Administrator and User |
| Signal-to-noise ratio | 50 dB |
| Sunshade canopy | No |
| Day/night mode | Automatic,Day,Night |
| Text overlay | available |
| Perceiving (25px/m) | 56.6 m |
| Integrated wall mount | No |
| White balance | Indoor,Outdoor,User defined,Push |
| Backlight compensation | No |
| Housing type | Inside |
| Housing material | Metal |
| Heating | Yes |
| Internal memory | microSD card slot |
| Camera Type | Board Camera |
| Photosensitivity | 0,1 Lux bei F2,5,30 IRE |
| Low Speed Shutter | No |
| Mounting type | Ceiling, Wall |
| Manufacturer order number | 234978 |
| Power over Ethernet | IEEE 802.3af |
| Control Interfaces | Ethernet |
| Power supply | 12VDC |
| System | Electronic Day&Night |
| Day/night switching | Electronic |
| Temperature Range (Operating) | -10°C ~ +50°C |
| Vandalism-resistant | No |
| Video Compression | H.264, M-JPEG |
| Wide Dynamic Range (WDR) | WDR 100dB |
| Wiper | No |
| Certifications | IK08 |
| Brand | eneo |

Additional Productinformation

Related products

Recommendations

Services