Specifications text

Article number

234978

Type

ISM-32F0036M00

Short description

1/2,8" Mini Network Modul, 1920x1080, Day&Night, 3DNR, 3,6mm, 12VDC, PoE, Indoor

Main Features

Sony STARVIS 2MP Image Sensor

Resolution max. 1920x1080 (Full HD)

Video Compression Format: H.264/MJPEG

Motion Detection

Built in SD micro slot

Low Speed Shutter

2DNR, 3DNR and WDR support

PoE Cable

Lens 3.6mm (further lenses upon request)

Power Supply 12VDC (1.8W) or PoE (IEEE802.3af)

Operation Temperature -10°C - +50°C

Other Features (Specifications)

| Dimensions (HxWxD) | 35x35mm |
| --- | --- |
| Camera Type | Board Camera |
| Resolution Standard | HD 1080p |
| System | electronic Day&Night |
| Sensor size | 1/2.8" |
| Imager | CMOS |
| Video compression | H.264, M-JPEG |
| Image resolution max. | 1920x1080 |
| Image Transmission Rate | 30 fps |
| Day&Night switch | Electronic |
| Sensitivity | 0,1 Lux at F2,5, 30 IRE |
| Low Speed Shutter | yes |
| Digital Noise Reduction | yes |
| Backlight Compensation | BLC, HLC |
| Wide Dynamic Range (WDR) | WDR 100dB |
| Motion Detector | yes |
| Focal Length | 3,6 mm |
| Iris Control | no |
| Digital Zoom | no |
| Observe (62 px/m) | 22 m |
| Recognize (125 px/m) | 9,9 m |
| Identify (250 px/m) | 3,6 m |
| Detect (25px/m) | 56,6 m |
| Horizontal Angle of View | 92° |
| Vertical Angle of View | 49° |
| llumination | no |
| Housing | Indoor |
| Color (housing) | black |
| Hosuing material | Metal |
| Blower | no |
| Alarm inputs | nein |
| Alarm outputs | no |
| Audio support | no |
| Ethernet interface | 100Base-TX |
| Serial interfaces | Ethernet |
| Internal storage | microSD-Kartenslot |
| Analysis functions | Motion detection |
| Mounting | Ceiling mount, Wall mount |
| Supply voltage | 12VDC |
| Power consumption | 1.8W |
| Power over Ethernet | IEEE 802.3af |
| Vandal proof | no |
| Operating temperature | -10°C ~ +50°C |
| Certifications | IK08 |
| Camera type | Modul |
| Weight | 200g |
| Manufacturer order number | 234978 |
| Brand | eneo |

Additional Productinformation

Related products

| eneo Site Manager | Management Software for eneo IP products |
| --- | --- |
| IAD-5SG1008MUA | Gigabit Switch, Unmanaged, 120W, 8x PoE, 1xSFP, 1x RJ45, 10/100/1000 Mbps, Desktop |

Recommendations

Services