Specifications text

Article number

234978

Type

eneo ISM-32F0036M00

Short description

1/2,8" Mini Network Modul, 1920x1080, Day&Night, 3DNR, 3,6mm, 12VDC, PoE, Indoor

Main Features

Sony STARVIS 2MP Image Sensor

Resolution max. 1920x1080 (Full HD)

Video Compression Format: H.264/MJPEG

Motion Detection

Built in SD micro slot

Low Speed Shutter

2DNR, 3DNR and WDR support

PoE Cable

Lens 3.6mm (further lenses upon request)

Power Supply 12VDC (1.8W) or PoE (IEEE802.3af)

Operation Temperature -10°C - +50°C

Other Features (Specifications)

| General |  |
| --- | --- |
| Lighting | No |
| Dimensions (WxHxD) (extended) | 35x35mm |
| Weight | 0.2 kg |
| Text overlay | available |
| Color (Housing) | black |
| Housing type | Inside |
| Housing material | Metal |
| Camera Type | Board Camera |
| Power consumption | 1.8 W |
| Mounting type | Ceiling, Wall |
| Power over Ethernet | IEEE 802.3af |
| Power supply | 12VDC |
| Temperature Range (Operating) | -10°C ~ +50°C |
| Wide Dynamic Range (WDR) | WDR 100dB |
| Certifications | IK08 |
| Camera |  |
| Backlight compensation | Yes |
| Low Speed Shutter | Yes |
| Resolution standard | 2 MP |
| Recording sensor | CMOS |
| Chip size | 1/2,8" |
| Flickerless function | Yes |
| Camera shape | Module |
| Signal-to-noise ratio | 50 dB |
| Day/night mode | Automatic,Day,Night |
| White balance | Indoor,Outdoor,User defined,Push |
| Photosensitivity | 0,1 Lux bei F2,5,30 IRE |
| System | Electronic Day&Night |
| Day/night switching | Electronic |
| Display |  |
| Image resolution max. | 1920x1080 |
| DORI |  |
| Watch (62px/m) | 22 m |
| Detect (125px/m) | 9,9 m |
| Identify (250px/m) | 3,6 m |
| Perceiving (25px/m) | 56,6 m |
| Functions |  |
| Audio support | No |
| Motion detector | Yes |
| Digital Noise Reduction (DNR) | Yes |
| Analysis | Motion Detection |
| Alarm processing | FTP |
| Browser | Edge: MJPEG, Mozilla: MJPEG, MS Internet Explorer from Vers. 9.0 or higher, Safari: MJPEG |
| Menu languages | English |
| Password | Administrator and User |
| Internal memory | microSD card slot |
| Notes |  |
| Remark | To guarantee you an optimum user experience,we recommend that you only use components of the same brand in a system. If several brands are used in combination,eneo cannot guarantee functionality. If combined use is necessary,we offer a compatibility check |
| Interfaces / Inputs/ Outputs |  |
| External connections | RJ-45,connection cable approx.15cm |
| Control Interfaces | Ethernet |
| Parts supplied |  |
| Scope of delivery | manual,bag of screws,mounting bracket |
| Lens |  |
| Horizontal angle of view | 92° |
| Vertical angle of view | 49° |
| Iris Control | No |
| Focal length | 3,6 mm |
| Video |  |
| Image transfer rate max. | 30 fps |
| Resolution (extended) | 1920x1080 (16:9),1280x720 (16:9),800x600 (4:3),704x480 (4:3),640x480 (4:3),320x240 (4:3) |
| Data stream types | 2x H.264,1x MJPEG simultan |
| Video Compression | H.264, M-JPEG |
| Product labeling: |  |
| Manufacturer order number | 234978 |
| Brand | eneo |

Additional Productinformation

Related products

Recommendations

Services