Specifications text

Article number

236322

Type

INP-72A0018MIA

Short description

Netzwerk Dome, PTZ, 18x 1920x1080, Day/Night, WDR, AI Alarm, Audio, 12VDC, PoE+, IP6

Main Features

1/2.7" SmartSens CMOS sensor

Maximum resolution 1920x1080 pixels, 30fps

AI analysis functions including object classification

WDR 120dB, 3D-DNR noise reduction

Motorised autofocus zoom lens 18x 5.0-90mm

Smart Codec H.265/H.264, multi-streaming

Browser independent (Edge, Safari, Firefox, Chrome)

Supports ONVIF profiles S/T/G

Local storage via Micro SD card

Audio & alarm input and output

NDAA compliance

IR LED illumination, IP66

Other Features (Specifications)

| EAN/GTIN | 4260229840535 |
| --- | --- |
| Dimensions (HxWxD) | 270x160mm |
| Series | eneo IN-Series |
| Camera Type | Face recognition, License plate recognition, People Count, Retail Intelligence |
| Resolution Standard | HD 1080p |
| System | True Day&Night |
| Sensor size | 1/2.7" |
| Imager | CMOS |
| Video compression | H.264, H.264+ (Smart Stream), H.265, H.265+ (Smart Stream), M-JPEG |
| Image resolution max. | 1920x1080 |
| Image Transmission Rate | 30 fps |
| Day&Night switch | ICR |
| Sensitivity | 0,0003 Lux at F1,6 |
| Low Speed Shutter | yes |
| Digital Noise Reduction | yes |
| Backlight Compensation | BLC, HLC |
| Wide Dynamic Range (WDR) | WDR 120dB |
| Motion Detector | yes |
| Lens Type | auto focus zoom |
| Focal Length | 5 - 90 mm |
| Iris Control | DC |
| Zoom Factor | 18x |
| Observe (62 px/m) | 29,4 - 518,8 m |
| Recognize (125 px/m) | 14,4 - 258 m |
| Identify (250 px/m) | 7,2 - 127 m |
| Detect (25px/m) | 73,9 - 1260 m |
| Horizontal Angle of View | 53,3 - 3,8° |
| Vertical Angle of View | 30,7 - 2,2° |
| llumination | Infrared |
| Housing | Outdoor |
| Color (housing) | black, Pantone P 179-1, RAL9003 |
| Hosuing material | Aluminium |
| Heater | no |
| Blower | no |
| Rotation range | 360° |
| Speed (manual) | 80°/Sek. |
| Speed (preset) | 120°/Sek. |
| Alarm inputs | 1 |
| Alarm outputs | 1 |
| Audio support | yes |
| Ethernet interface | 100Base-TX, 10Base-T |
| Serial interfaces | Ethernet |
| Internal storage | microSD-Kartenslot |
| Analysis functions | Attribute Detection (AD), Cross Counting (CC), Crowd Density Detection (CD), Face Attendance (FA), Face Detection (FD), Face Recogntion (FR), Heat Map (HM), Human&Vehicle Detection (PD&VD), License Plate Detection (LPD), License Plate Recognition (LPR), Line Crossing Detection (LCD), Perimeter Intrusion Detection (PID), Queue Lenght Detection (QD), Rare Sound Detection (RSD), Stationary Object Detection (SOD) |
| Mounting | Wall mount |
| Supply voltage | 12VDC, PoE+ |
| Power consumption | 22W |
| Power over Ethernet | IEEE 802.3at |
| Vandal proof | no |
| Operating temperature | -30°C ~ +60°C, @ Humidity 90% or less (non-condensing) |
| ONVIF | ONVIF Profile G, ONVIF Profile S, ONVIF Profile T |
| Protection rating | IP66 |
| Camera type | PTZ |
| Audio | 1/1 Line input and output, 2 Way Audio |
| Remote configuration | Browser |
| Illumination range | approx. 100m |
| Weight | 2.5kg |
| Manufacturer order number | 236322 |
| Brand | eneo |

Additional Productinformation

Related products

| CM-22 | Corner mounting adapter for wall bracket WB-24 and PTZ dome camera INP-75A0033MIA |
| --- | --- |
| eneo INsight Installation Tool | Supports adjustments for cameras with AI features for IN-series |
| eneo Site Manager | Management Software for eneo IP products |
| KBD-3NET | Network keyboard with joystick, supports ONVIF protocol, RJ45, 12VDC |
| PM-21 | Pole mounting adapter for camera connection box AK-20 and AK-21 |

Recommendations

Services

| Vorkonfiguration IP-Kamera | Preconfiguration 1x IP camera Function test, FW update |
| --- | --- |