Specifications text

Article number

236318

Type

IND-32F0016P0A

Short description

Network Dome Fix, 360° 4000x3000, Day/Night, DWDR, 1,6mm, IR, 12VDC, PoE, IP66

Main Features

1/2.3" Omnivision CMOS sensor

1.6mm fixed focal length FishEye lens

360° panoramic, quad, panomorphic view,ceiling, wall

Integration with eneo INsight software & App

Day/night function, 3D-DNR, slow speed shutter

Resolution max. 4000x3000@15fps

Smart Codec H.265/H.264, Multi-Streaming

Triple stream (main, sub, mobile)

Browser independent (Edge, Safari, Firefox, Chrome)

IR-LED illumination, microphone and loudspeaker integrated

NDAA compliance

For indoor and outdoor use, IP66

Other Features (Specifications)

| EAN/GTIN | 4260229840573 |
| --- | --- |
| Audio support | Yes |
| Digital Noise Reduction (DNR) | Yes |
| Multicast | Yes |
| Heating | Yes |
| Alarm inputs | 1 |
| Resolution standard | 12 MP |
| Recording sensor | CMOS, OmmiVision OV12895-J94A |
| Lighting | Infrared |
| Image resolution max. | 4000x3000 |
| Image transfer rate max. | 15 fps |
| Horizontal angle of view | 360° |
| Focal length | 1,6 mm |
| Chip size | 1/2,3" |
| Dimensions (WxHxD) (extended) | 120x55mm |
| Alarm processing | recording internal storage (SD card),E-Mail,HTTP,FTP,TCP,UDP,alarm out,Siren,Cloud Upload |
| Audio | 93±3dB max., Built-in microphone, Built-in speaker |
| Resolution (extended) | 4000x3000,3000x3000,2560x2560,2160x2160,1520x1520,1280x960,1080x1080,720x720,640x480 |
| Illumination range | Approx. 10m |
| Remark | From firmware version 8.2.4.1,the analysis function Object Classification Human / Vehicle (PD&VD) is available |
| Watch (62px/m) | 14.9 m |
| Picture size | 360° panorama |
| Browser | Edge, Firefox, Google Chrome, Safari |
| Data stream types | Triple-Stream(Main/Sub/Mobile) |
| Detect (125px/m) | 6.5 m |
| ESC | 1/5 ~ 1/20.000sec. |
| External connections | RJ-45,Alarm in/outputs,12VDC input (DC plug),micro SD card slot 256 GB |
| Flickerless function | Yes |
| Identify (250px/m) | 3.8 m |
| Camera shape | Minsters |
| Scope of delivery | manual,bag of screws |
| Manual Electric Shutter (MES) | 1-5~1-20,000sec. |
| Menu languages | English, French, German, Italian, Japanese, Portuguese, Russian, Spanish, Turkish |
| Password | Administrator and User |
| Privacy Zone | Areas-ON/OFF switchable |
| SDK | upon request |
| Signal-to-noise ratio | 39 dB |
| Day/night mode | Automatic,Day,Night,Image,Time |
| Text overlay | Switchable on/off |
| Perceiving (25px/m) | 38 m |
| White balance | Automatic,Manual |
| Color (Housing) | RAL9003,Pantone P 179-1 |
| Housing type | Outside |
| Housing material | Metal, Plastic |
| Internal memory | microSD card slot |
| Camera Type | Fisheye Camera |
| Photosensitivity | 0,01 Lux bei F1,6 |
| MOD (Minimum Object Distance) | 0.3 m |
| Mounting type | Ceiling, Wall |
| Network Protocols | DDNS, DHCP, DNA, FTP, HTTP, HTTPS, NTP, RTP, RTSP, SMTP, TCP-IP, UPnP |
| Lens Type | Prime lens |
| ONVIF | ONVIF Profile G, ONVIF Profile M, ONVIF Profile S, ONVIF Profile T |
| Manufacturer order number | 236318 |
| Power over Ethernet | IEEE 802.3af |
| Protection | IP66 |
| Series | eneo IN-Serie |
| Control Interfaces | Ethernet |
| Power supply | 12VDC, Poe |
| System | True Day&Night |
| Day/night switching | Switchable IR Filter (ICR) |
| Temperature Range (Operating) | -35°C ~ +60°C,@ Luftfeuchtigkeit 90% oder weniger (nicht kondensierend) |
| Video Compression | H.264, H.264+, H.265, H.265+, M-JPEG |
| Wide Dynamic Range (WDR) | D-WDR |
| Brand | eneo |

Additional Productinformation

Related products

| AK-27 | Camera connection box Fisheye for ceiling tube DOME-CH6 and wall mount DOME-WB6, Indoor |
| --- | --- |
| AK-28 | Camera connection box for wall and ceiling mount, for Fisheye IND-32F0016P0A, Indoor |
| eneo Site Manager | Management Software for eneo IP products |

Recommendations

| AK-27 | Camera connection box Fisheye for ceiling tube DOME-CH6 and wall mount DOME-WB6, Indoor |
| --- | --- |
| AK-28 | Camera connection box for wall and ceiling mount, for Fisheye IND-32F0016P0A, Indoor |

Services

| Vorkonfiguration IP-Kamera | Preconfiguration 1x IP camera Function test, FW update |
| --- | --- |