Specifications text

Article number

236326

Type

INP-54M2812M0A

Short description

Network Dome, PTZ, 4x MVF, 2560x1440, Day/Night, WDR, A Audio, 12VDC, PoE, IP66, IK10

Main Features

1/1.8" High Sensitivity SmartSens CMOS sensor,

Maximum resolution 2560x1440 pixels, 60fps

AI analysis functions including object classification

WDR 120dB, 3D-DNR noise reduction

Motorised varifocal lens 2.8-12mm

Smart Codec H.265/H.264, multi-streaming

Browser independent (Edge, Safari, Firefox, Chrome)

Supports ONVIF profiles S/T/G/M

Local storage via Micro SD card

Built-in loudspeaker and microphone

NDAA compliance

IR and white light LED illumination, IP66, IK10

Other Features (Specifications)

| General |  |
| --- | --- |
| Vandalism-resistant | Yes |
| Lighting | Infrared, White |
| Dimensions (WxHxD) (extended) | 153x118,8mm |
| Illumination range | Approx. 20m |
| ESC | 1/5 ~ 1/20.000sec. |
| SDK | upon request |
| Text overlay | Switchable on/off |
| Color (Housing) | RAL9003,Pantone P 179-1 |
| Housing type | Outside |
| Housing material | Aluminium |
| Camera Type | Face, License plate recognition, People counting, Retail Intelligence |
| Mounting type | Ceiling, Wall |
| ONVIF | ONVIF Profile G, ONVIF Profile M, ONVIF Profile S, ONVIF Profile T |
| Power over Ethernet | IEEE 802.3af |
| Protection | IP66 |
| Power supply | 12VDC, Poe |
| Temperature Range (Operating) | -30°C ~ +60°C,@ Luftfeuchtigkeit 90% oder weniger (nicht kondensierend) |
| Wide Dynamic Range (WDR) | WDR 120dB |
| Certifications | IK10 |
| Zoomfactor (optical) | 4 |
| Camera |  |
| Backlight compensation | Yes |
| Low Speed Shutter | Yes |
| Resolution standard | 4 MP |
| Recording sensor | CMOS, SC4210 |
| Chip size | 1/1,8" |
| Flickerless function | Yes |
| Camera shape | PTZ |
| Manual Electric Shutter (MES) | 1-5~1-20,000sec. |
| Signal-to-noise ratio | 49.2 dB |
| Day/night mode | Automatic,Day,Time |
| White balance | Automatic,Manual |
| Photosensitivity | 0,001 Lux bei F1,6 |
| System | True Day&Night |
| Day/night switching | Switchable IR Filter (ICR) |
| Display |  |
| Image resolution max. | 2560x1440 |
| DORI |  |
| Watch (62px/m) | 13.8 m |
| Detect (125px/m) | 5.5 m |
| Perceiving (25px/m) | 36.7 m |
| Functions |  |
| Audio support | Yes |
| Motion detector | Yes |
| Digital Noise Reduction (DNR) | Yes |
| Multicast | Yes |
| Turning range | 350 |
| Alarm processing | recording internal storage (SD card),E-Mail,HTTP,FTP,TCP,UDP,Siren,Strobe light,Cloud Upload |
| Audio | 93±3dB max., Built-in microphone, Built-in speaker |
| Browser | Edge, Firefox, Google Chrome, Safari |
| Menu languages | English, French, German, Italian, Japanese, Portuguese, Russian, Spanish, Turkish |
| Password | Administrator and User |
| Privacy Zone | Areas-ON/OFF switchable |
| Tours | no |
| Speed (manual control) | 50 °/sec |
| Speed (Target Approach) | 50 °/sec |
| Internal memory | microSD card slot |
| Notes |  |
| Remark | NDAA compliance under Section 889,The face recognition functions are only available with an inserted SD card or in conjunction with a recorder of the eneo IN series.,To guarantee you an optimum user experience,we recommend that you only use components of |
| Interfaces / Inputs/ Outputs |  |
| External connections | RJ-45,12VDC input (DC plug),connection cable approx.45cm,micro SD card slot 256 GB |
| Control Interfaces | Ethernet |
| Parts supplied |  |
| Scope of delivery | manual,bag of screws |
| Network |  |
| Network Protocols | DDNS, DHCP, DNA, FTP, HTTP, HTTPS, NTP, RTP, RTSP, SMTP, TCP-IP, UPnP |
| Lens |  |
| Horizontal angle of view | 114,6 - 48,3° |
| Focal length | 2,8 - 12 mm |
| Aperture Range (F) | F1.3 |
| MOD (Minimum Object Distance) | 1.75 m |
| Lens Type | Varifocal (motorized) |
| Video |  |
| Image transfer rate max. | 60 fps |
| Resolution (extended) | 2560x1440 (16:9),2304x1296 (16:9),1920x1080 (16:9),1280x720 (16:9),640x480 (4:3),352:240 (4:3) |
| Picture size | 16:9, 4:3 |
| Data stream types | Triple-Stream(Main/Sub/Mobile) |
| Video Compression | H.264, H.264+, H.265, H.265+ |
| Product labeling: |  |
| EAN/GTIN | 4260229840528 |
| Manufacturer order number | 236326 |
| Series | eneo IN-Serie |
| Brand | eneo |

Additional Productinformation

Related products

| AK-29 | Camera connection box for PTZ Dome INP-54M2812M0A |
| --- | --- |
| DOME-WB8 | Wall bracket for PTZ dome INP-54M2812M0A and IND-5 Dome Series |
| eneo INsight | Control software for recorder IN- and MN-series |
| eneo INsight Installation Tool | Supports adjustments for cameras with AI features for IN-series |
| eneo INsight mobile | Mobile App for recorder IN- and MN-series |
| eneo Site Manager | Management Software for eneo IP products |
| KBD-3NET | Network keyboard with joystick, supports ONVIF protocol, RJ45, 12VDC |

Recommendations

| jetpower-PSU-XS-11C Steckernetzteil |  |
| --- | --- |

Services

| LACKIERUNG Fix-Dome-Kamera | Extra charge for special paint fixed dome in RAL color |
| --- | --- |
| LACKIERUNG Grundkosten | Painting basic costs once per RAL color and delivery lot |
| Vorkonfiguration IP-Kamera | Preconfiguration 1x IP camera Function test, FW update |