Specifications text

Article number

237908

Type

IAM-5SE1004IUP

Short description

Gigabit Switch, Unmanaged 120W, 4x PoE, Industrial Design -20°C~+65°C, PD port

Main Features

Gigabit PoE Switch, DIN Rail

4x 10/100/1000 PoE ports, IEEE 802.3af/at/bt

Maximum 90W power budget

1 x SFP Gigabit uplink port

1 x RJ45 Gigabit uplink port, PD-capable (90W PoE input)

CCTV mode, extended transmission distance

1 x output relay

Overvoltage protection IEC61000-4-5 (6KV)

Temperature range -20°C ~ +65°C

Other Features (Specifications)

| Number of PoE ports | 4 |
| --- | --- |
| Design | DIN Rail Module |
| Dimensions (WxHxD) (extended) | 44x110x160mm |
| Remark | Power supply unit is not included in the scope of delivery,To achieve the specified PoE budget (total) value,please use suitable power supplies or PoE injector.PoE budget: 90W max. for a single PoE port; 75W max. for all PoE ports when supplied via PD por |
| Application environment | inside |
| ESD protection | IEC61000-4-2 |
| Weight | 540 kg |
| Scope of delivery | "manual,power cord |
| Eneo _netzwerkstandards | IEEE 802.1D Spanning Tree Protocol, IEEE 802.3 10BaseT, IEEE 802.3u 100BaseTX, IEEE 802.3x Flow Control |
| Internal Switching Capacity (Gbit) | 12 Gbps |
| Internal Switching Capacity (MP per second) | 8.9 Mpps |
| Ethernet interface | 1000Base-T, 100Base-TX, 10Base-T |
| Type | Unmanaged |
| Power consumption | 130 W |
| Mounting type | DIN Rail |
| Manufacturer order number | 237908 |
| PoE Budget (Total) | 120 W |
| Ports | 1x 10/100/1000Base-T (PoE-PD), 1x 1000Base-X SFP, 4x 10/100/1000Base-T |
| Power over Ethernet | IEEE 802.3af, IEEE 802.3at, IEEE 802.3bt |
| Series | eneo Switch |
| Power supply | 48-56VDC, Poe |
| Temperature Range (Operating) | -40°C ~ +65°C |
| Brand | eneo |

Additional Productinformation

Related products

Recommendations

Services