Specifications text

Article number

237908

Type

IAM-5SE1004IUP

Short description

Gigabit Switch, Unmanaged 120W, 4x PoE, Industrial Design -20°C~+65°C, PD port

Main Features

Gigabit PoE Switch, DIN Rail

4x 10/100/1000 PoE ports, IEEE 802.3af/at/bt

Maximum 90W power budget

1 x SFP Gigabit uplink port

1 x RJ45 Gigabit uplink port, PD-capable (90W PoE input)

CCTV mode, extended transmission distance

1 x output relay

Overvoltage protection IEC61000-4-5 (6KV)

Temperature range -20°C ~ +65°C

Other Features (Specifications)

| Dimensions (HxWxD) | 44x110x160mm |
| --- | --- |
| Type | Unmanaged |
| Series | eneo Switch |
| Mounting | DIN rail |
| Blower | no |
| Ethernet interface | 1000Base-T, 100Base-TX, 10Base-T |
| VLAN | no |
| Ports | 1x 10/100/1000Base-T (PoE-PD), 1x 1000Base-X SFP, 4x 10/100/1000Base-T |
| PoE ports (number) | 4 |
| Management | no |
| Model | DIN rail module |
| Supply voltage | 48~57 VDC, PoE |
| Power consumption | 130W |
| Power over Ethernet | IEEE 802.3af, IEEE 802.3at, IEEE 802.3bt |
| PoE budget | 120 W |
| Operating temperature | -40°C ~ +65°C |
| Weight | 540g |
| Manufacturer order number | 237908 |
| Brand | eneo |

Additional Productinformation

Related products

| SFP-0850M10 | SFP Modul, 1.25Gbps, 550m, Multimode, 850nm, Duplex LC Connector, 0°~+70°C |
| --- | --- |
| SFP-0850M40 | SFP Modul, 1,25Gbps, 550m, Multimode, 850nm, Duplex LC Connector, -40° ~ +70°C |
| SFP-1310M40 | SFP Modul, 1.25Gbps, 550m, Multimode, 1310nm, Duplex LC Connector, -40°~+75°C |
| SFP-1310S10 | SFP Modul, 1.25Gbps, 20km, Singlemode, 1310nm, Duplex LC Connector, 0°~+70°C |
| SFP-1310S40 | SFP Modul, 1.25Gbps, 20km, Singlemode, 1310nm, Duplex LC Connector -40°~+75°C |

Recommendations

Services