Specifications text

Article number

238675

Type

Solvido SOL-VICAM-MOBILE-LTE

Short description

INB-68M2812MFA, Junction box 3840 x 2160, 24h Farbe, IR, LTE, Varifocal lens 2,8 - 12 mm, WDR

Main Features

Portable video surveillance incl. connection cable

1/2,8" Sony STARVIS CMOS-Sensor, 8MP/30fps

Plug & Play

Incl. 5m power supply cable with device connector

Dimensions: 300 x 200 x 400 mm

Wireless connectivity with LTE

Applicable for construction sites

Effortless mounting/demounting

Remote access with QR-Code

INB-68M2812MFA

SIM card not included

Antenna does not support WLAN

Other Features (Specifications)

| General |  |
| --- | --- |
| Heating | No |
| Lighting | Infrared, White |
| Housing type | Outside |
| Housing material | Aluminium |
| Camera Type | Face, Intrusion Detection, People counting, Retail Intelligence |
| ONVIF | ONVIF Profile G, ONVIF Profile S, ONVIF Profile T |
| Protection | IP66 |
| Power supply | 230VAC |
| Temperature Range (Operating) | -30°C ~ +50°C |
| Wide Dynamic Range (WDR) | 120dB |
| Camera |  |
| Backlight compensation | Yes |
| Low Speed Shutter | Yes |
| Resolution standard | 8 MP |
| Recording sensor | CMOS |
| Chip size | 1/2,8" |
| Photosensitivity | 0,002 Lux bei F1,0 |
| System | 24h Color |
| Day/night switching | Switchable IR Filter (ICR) |
| Display |  |
| Image resolution max. | 3840x2160 |
| Functions |  |
| Audio support | Yes |
| Motion detector | Yes |
| Digital Noise Reduction (DNR) | Yes |
| Internal memory | microSDHC card, microSDXC card |
| Interfaces / Inputs/ Outputs |  |
| Control Interfaces | Ethernet |
| Lens |  |
| Horizontal angle of view | 107 - 34° |
| Focal length | 2,8 - 12 mm |
| Lens Type | Varifocal (motorized) |
| Video |  |
| Image transfer rate max. | 30 fps |
| Video Compression | H.264, H.264+, H.265, H.265+, M-JPEG |
| Product labeling: |  |
| Manufacturer order number | 238675 |
| Series | Solvido WDR Kameras |
| Brand | Solvido |

Additional Productinformation

Related products

| Solvido SOL-WST-MB-20 | Mastbefestigung 20cm für SOL-IQbox-mini, Set (2St) |
| --- | --- |
| Solvido SOL-WST-WB | Wall mounting for CUS-WST and SOL-IQbox series set (4pcs) wall clearance 10mm |

Recommendations

| Western Digital SD card,WDD256G1P0C,256GB,Class10 | MicroSDXC Speicherkarte, 256GB, Class 10, UHS-I U1, 128TBW, für Videoüberwachung |
| --- | --- |
| Western Digital WDD032G1P0C |  |
| Western Digital WDD064G1P0C | MicroSDXC Speicherkarte, 64GB, Class 10, UHS-I U1, 32TBW, für Videoüberwachung |
| Western Digital WDD128G1P0C |  |

Services