Specifications text

Article number

236668

Type

INM-78M2812M0A

Short description

Network Multisensor camera, 4x 3840x2160, Day&Night, 2,8-12mm IR, White light, WDR, IK10, IP67

Main Features

Multidirectional camera 4x 1/1.8" CMOS

Max. Resolution 3840 x 2160, frame rate 25fps

Lenses with motorised focal length 2.8-12mm

AI analysis functions including object classification

Smart codec H.265/H.264, multi-streaming

Browser-independent (Edge, Safari, Firefox, Chrome)

Supports T-WDR, BLC, HLC and 3D DNR

Supports ONVIF profiles G, S and T

Integrated IR and white light illumination

For outdoor use IP67, vandalism protected IK10

NDAA compliance

Wall bracket and PoE injector included

Other Features (Specifications)

| EAN/GTIN | 4260229840627 |
| --- | --- |
| Audio support | Yes |
| Motion detector | Yes |
| Digital Noise Reduction (DNR) | Yes |
| Multicast | Yes |
| Backlight compensation | Yes |
| Low Speed Shutter | Yes |
| Vandalism-resistant | Yes, Yes |
| Resolution standard | 32 MP |
| Recording sensor | CMOS |
| Lighting | Infrared, White |
| Image resolution max. | 3840x2160 |
| Image transfer rate max. | 25 fps |
| Horizontal angle of view | 48 - 112° |
| Focal length | 2,8 - 12 mm |
| Chip size | 1/1,8" |
| Dimensions (WxHxD) (extended) | 300x195mm |
| Alarm processing | recording internal storage (SD card),E-Mail,HTTP,FTP,TCP,UDP,alarm out,Cloud Upload |
| Audio | 2 Way Audio |
| Resolution (extended) | 3840x2140 (16:9),2592x1944 (4:3),2592x1520 (16:9),2304x1296 (16:9),1920x1080 (16:9),1280x960 (4:3),1280x720 (16:9),640x480 (4:3),320x240 (4:3) |
| Illumination range | Approx. 35 m IR ¦ Approx. 25 m White light |
| Remark | To guarantee you an optimum user experience,we recommend that you only use components of the same brand in a system. If several brands are used in combination,eneo cannot guarantee functionality. If combined use is necessary,we offer a compatibility check |
| Watch (62px/m) | 21.6 m |
| Aperture Range (F) | F1.3 |
| Browser | Firefox, Google Chrome, MS Internet Explorer 10/11, Safari |
| Data stream types | Triple-Stream(Main/Sub/Mobile) |
| Detect (125px/m) | 9.6 m |
| ESC | 1/5 ~ 1/20.000sec. |
| External connections | RJ-45,12VDC input,24VAC input,Alarm in/output,Audio input/output,micro SD card slot 256 GB |
| Identify (250px/m) | 3.7 m |
| Camera shape | Minsters |
| Scope of delivery | manual,bag of screws,wall bracket,PoE-Injector 100-240VAC~ 56VDC,60W |
| Manual Electric Shutter (MES) | 1-5~1-20,000sec. |
| Menu languages | English, French, German, Italian, Japanese, Portuguese, Russian, Spanish, Turkish |
| Password | Administrator and User |
| Privacy Zone | Areas-ON/OFF switchable |
| SDK | upon request |
| Signal-to-noise ratio | 51 dB |
| Day/night mode | Day,Night,Automatic,Time |
| Text overlay | Switchable on/off |
| Perceiving (25px/m) | 56.7 m |
| White balance | Automatic |
| Color (Housing) | RAL9003,Pantone P 179-1 |
| Color (dome) | clear |
| Housing type | Outside |
| Housing material | Aluminium |
| Internal memory | 256GB, microSD card, microSDHC card, microSDXC card |
| Camera Type | Multi-sensor camera |
| Photosensitivity | 0,006 Lux bei F1,2 |
| Mounting type | Wall |
| Network Protocols | DDNS, DHCP, DNA, FTP, HTTP, HTTPS, NTP, PPPoE, RTP, RTSP, SMTP, TCP-IP, UPnP |
| Lens Type | Varifocal (motorized) |
| ONVIF | ONVIF Profile G, ONVIF Profile M, ONVIF Profile S, ONVIF Profile T |
| Manufacturer order number | 236668 |
| Power over Ethernet | IEEE 802.3bt |
| Protection | IP67 |
| Series | eneo IN-Serie |
| Control Interfaces | Ethernet |
| Power supply | 12VDC, 24VAC, PoE++ |
| System | Electronic Day&Night |
| Day/night switching | Switchable IR Filter (ICR) |
| Temperature Range (Operating) | -30°C ~ +55°C,@ Luftfeuchtigkeit 95% oder weniger (nicht kondensierend) |
| Video Compression | H.264, H.264+, H.265, H.265+, M-JPEG |
| Wide Dynamic Range (WDR) | WDR 120dB |
| Certifications | IK10 |
| Brand | eneo |

Additional Productinformation

Related products

| CM-22 | Corner mounting adapter for wall bracket WB-24 and PTZ dome camera INP-75A0033MIA |
| --- | --- |
| DOME-CH7 | Ceiling mount IN-Series Length approx. 294mm |
| eneo INsight Installation Tool | Supports adjustments for cameras with AI features for IN-series |
| eneo Site Manager | Management Software for eneo IP products |
| KBD-3NET | Network keyboard with joystick, supports ONVIF protocol, RJ45, 12VDC |
| PM-21 | Pole mounting adapter for camera connection box AK-20 and AK-21 |

Recommendations

| jetpower-PSU-XS-11C Steckernetzteil |  |
| --- | --- |

Services

| LACKIERUNG Fix-Dome-Kamera | Extra charge for special paint fixed dome in RAL color |
| --- | --- |
| LACKIERUNG Grundkosten | Painting basic costs once per RAL color and delivery lot |
| Vorkonfiguration IP-Kamera | Preconfiguration 1x IP camera Function test, FW update |