Specifications text

Article number

236668

Type

eneo INM-78M2812M0A

Short description

Network Multisensor Camera, 360°, 4x 3840x2160, Day&Night, WDR, AI, 4x 2,8-12mm, IR, White light, IP67, IK10

Main Features

Multidirectional camera 4x 1/1.8" CMOS sensor

Max. Resolution 32MP, 4x 3840 x 2160, frame rate 30fps

4 Lenses with motorised focal length 2.8-12mm

AI analysis functions including object classification

Smart codec H.265/H.264, multi-streaming

Browser-independent (Edge, Safari, Firefox, Chrome)

Supports T-WDR, BLC, HLC and 3D DNR

Supports ONVIF profiles S/T/G/M

Integrated IR and white light illumination

For outdoor use IP67, vandalism protected IK10

NDAA compliance

Wall bracket and PoE injector included

Other Features (Specifications)

| General |  |
| --- | --- |
| Vandalism-resistant | Yes |
| Lighting | Infrared, White |
| Dimensions (WxHxD) (extended) | 300x195mm |
| Illumination range | Approx. 35 m IR ¦ Approx. 25 m White light |
| ESC | 1/5 ~ 1/20.000sec. |
| Weight | 4.6 kg |
| SDK | upon request |
| Text overlay | Switchable on/off |
| Color (Housing) | RAL9003,Pantone P 179-1 / Other colors available upon request |
| Color (dome) | clear |
| Housing type | Outside |
| Housing material | Aluminium |
| Camera Type | Multi-sensor camera |
| Power consumption | 43 W |
| Mounting type | Wall |
| ONVIF | ONVIF Profile G, ONVIF Profile M, ONVIF Profile S, ONVIF Profile T |
| Power over Ethernet | IEEE 802.3bt |
| Protection | IP67, NEMA 4X |
| Power supply | 12VDC, 24VAC, PoE++ |
| Temperature Range (Operating) | -30°C ~ +55°C,@ Luftfeuchtigkeit 95% oder weniger (nicht kondensierend) |
| Wide Dynamic Range (WDR) | WDR 120dB |
| Certifications | IK10 |
| Camera |  |
| Backlight compensation | Yes |
| Low Speed Shutter | Yes |
| Resolution standard | 32 MP |
| Recording sensor | CMOS |
| Chip size | 1/1,8" |
| Camera shape | Minsters |
| Manual Electric Shutter (MES) | 1-5~1-20,000sec. |
| Signal-to-noise ratio | 51 dB |
| Day/night mode | Day,Night,Automatic,Time |
| White balance | Automatic |
| Photosensitivity | 0,006 Lux bei F1,2 |
| System | Electronic Day&Night |
| Day/night switching | Switchable IR Filter (ICR) |
| Display |  |
| Image resolution max. | 4x 3840x2160 |
| DORI |  |
| Watch (62px/m) | 23,4 - 71 m |
| Detect (125px/m) | 11,4 - 35,1 m |
| Identify (250px/m) | 5,4 - 17,6 m |
| Perceiving (25px/m) | 58,6 - 177 m |
| Functions |  |
| Analysis | Image rotation for cameras mounted at a 90° angle, Abandoned and Removed Objects (SOD), Attendance (FA), Crowds (CD), Face Attributes (AD), Face Capture (FD), Face Recognition (FR), Heat Map (HM), Intrusion (PID), Leave Region, License plate recognition (LPD), License plate recognition (LPR), Line crossing (LCD), Object Classification Human / Vehicle (PD&VD), Passage count (CC), Queues (QD), Rare Sounds (RSD) |
| Audio support | Yes |
| Motion detector | Yes |
| Digital Noise Reduction (DNR) | Yes |
| Multicast | Yes |
| Alarm processing | recording internal storage (SD card),E-Mail,HTTP,FTP,TCP,UDP,alarm out,Cloud Upload |
| Audio | 2 Way Audio |
| Browser | Firefox, Google Chrome, MS Internet Explorer 10/11, Safari |
| Menu languages | English, French, German, Italian, Japanese, Portuguese, Russian, Spanish, Turkish |
| Password | Administrator and User |
| Privacy Zone | Areas-ON/OFF switchable |
| Internal memory | 256GB, microSD card, microSDHC card, microSDXC card |
| Notes |  |
| Remark | For optimal results,use only eneo components. For mixed brands,we offer a compatibility check for a fee (info: item #229235 at VIDEOR). |
| Interfaces / Inputs/ Outputs |  |
| External connections | RJ-45,12VDC input,24VAC input,Alarm in/output,Audio input/output,micro SD card slot 256 GB |
| Control Interfaces | Ethernet |
| Parts supplied |  |
| Scope of delivery | manual,bag of screws,wall bracket,PoE-Injector 100-240VAC~ 56VDC,60W |
| Network |  |
| Network Protocols | DDNS, DHCP, DNA, FTP, HTTP, HTTPS, IEE 802.1, NTP, PPPoE, RTP, RTSP, SMTP, TCP-IP, UPnP |
| Lens |  |
| Horizontal angle of view | 360° |
| Vertical angle of view | 27 - 58° |
| Focal length | 2,8 - 12 mm |
| Aperture Range (F) | F1.3 |
| Lens Type | Varifocal (motorized) |
| Video |  |
| Image transfer rate max. | 25 fps |
| Resolution (extended) | 3840x2140 (16:9),2592x1944 (4:3),2592x1520 (16:9),2304x1296 (16:9),1920x1080 (16:9),1280x960 (4:3),1280x720 (16:9),640x480 (4:3),320x240 (4:3) |
| Data stream types | Triple-Stream(Main/Sub/Mobile) |
| Video Compression | H.264, H.264+, H.265, H.265+, M-JPEG |
| Product labeling: |  |
| EAN/GTIN | 4260229840627 |
| Manufacturer order number | 236668 |
| Series | eneo IN-Serie |
| Brand | eneo |

Additional Productinformation

Related products

| eneo CM-22 | Corner mounting adapter for wall bracket WB-24 and PTZ dome camera INP-75A0033MIB |
| --- | --- |
| eneo DOME-CH7 | Ceiling mount IN-Series Length approx. 294mm |
| eneo INsight | Control software for recorder IN- and MN-series |
| eneo INsight Installation Tool | Supports adjustments for cameras with AI features for IN-series |
| eneo INsight mobile | Mobile App for recorder IN- and MN-series |
| eneo PM-21 | Pole mounting adapter, steel for camera connection box AK-20, AK-21 and wall mount WB-24 |
| eneo PM-21 | Pole mounting adapter V2A stainless steel, for camera connection box AK-20, AK-21 and wall mount WB-24 |
| eneo Site Manager | Management Software for eneo IP products |

Recommendations

| jetrics jetcat6SFTP-GN-0,5/1 |  |
| --- | --- |
| jetrics jetcat6SFTP-GN-1/1 |  |
| jetrics jetcat6SFTP-GN-2/1 |  |
| jetrics jetcat6SFTP-GN-3/1 |  |
| jetrics jetcat6SFTP-GN-7,5/1 |  |

Services

| PAINTING SERVICE basic costs | once per RAL color and delivery lot |
| --- | --- |
| PAINTING SERVICE fix dome camera | in RAL color |
| Vorkonfiguration IP-Kamera | Preconfiguration 1x IP camera Function test, FW update |