Specifications text

Article number

238647

Type

IND-58F0028MTLA

Short description

Network Dome Fix, Turret, Low Light, 3840x2160, Day/Night WDR, AI, 2,8mm, 12VDC, PoE, IP67

Main Features

1/1.8" Sony STARVIS CMOS sensor

Very good detail recognition even in dark areas

Maximum resolution 3840x2160 pixels, 30fps

AI analysis functions including object classification

WDR 120dB, 3D DNR noise reduction

Lens fixed focal length 2.8mm

Smart codec H.265/H.264, multi-streaming

Browser-independent (Edge, Safari, Firefox, Chrome)

Supports ONVIF profiles S/T/G/M

Local storage via micro SD card

NDAA compliance

White light LED illumination, IP67

Other Features (Specifications)

| EAN/GTIN | 4260229840641 |
| --- | --- |
| Audio support | Yes |
| Motion detector | Yes |
| Digital Noise Reduction (DNR) | Yes |
| Multicast | Yes |
| Backlight compensation | Yes |
| Low Speed Shutter | Yes |
| Resolution standard | 8 MP |
| Recording sensor | CMOS |
| Lighting | White |
| Image resolution max. | 3840x2160 |
| Image transfer rate max. | 30 fps |
| Horizontal angle of view | 112° |
| Focal length | 2,8 mm |
| Chip size | 1/1,8" |
| Dimensions (WxHxD) (extended) | 97,9x115,5mm |
| Alarm processing | recording internal storage (SD card),E-Mail,HTTP,FTP,TCP,UDP,Red & bule flashing lights,Siren,Cloud Upload |
| Audio | 2 Way Audio, Built-in microphone, Built-in speaker |
| Resolution (extended) | 3840x2160 (16:9),2592x1944 (4:3),3072x1728 (16:9),2592x1520 (16:9),2304x1296 (16:9),1920x1080 (16:9),640x480 (4:3),320x240 (4:3) |
| Illumination range | Approx 25m |
| Remark | The face detection and license plate recognition functions are only available with an inserted SD card or in conjunction with a recorder from the eneo IN series.,NDAA compliance under Section 889,To guarantee you an optimum user experience,we recommend th |
| Watch (62px/m) | 22.8 m |
| Picture size | 16:9, 4:3 |
| Browser | Edge, Firefox, Google Chrome, Safari |
| Data stream types | Triple-Stream(Main/Sub/Mobile) |
| Detect (125px/m) | 10.3 m |
| ESC | 1/5 ~ 1/20.000sec. |
| External connections | RJ-45,12VDC input (DC plug),connection cable approx.30cm,micro SD card slot 256 GB |
| Flickerless function | Yes |
| Identify (250px/m) | 3.5 m |
| Camera shape | Minsters |
| Scope of delivery | manual,bag of screws |
| Manual Electric Shutter (MES) | 1-5~1-20,000sec. |
| Menu languages | English, French, German, Italian, Japanese, Portuguese, Russian, Spanish, Turkish |
| Password | Administrator and User |
| Privacy Zone | Areas-ON/OFF switchable |
| SDK | upon request |
| Signal-to-noise ratio | 49 dB |
| Day/night mode | not available |
| Text overlay | Switchable on/off |
| Perceiving (25px/m) | 59 m |
| White balance | Automatic,Manual |
| Color (Housing) | RAL9003,Pantone P 179-1 |
| Housing type | Outside |
| Housing material | Aluminium |
| Internal memory | microSD card slot |
| Camera Type | Face, License plate recognition, People counting, Retail Intelligence |
| Photosensitivity | 0.0005 Lux @ F1,0 |
| MOD (Minimum Object Distance) | 1 m |
| Mounting type | Ceiling, Wall |
| Network Protocols | DDNS, DHCP, DNA, FTP, HTTP, HTTPS, NTP, RTP, RTSP, SMTP, TCP-IP, UPnP |
| Lens Type | Prime lens |
| ONVIF | ONVIF Profile G, ONVIF Profile M, ONVIF Profile S, ONVIF Profile T |
| Manufacturer order number | 238647 |
| Power over Ethernet | IEEE 802.3af |
| Protection | IP67 |
| Series | eneo IN-Serie |
| Control Interfaces | Ethernet |
| Power supply | 12VDC, Poe |
| System | 24h Color |
| Day/night switching | no |
| Temperature Range (Operating) | -30°C ~ +55°C,@ Luftfeuchtigkeit 95% oder weniger (nicht kondensierend) |
| Video Compression | H.264, H.264+, H.265, H.265+, M-JPEG |
| Wide Dynamic Range (WDR) | WDR 120dB |
| Brand | eneo |

Additional Productinformation

Related products

| AK-21 | Camera connection box for Wall and ceiling mounting, IN Series Bullet Cameras |
| --- | --- |
| DOME-WB7 | Wall bracket for Mini Dome IND-42M2808M0A |
| eneo INsight Installation Tool | Supports adjustments for cameras with AI features for IN-series |
| eneo Site Manager | Management Software for eneo IP products |

Recommendations

| eneo INsight mobile | Mobile App for recorder IN- and MN-series |
| --- | --- |
| jetcon-Cat-Cat-FC | Feldkonfektionierbar |
| jetcon-RJ45S-Cat6-FC |  |
| jetcon-RJ45S-Cat6-small-FC | Feldkonfektionierbar |
| jetcon-RJ45S-Cat6A-F | Feldkonfektionierbar |
| jetpower-PSU-XS-11C Steckernetzteil |  |
| LACKIERUNG Fix-Dome-Kamera | Extra charge for special paint fixed dome in RAL color |

Services

| Vorkonfiguration IP-Kamera | Preconfiguration 1x IP camera Function test, FW update |
| --- | --- |