Specifications text

Article number

236317

Type

INT-8SF0003M0A

Short description

Network Hybrid Bullet Camera Day/Night, 2592x1944 optical, 256x192 thermal, IP66, PoE, 12V

Main Features

Dual network bullet camera

Optical resolution 2592x1944 pixels

Thermal resolution 256x192 pixels

Fixed focal 3.6mm (optical) H:81°, V:57°

Fixed focal 3.2mm (thermal) H:56°, V:42°

AI analysis functions including object classification (optical)

Temperature alarm measuring range -20°~ +150° (±8°)

Supports audio and alarm inputs and outputs

Local storage via micro SD card

Supports ONVIF profiles S/T/G/M

NDAA compliance

Suitable for outdoor use IP66

Other Features (Specifications)

| EAN/GTIN | 4260229840603 |
| --- | --- |
| Audio support | Yes |
| Motion detector | Yes |
| Digital Noise Reduction (DNR) | Yes |
| Sunshade canopy | Yes |
| Integrated wall mount | Yes |
| Backlight compensation | Yes |
| Low Speed Shutter | Yes |
| Vandalism-resistant | No |
| Alarm inputs | 1 |
| Recording sensor | CMOS |
| Lighting | Infrared, White |
| Image resolution max. | 2592x1944 |
| Image transfer rate max. | 25 fps |
| Horizontal angle of view | Optisch 81°,Thermal 56° |
| Vertical angle of view | Optisch 57°,Thermal 42° |
| Focal length | 3,2 mm,3,6 mm |
| Remark | The viewing angles of the camera modules vary. We recommend only using components from the same brand together. You can find more information about our paid compatibility check with other brands at VIDEOR #229235. |
| Watch (62px/m) | 27 |
| Browser | Edge, Firefox, Google Chrome, Safari |
| Data stream types | Triple-Stream(Main/Sub/Mobile) |
| Detect (125px/m) | 12.8 |
| ESC | 1/25 ~ 1/25.000Sek. |
| Scope of delivery | manual,bag of screws |
| Manual Electric Shutter (MES) | 1-25~1-25,000Sec |
| Menu languages | English, French, German, Italian, Japanese, Portuguese, Russian, Spanish, Turkish |
| Password | Administrator and User |
| Eneo Privacy | Areas-ON/OFF switchable |
| Text overlay | Switchable on/off |
| Perceiving (25px/m) | 68.5 |
| Color (Housing) | RAL9003,Pantone P 179-1 |
| Housing type | Outside |
| Housing material | Aluminium |
| Internal memory | microSD card slot |
| Camera Type | Thermal Hybrid Bullet |
| Power consumption | 18.2 W |
| Photosensitivity | 0,003 Lux @ F1,6 AGC on |
| Mounting type | Ceiling, Wall |
| Lens Type | Prime lens |
| ONVIF | ONVIF Profile G, ONVIF Profile M, ONVIF Profile S, ONVIF Profile T |
| Manufacturer order number | 236317 |
| Power over Ethernet | IEEE 802.3af |
| Protection | IP67 |
| Series | eneo IN-Serie |
| Control Interfaces | Ethernet |
| Power supply | 12VDC, Poe |
| System | Thermal image, True Day&Night |
| Temperature Range (Operating) | -30°C ~ +60°C |
| Thermal resolution | 256x192 |
| Video Compression | H.264, H.265, M-JPEG |
| Wide Dynamic Range (WDR) | WDR 120dB |
| Brand | eneo |

Additional Productinformation

Related products

| AK-20 | Camera connection box for Wall and ceiling mounting, IN Series Bullet Cameras |
| --- | --- |
| AK-21 | Camera connection box for Wall and ceiling mounting, IN Series Bullet Cameras |
| AK-24 | Dual camera connection box for selected Bullet or Fix Dome IN-Series cameras |
| eneo INsight Installation Tool | Supports adjustments for cameras with AI features for IN-series |
| eneo INsight mobile | Mobile App for recorder IN- and MN-series |
| eneo Site Manager | Management Software for eneo IP products |
| PM-21 | Pole mounting adapter for camera connection box AK-20 and AK-21 |

Recommendations

| jetpower-PSU-XS-11C Steckernetzteil |  |
| --- | --- |

Services

| CON-KOMP | Compatibility check for cameras, NVR, VMS systems (one pair of devices) |
| --- | --- |
| Vorkonfiguration IP-Kamera | Preconfiguration 1x IP camera Function test, FW update |