Specifications text

Article number

236317

Type

INT-8SF0003M0A

Short description

Network Hybrid Bullet Camera Day/Night, 2592x1944 optical, 256x192 thermal, IP66, PoE, 12V

Main Features

Dual network bullet camera

Optical resolution 2592x1944 pixels

Thermal resolution 256x192 pixels

Fixed focal 3.6mm (optical) H:81°, V:57°

Fixed focal 3.2mm (thermal) H:56°, V:42°

AI analysis functions including object classification (optical)

Temperature alarm measuring range -20°~ +150° (±8°)

Supports audio and alarm inputs and outputs

Local storage via micro SD card

Supports ONVIF profiles S, G and profile T

NDAA compliance

Suitable for outdoor use IP66

Other Features (Specifications)

| EAN/GTIN | 4260229840603 |
| --- | --- |
| Audio support | Yes |
| Recording sensor | CMOS |
| Lighting | Infrared, White |
| Motion detector | No |
| Image resolution max. | 2592x1944 |
| Image transfer rate max. | 25 fps |
| Horizontal angle of view | 81°,56° |
| Vertical angle of view | [57,+∞] |
| Focal length | 3,2 mm,3,6 mm |
| Digital Noise Reduction (DNR) | Yes |
| Remark | Please note that the viewing angles of the camera modules are different.,Note on the analysis functions: The Thermal imaging camera module of the Hybrid camera only supports the temperature exception alarm and fire detector functions.,To guarantee you an |
| Watch (62px/m) | 31.1 m |
| Browser | Edge, Firefox, Google Chrome, Safari |
| Data stream types | Triple-Stream(Main/Sub/Mobile) |
| Detect (125px/m) | 7 m |
| Scope of delivery | manual,bag of screws |
| Menu languages | English, French, German, Italian, Japanese, Portuguese, Russian, Spanish, Turkish |
| Password | Administrator and User |
| Eneo Privacy | Areas-ON/OFF switchable |
| Sunshade canopy | Yes |
| Perceiving (25px/m) | 77.6 m |
| Integrated wall mount | Yes |
| Backlight compensation | No |
| Housing type | Outside |
| Housing material | Aluminium |
| Heating | Yes |
| Internal memory | microSD card slot |
| Camera Type | Thermal Hybrid Bullet |
| Power consumption | 10.2 W |
| Photosensitivity | 0,003 Lux @ F1,6 AGC on |
| Low Speed Shutter | No |
| Mounting type | Ceiling, Wall |
| Lens Type | Prime lens |
| ONVIF | ONVIF Profile G, ONVIF Profile M, ONVIF Profile S, ONVIF Profile T |
| Manufacturer order number | 236317 |
| Protection | IP67 |
| Series | eneo IN-Serie |
| Control Interfaces | Ethernet |
| Power supply | 12VDC, Poe |
| System | Thermal image, True Day&Night |
| Temperature Range (Operating) | -30°C ~ +60°C |
| Thermal resolution | 256x192 |
| Vandalism-resistant | No |
| Video Compression | H.264, H.265, M-JPEG |
| Wide Dynamic Range (WDR) | D-WDR, D-WDR, D-WDR, D-WDR, D-WDR, D-WDR, D-WDR |
| Brand | eneo |

Additional Productinformation

Related products

| AK-20 | Camera connection box for Wall and ceiling mounting, IN Series Bullet Cameras |
| --- | --- |
| AK-21 | Camera connection box for Wall and ceiling mounting, IN Series Bullet Cameras |
| AK-24 | Dual camera connection box for selected Bullet or Fix Dome IN-Series cameras |
| eneo INsight Installation Tool | Supports adjustments for cameras with AI features for IN-series |
| eneo INsight mobile | Mobile App for recorder IN- and MN-series |
| eneo Site Manager | Management Software for eneo IP products |
| PM-21 | Pole mounting adapter for camera connection box AK-20 and AK-21 |

Recommendations

| eneo INsight mobile | Mobile App for recorder IN- and MN-series |
| --- | --- |
| jetpower-PSU-XS-11C Steckernetzteil | Universal Netzteil 12V/1,5A ID 2.1 x OD 5.5, 11mm, inlusive Adapter von 11 auf 9mm Länge |
| Vorkonfiguration IP-Kamera | Preconfiguration 1x IP camera Function test, FW update |

Services

| Vorkonfiguration IP-Kamera | Preconfiguration 1x IP camera Function test, FW update |
| --- | --- |