Specifications text

Article number

236317

Type

eneo INT-8SF0003M0A

Short description

Network Hybrid Thermal Bullet Camera Day/Night, 2592x1944 optical, 256x192 thermal, IP66, PoE, 12VDC

Main Features

Dual network bullet camera

Optical resolution 2592x1944 pixels

Thermal resolution 256x192 pixels

Fixed focal 3.6mm (optical) H:81°, V:57°

Fixed focal 3.2mm (thermal) H:56°, V:42°

AI analysis functions including object classification (optical)

Temperature alarm measuring range -20°~ +150° (±8°)

Supports audio and alarm inputs and outputs

Local storage via micro SD card

Supports ONVIF profiles S/T/G/M

NDAA compliance

Suitable for outdoor use IP66

Other Features (Specifications)

| General |  |
| --- | --- |
| Heating | No |
| Sunshade canopy | Yes |
| Integrated wall mount | Yes |
| Vandalism-resistant | No |
| Lighting | Infrared, Warm Light |
| ESC | 1/25 ~ 1/25.000Sek. |
| Weight | 1.45 kg |
| Text overlay | Switchable on/off |
| Color (Housing) | RAL9003,Other colors available upon request |
| Housing type | Outside |
| Housing material | Aluminium |
| Camera Type | Thermal Hybrid Bullet |
| Power consumption | 18.2 W |
| Mounting type | Ceiling, Wall |
| ONVIF | ONVIF Profile G, ONVIF Profile M, ONVIF Profile S, ONVIF Profile T |
| Power over Ethernet | IEEE 802.3af |
| Protection | IP67, NEMA 4X |
| Power supply | 12VDC, Poe |
| Temperature Range (Operating) | -30°C ~ +60°C |
| Thermal resolution | 256x192 |
| Wide Dynamic Range (WDR) | WDR 120dB |
| Camera |  |
| Backlight compensation | Yes |
| Low Speed Shutter | Yes |
| Recording sensor | CMOS |
| Manual Electric Shutter (MES) | 1-25~1-25,000Sec |
| Photosensitivity | 0,003 Lux @ F1,6 AGC on |
| System | Thermal image, True Day&Night |
| Display |  |
| Image resolution max. | 2592x1944 |
| DORI |  |
| Watch (62px/m) | 27,3 m |
| Detect (125px/m) | 14 m |
| Identify (250px/m) | 7 m |
| Perceiving (25px/m) | 68,5 m |
| Functions |  |
| Analysis | Abandoned and Removed Objects (SOD), Attendance (FA), Crowds (CD), Enter Region, Face Capture (FD), Face Recognition (FR), Fire alarm, Heat Map (HM), Intrusion, Leave Region, Line crossing (LCD), Object Classification Human / Vehicle (PD&VD), Passage count (CC), Queues (QD), Rare Sounds (RSD), Temperature Exception Alarm |
| Audio support | Yes |
| Motion detector | Yes |
| Digital Noise Reduction (DNR) | Yes |
| Browser | Edge, Firefox, Google Chrome, MS Internet Explorer 10/11, Safari |
| Menu languages | English, French, German, Italian, Japanese, Portuguese, Russian, Spanish, Turkish |
| Password | Administrator and User |
| Eneo Privacy | Areas-ON/OFF switchable |
| Internal memory | microSD card slot |
| Notes |  |
| Remark | The viewing angles of the camera modules vary. We recommend only using components from the same brand together. You can find more information about our paid compatibility check with other brands at VIDEOR #229235. |
| Interfaces / Inputs/ Outputs |  |
| Alarm inputs | 1 |
| Control Interfaces | Ethernet |
| Parts supplied |  |
| Scope of delivery | Quick installation guide,drilling template,3x wall plugs with screws,1x Allen wrench,waterproof cover for RJ45 connector |
| Lens |  |
| Horizontal angle of view | Optisch 81°,Thermal 56° |
| Vertical angle of view | Optisch 57°,Thermal 42° |
| Focal length | 3,2 mm,3,6 mm |
| Lens Type | Prime lens |
| Video |  |
| Image transfer rate max. | 25 fps |
| Data stream types | Triple-Stream(Main/Sub/Mobile) |
| Video Compression | H.264, H.265, M-JPEG |
| Product labeling: |  |
| EAN/GTIN | 4260229840603 |
| Manufacturer order number | 236317 |
| Series | eneo IN-Serie |
| Brand | eneo |

Additional Productinformation

Related products

| eneo AK-20 | Camera connection box for Wall and ceiling mounting, IN Series Bullet Cameras |
| --- | --- |
| eneo AK-21 | Camera connection box for Wall and ceiling mounting, Bullet and Dome Cameras IN series |
| eneo AK-24 | Dual camera connection box for selected Bullet or Fix Dome IN-Series cameras |
| eneo INsight | Control software for recorder IN- and MN-series |
| eneo INsight Installation Tool | Supports adjustments for cameras with AI features for IN-series |
| eneo INsight mobile | Mobile App for recorder IN- and MN-series |
| eneo PM-21 | Pole mounting adapter, steel for camera connection box AK-20, AK-21 and wall mount WB-24 |
| eneo PM-21 | Pole mounting adapter V2A stainless steel, for camera connection box AK-20, AK-21 and wall mount WB-24 |
| eneo Site Manager | Management Software for eneo IP products |

Recommendations

| eneo NSMI-4004M0A | Gigabit Switch Industrial, Managed, 4x PoE, 2xSFP, 30W each Port, -40°C~75°C, 44~57 VDC, w/o power supply |
| --- | --- |
| eneo NSMI-4008M0A | Gigabit Switch Industrial, Managed, 8x PoE, 2xSFP, 30W each Port, -40°C~75°C, 44~57 VDC, w/o power supply |
| eneo NSMR-2008M0A | Gigabit Switch Rack, Managed, 8x PoE, 2xSFP, max. 30W each Port, internal Power Supply max. 150W |
| eneo NSMR-2016M0A | Gigabit Switch Rack, Managed, 16x PoE, 2xSFP, max. 30W each Port, integrated Power Suppply max. 250W |
| eneo NSMR-2024M0A | Gigabit Switch Rack, Managed, 24x PoE, 2xSFP, max. 30W each Port, integrated Power Supply max. 400W |
| eneo NSUI-3004M0A | Gigabit Switch Industrial, Unmanaged, 4x PoE, 1xRJ45, 1xSFP, 30W each Port, -40°C~75°C, 44~57 VDC, w/o power supply |
| eneo NSUI-3008M0A | Gigabit Switch Industrial, Unmanaged, 8x PoE, 2xSFP, 30W each Port, -40°C~75°C, 44~57 VDC, w/o power supply |
| eneo NSUR-1008M0A | Gigabit Switch Rack, Unmanaged, 8x PoE, 2xSFP, max. 30W each Port, integrated Power Supply max. 150W |
| eneo NSUR-1016M0A | Gigabit Switch Rack, Unmanaged, 16x PoE, 2xSFP, max. 30W each Port, integrated Power Supply max. 250W |
| eneo NSUR-1024M0A | Gigabit Switch Rack, Unmanaged, 24x PoE, 2xSFP, max. 30W each Port, integrated Power Supply max. 400W |
| Jetrics jetcat6SFTP-GN-0,5/1 |  |
| Jetrics jetcat6SFTP-GN-1/1 |  |
| Jetrics jetcat6SFTP-GN-2/1 |  |
| Jetrics jetcat6SFTP-GN-3/1 |  |
| Jetrics jetcat6SFTP-GN-7,5/1 |  |
| Jetrics jetpower-PSU-XS-11C Steckernetzteil |  |

Services

| PAINTING SERVICE basic costs | once per RAL color and delivery lot |
| --- | --- |
| PAINTING SERVICE bullet camera | in RAL color |
| Vorkonfiguration IP-Kamera | Preconfiguration 1x IP camera Function test, FW update |