Specifications text

Article number

238642

Type

INB-58F0002M0PA

Short description

Network Camera Bullet, Panorama 180°, Lens 2.0mm, max. 3840x2160, AI, WDR, IR, WL, PoE, 12VDC, IP67

Main Features

1/2.8" Sony STARVIS CMOS sensor,

Maximum resolution 3840x2160 pixels, 20fps

Panorama ultra wide-angle camera 180° HFoV

AI analysis functions including object classification

WDR 120dB and 3D-DNR noise reduction

Lens 2.0mm fixed focal length

Smart codec H.265/H.264, multi-streaming

Browser-independent (Edge, Safari, Firefox, Chrome)

Supports ONVIF profiles S/T/G/M and audio

Local storage via micro SD card

NDAA compliance

Integrated IR and white light LED illumination, IP67

Other Features (Specifications)

| General |  |
| --- | --- |
| Sunshade canopy | Yes |
| Integrated wall mount | Yes |
| Lighting | Infrared, White |
| Dimensions (WxHxD) (extended) | 95x241,3mm |
| Illumination range | Approx. 25 m Infrared ¦ Approx. 15 m White light |
| ESC | 1/5 ~ 1/20.000sec. |
| SDK | upon request |
| Text overlay | Switchable on/off |
| Color (Housing) | RAL9003,Pantone P 179-1 |
| Housing type | Outside |
| Housing material | Aluminium |
| Camera Type | Multi-directional camera, People counting, Retail Intelligence |
| Mounting type | Ceiling, Wall |
| ONVIF | ONVIF Profile G, ONVIF Profile M, ONVIF Profile S, ONVIF Profile T |
| Power over Ethernet | IEEE 802.3af |
| Protection | IP67 |
| Power supply | 12VDC, Poe |
| Temperature Range (Operating) | -35°C ~ +60°C,@ Luftfeuchtigkeit 90% oder weniger (nicht kondensierend) |
| Wide Dynamic Range (WDR) | WDR 120dB |
| Camera |  |
| Backlight compensation | Yes |
| Low Speed Shutter | Yes |
| Resolution standard | 8 MP |
| Recording sensor | CMOS, Sony IMX415 |
| Chip size | 1/2,8" |
| Flickerless function | On-Off |
| Camera shape | Bullet |
| Manual Electric Shutter (MES) | 1-5~1-20,000sec. |
| Signal-to-noise ratio | 44 dB |
| Day/night mode | Day,Night,Time,night-color mode |
| White balance | Automatic,Manual |
| Photosensitivity | 0,007 Lux bei F1,0 |
| System | True Day&Night |
| Day/night switching | Switchable IR Filter (ICR) |
| Display |  |
| Image resolution max. | 3840x2160 |
| DORI |  |
| Watch (62px/m) | 10.4 m |
| Detect (125px/m) | 3.8 m |
| Identify (250px/m) | 2.6 m |
| Perceiving (25px/m) | 27.1 m |
| Functions |  |
| Audio support | Yes |
| Motion detector | Yes |
| Digital Noise Reduction (DNR) | Yes |
| Multicast | Yes |
| Alarm processing | recording internal storage (SD card),E-Mail,HTTP,FTP,TCP,UDP,alarm out,Cloud Upload |
| Audio | Built-in microphone, Built-in speaker |
| Browser | Edge, Firefox, Google Chrome, Safari |
| Menu languages | English, French, German, Italian, Japanese, Portuguese, Russian, Spanish, Turkish |
| Password | Administrator and User |
| Privacy Zone | Areas-ON/OFF switchable |
| Internal memory | 256GB, microSDHC card, microSDXC card |
| Notes |  |
| Remark | NDAA compliance under Section 889,The face detection and license plate recognition functions are only available with an inserted SD card or in conjunction with a recorder from the eneo IN series.,To guarantee you an optimum user experience,we recommend th |
| Interfaces / Inputs/ Outputs |  |
| External connections | RJ-45,12VDC input (DC plug),micro SD card slot 256 GB |
| Control Interfaces | Ethernet |
| Parts supplied |  |
| Scope of delivery | manual,bag of screws |
| Network |  |
| Network Protocols | DDNS, DHCP, DNA, FTP, HTTP, HTTPS, NTP, RTP, RTSP, SMTP, TCP-IP, UPnP |
| Lens |  |
| Horizontal angle of view | 180° |
| Focal length | 2 mm |
| MOD (Minimum Object Distance) | 0.6 m |
| Lens Type | Prime lens |
| Video |  |
| Image transfer rate max. | 20 fps |
| Resolution (extended) | 3840x2160 (16:9) @20fps,2592x1944 (4:3),2304x1296 (16:9),1920x1080 (16:9),1280x960 (4:3),1280x720 (16:9),640x480 (4:3) |
| Picture size | 16:9, 4:3 |
| Data stream types | Triple-Stream(Main/Sub/Mobile) |
| Video Compression | H.264, H.264+, H.265, H.265+, M-JPEG |
| Product labeling: |  |
| EAN/GTIN | 4260229840610 |
| Manufacturer order number | 238642 |
| Series | eneo IN-Serie |
| Brand | eneo |

Additional Productinformation

Related products

| AK-21 | Camera connection box for Wall and ceiling mounting, IN Series Bullet Cameras |
| --- | --- |
| eneo INsight | Control software for recorder IN- and MN-series |
| eneo INsight Installation Tool | Supports adjustments for cameras with AI features for IN-series |
| eneo INsight mobile | Mobile App for recorder IN- and MN-series |
| eneo Site Manager | Management Software for eneo IP products |

Recommendations

| jetcat6SFTP-3/1 |  |
| --- | --- |
| jetpower-PSU-XS-11C Steckernetzteil |  |

Services

| LACKIERUNG Bulletkamera | Extra charge for special paint bullet camera in RAL color |
| --- | --- |
| LACKIERUNG Grundkosten | Painting basic costs once per RAL color and delivery lot |
| Vorkonfiguration IP-Kamera | Preconfiguration 1x IP camera Function test, FW update |