Specifications text

Article number

238664

Type

INR-58N640005A

Short description

Network Video Recorder 64x IP, RAID, 16xSATA, 4xHDMI, H.265, H.264, without HDD

Main Features

Maximum resolution 12MP (recording)

Recording modes continuous, manual or AI-based

Maximum data rate input 640Mbps, output 640Mbps

Maximum 16 internal hard drives possible, hotswap capable

RAID 0,1,5,6,10 support

Supports 16 alarm inputs and 4 alarm outputs

4x HDMI output, 2x RJ45 network connection, 1xUSB connection

1 x audio input and output

eSATA connection for external memory expansion

Remote access via browser, eneo Insight or Insight mobile

ONVIF Profile S support, NDAA compliance

USB mouse and 19" mounting bracket included in the scope of delivery

Other Features (Specifications)

| Brand | eneo |
| --- | --- |
| EAN/GTIN | 4260229840634 |
| Dimensions (HxWxD) | 1498x440x460mm |
| Series | eneo IN-Series |
| Blower | yes |
| Channels total | 64 |
| IP channels | 64 |
| Max. recording resolution | 4000x3000 |
| Compression | H.264, H.264+, H.265, H.265+ |
| Spot monitor | no |
| Monitor Outputs | 4x HDMI |
| Network | 2x 10/100/1000 Base-Tx, RJ45 |
| Alarm inputs | 16 |
| Alarm output | 4 |
| Audio support | yes |
| Audio inputs | 1 |
| Serial interfaces | RS-232, RS-485 |
| Max. internal HDD capacity | 288 TB |
| Internal HDD capacity | 0 TB |
| Integrated writer | no |
| HDD slots | 16x SATA |
| HDD hotswap | yes |
| USB ports | 1x USB 2.0, 2x USB 3.0 |
| RAID | + Hot Spare, RAID 0, RAID 1, RAID 10, RAID 5, RAID 6 |
| Data tranfer rate max. | 1280 Mbps |
| Analysis functions | Depending on connected IP camera |
| Remote access | Android, iPhone, remote software, WEB Browser (Java) |
| 19" rack mount | yes |
| Supply voltage | 100-240VAC; ATX250W intern |
| Power consumption | 8W, without HDD |
| Operating temperature | -10°C ~ +50°C, @ Humidity 90% or less (non-condensing) |
| ONVIF | ONVIF Profile S, yes |
| Remote configuration | Remote software |
| Weight | 9kg |
| Network connection | 2 x 10/100/1000BASE-T |
| Data transfer rate incoming | 640 Mbps |
| Audio output | 1 |
| Operating system | Linux |
| Manufacturer order number | 238664 |

Additional Productinformation

Before installing the device in a 19" rack, please ensure that a shelf or suitable slide rails are installed in the cabinet. It is not possible to support the recorder using only the 19" mounting brackets supplied. These are only used to fix the position of the device.To ensure heat dissipation and proper functioning of the device, it is forbidden to stack the device or place other heavy objects on top of it.Use of the 4 HDMI outputs:  3x HDMI supported resolutions: 1024x768, 1280x720, 1280x1024, 1440x900, 1920x1080 , 1920x1080 , 1600x1200?1920x1200, 3840x2160, 7680x4320;  1x HDMI supported resolutions: 1024x768, 1280x720, 1280x1024, 1440x900, 1920x1080;  Note:If the output resolution of HDMI 1 is set to 8K, the maximum output resolution of HDMI 3 and 4 is limited to 1080P and HDMI 2 will be disabled.

Related products

| eneo INsight | Control software for recorder IN- and MN-series |
| --- | --- |
| eneo INsight Evidence Locker | For the research of HDD's for recorders of IN-series |
| eneo INsight Evidence Player | For replay of exported data for recorder IN-series IN-Serie |
| eneo INsight mobile | Mobile App for recorder IN- and MN-series |
| HDD-4000SATA Purple | Hard Disk Drive 4TB, SATA, for eneo Video Recorders, expansion |
| HDD-6000SATA Purple | Hard Disk Drive 6TB, SATA, for eneo Video Recorders, expansion |
| INR-eSATA | HDD Box, eSATA, für max 4 HDD max. 8TB pro HDD, ohne HDD |

Recommendations

| eneo INsight mobile | Mobile App for recorder IN- and MN-series |
| --- | --- |
| HDD-8000SATA Purple | Hard Disk Drive 8TB, SATA, for eneo Video Recorders, expansion |
| WD181PURP | Festplatte, 3,5", SATA 6Gb/s, 18TB, 512MB Cache, 24x7, für Videoüberwachung optimiert |

Services

| CON-eneo-KON-16 | Preconfiguration 1x eneo recorder with up to 16 eneo IP cameras |
| --- | --- |
| CON-eneo-KON-32 | Preconfiguration 1x eneo recorder with up to 32 eneo IP cameras |
| CON-HDD-eneo | Hard disk installation service for eneo |
| CON-KOMP | Compatibility check for cameras, NVR, VMS systems (one pair of devices) |