Specifications text

Article number

238664

Type

eneo INR-58N640005A

Short description

Network Video Recorder 64x IP, RAID, 16xSATA, 4xHDMI, H.265, H.264, without HDD

Main Features

Maximum resolution 12MP (recording)

Recording modes continuous, manual or AI-based

Maximum data rate input 640Mbps, output 640Mbps

Maximum 16 internal hard drives possible,

hotswap capable, RAID 0,1,5,6,10 support

Supports 16x alarm inputs and 4x alarm outputs

4x HDMI output, 2x RJ45 network connection,

1x USB connection, 1x audio input and output

eSATA connection for external memory expansion

Remote access browser, eneo Insight or Insight mobile

ONVIF Profile S support, NDAA compliance

USB mouse, 19" mounting bracket included

Other Features (Specifications)

| System requirements |  |
| --- | --- |
| Operating system | Linux |
| General |  |
| Dimensions (WxHxD) (extended) | 440x149x460mm |
| External Eneo Operating Devices | Mouse |
| Control software | eneo INsight |
| HDD Slots Number | 16,bis zu 18 TB je slot (Enterprise-Class) |
| Power consumption | 8 W |
| ONVIF | ONVIF Profile G, ONVIF Profile S, ONVIF Profile T |
| RAID Level | + Hot Spare, RAID 0, RAID 1, RAID 10, RAID 5, RAID 6 |
| Temperature Range (Operating) | -10°C ~ +50°C,@ Luftfeuchtigkeit 90% oder weniger (nicht kondensierend) |
| Recording |  |
| Resolution Recording (max) | 4000x3000 |
| Hard disk capacity max | 288 TB |
| Functions |  |
| Analysis | Depending on connected IP camera |
| Audio support | Yes |
| HDD Hotswap | Yes |
| Alarm triggers | AI, Contact, Disk Error, Image loss, Movement |
| Alarm response | Alarm Output, Buzzer, E-mail dispatch, FTP, Live Popup, PTZ Preset, Push (Mobile), Spot Popup |
| Menu languages | Arabic, Bulgarian, Danish, Dutch, English, Finnish, French, German, Greek, Hebrew, Hindi, Italian, Japanese, Korean, Polish, Portuguese, Romanian, Russian, Spanish, Thai, Turkish |
| Password | Administrator and User |
| Search function | Event, Instant playback, Museum, Text, Thumbnail, VCA Search |
| Remote Access | Android, Ios, Remote Software, WEB Browser (Java) |
| Notes |  |
| Remark | Two hard drives are required for maximum IP channel recording. Two alarms can be stored in the cloud. Only use eneo components. If mixing brands,a compatibility check (item #229235 at VIDEOR) is available for a fee. |
| Interfaces / Inputs/ Outputs |  |
| Alarm Input | Yes |
| Alarm output | Yes |
| Number of IP Channels | 64 |
| Audio Inputs | 1 |
| Ethernet interface | 1000Base-T, 100Base-T, 10Base-T |
| Memory interface (external) | 1x eSATA |
| Control Interfaces | RS-232, RS-485, USB 2.0, USB 3.0 |
| Video Outputs | 4x HDMI |
| Parts supplied |  |
| Scope of delivery | mouse,manual,19 inch rack mount angle |
| Network |  |
| Data throughput (outbound) | 640 Mbps |
| Data throughput (maximum) | 640 Mbps |
| Network Protocols | DDNS, DHCP, DNA, FTP, HTTP, HTTPS, NTP, RTP, RTSP, SMTP, TCP-IP, UPnP |
| Video |  |
| Video Compression | H.264, H.264+, H.265, H.265+ |
| Product labeling: |  |
| EAN/GTIN | 4260229840634 |
| Manufacturer order number | 238664 |
| Series | eneo IN-Serie |
| Brand | eneo |

Additional Productinformation

Related products

| eneo INR-eSATA |  |
| --- | --- |
| eneo INsight | Control software for recorder IN- and MN-series |
| eneo INsight Evidence Locker | For the research of HDD's for recorders of IN-series |
| eneo INsight Evidence Player | For replay of exported data for recorder IN-series |
| eneo INsight mobile | Mobile App for recorder IN- and MN-series |
| eneo Site Manager | Management Software for eneo IP products |
| Western Digital WD122PURP | WD Purple hard drive, 3.5", SATA 6 Gb/s, 12TB, 256MB Cache, 24x7, optimized for video surveillance |
| Western Digital WD181PURP | WD Purple Hard Drive, 3.5", SATA 6 Gb/s, 18 TB, 512 MB cache, 24/7, optimized for video surveillance |
| Western Digital WD44PURZ | WD Purple Hard Drive, 3.5", SATA 6 Gb/s 4TB, 128MB Cache, 24x7, optimized for video surveillance |
| Western Digital WD64PURZ | WD Purple hard drive, 3.5", SATA 6 Gb/s, 6TB, 256MB Cache, 24x7, optimized for video surveillance |
| Western Digital WD85PURZ | WD Purple Hard Drive, 3.5", SATA 6 Gb/s, 8 TB, 128 MB cache, 24/7, optimized for video surveillance |

Recommendations

| eneo NSMI-4004M0A | Gigabit Switch Industrial, Managed, 4x PoE, 2xSFP, 30W each Port, -40°C~75°C, 44~57 VDC, w/o power supply |
| --- | --- |
| eneo NSMI-4008M0A | Gigabit Switch Industrial, Managed, 8x PoE, 2xSFP, 30W each Port, -40°C~75°C, 44~57 VDC, w/o power supply |
| eneo NSMR-2008M0A | Gigabit Switch Rack, Managed, 8x PoE, 2xSFP, max. 30W each Port, internal Power Supply max. 150W |
| eneo NSMR-2016M0A | Gigabit Switch Rack, Managed, 16x PoE, 2xSFP, max. 30W each Port, integrated Power Suppply max. 250W |
| eneo NSMR-2024M0A | Gigabit Switch Rack, Managed, 24x PoE, 2xSFP, max. 30W each Port, integrated Power Supply max. 400W |
| eneo NSUI-3004M0A | Gigabit Switch Industrial, Unmanaged, 4x PoE, 1xRJ45, 1xSFP, 30W each Port, -40°C~75°C, 44~57 VDC, w/o power supply |
| eneo NSUI-3008M0A | Gigabit Switch Industrial, Unmanaged, 8x PoE, 2xSFP, 30W each Port, -40°C~75°C, 44~57 VDC, w/o power supply |
| eneo NSUR-1008M0A | Gigabit Switch Rack, Unmanaged, 8x PoE, 2xSFP, max. 30W each Port, integrated Power Supply max. 150W |
| eneo NSUR-1016M0A | Gigabit Switch Rack, Unmanaged, 16x PoE, 2xSFP, max. 30W each Port, integrated Power Supply max. 250W |
| eneo NSUR-1024M0A | Gigabit Switch Rack, Unmanaged, 24x PoE, 2xSFP, max. 30W each Port, integrated Power Supply max. 400W |
| jetrics jetcat6SFTP-GN-0,5/1 |  |
| jetrics jetcat6SFTP-GN-1/1 |  |
| jetrics jetcat6SFTP-GN-2/1 |  |
| jetrics jetcat6SFTP-GN-3/1 |  |
| jetrics jetcat6SFTP-GN-7,5/1 |  |
| jetrics jetcon HDMI Kabel 1 |  |
| jetrics jetcon HDMI Kabel 2 |  |
| jetrics jetcon HDMI Kabel 3 |  |

Services

| CON-eneo-KON-04 | Preconfiguration 1x eneo recorder with up to 4 eneo IP cameras |
| --- | --- |
| CON-eneo-KON-08 | Preconfiguration 1x eneo recorder1x eneo IE with up to 8 eneo IP cameras |
| CON-eneo-KON-16 | Preconfiguration 1x eneo recorder with up to 16 eneo IP cameras |
| CON-eneo-KON-32 | Preconfiguration 1x eneo recorder with up to 32 eneo IP cameras |
| CON-HDD-eneo | Hard disk installation service for eneo |
| CON-KOMP | Compatibility check for cameras, NVR, VMS systems (one pair of devices) |
| CON-NVR-KON-xx | Preconfiguration 1x Network Video Recorder with more than 16 IP cameras |