Specifications text

Article number

238858

Type

eneo VM-FHD15FLT

Short description

15" (38cm) LCD Monitor, Full HD 1920x1080, LED, HDMI. DisplayPort Flush Mount, Touch Screen

Main Features

Professional 15" (38cm) LCD touchscreen monitor

Designed for continuous operation 24x7x365 (industry level)

Maximum resolution 1920x1080 pixels

LED backlight

Short response time of 5ms

Optimum display thanks to digital 3D filtering

Viewing angle 178° horizontal and vertical

Contrast ratio typically 1,000:1

Versatile video inputs HDMI, DP

Integrated speakers

Operating voltage: 12VDC, external power supply included in delivery

Flush mount metal housing with white plastic cover

Other Features (Specifications)

| General |  |
| --- | --- |
| Dimensions (WxHxD) (extended) | 389x270x40,5mm,mit Frontcover: 439x298x46mm |
| Fortification | VESA Standard 100x100mm, VESA Standard 75x75mm |
| Weight | 5 kg |
| Energy efficiency class | D |
| Housing material | Metal |
| Power consumption | 13 W |
| Power supply | 12VDC |
| Display |  |
| Viewing Angle Horizontal | 178 ° |
| Viewing Angle Vertical | 178 ° |
| Screen diagonal (cm) | 38 cm |
| Screen diagonal (inches) | 15 " |
| Display | Color TFT-LED |
| Pixel error class | ISO 13406-2, Class 2 |
| Brightness (typical) | 300 cd/m² |
| Backlight | LED |
| Contrast ratio (typical) | 1000:1 |
| Response time (typical) | 5 ms |
| Aspect ratio | 16:9 |
| Functions |  |
| Comb filter | Yes |
| Menu languages | Chinese, English, French, German, Italian, Russian, Spanish |
| Notes |  |
| Remark | NDAA compliance under Section 889 |
| Interfaces / Inputs/ Outputs |  |
| Audio Inputs | No |
| Control Interfaces | no |
| Video Inputs | DisplayPort, HDMI, USB2.0 |
| Parts supplied |  |
| Scope of delivery | manual,power cord,Acrylic front cover,Power supply unit 100 ~ 240VAC / 12VDC,50/60Hz,HDMI cable,Thumbscrews for front cover |
| Video |  |
| Maximum Image Resolution (H x V) | 1920x1080 |
| Resolution (extended) | 800x600,1024x768,1280x1024,1600x900,1920x1080 |
| Product labeling: |  |
| EAN/GTIN | 4260229841129 |
| Manufacturer order number | 238858 |
| Series | eneo Monitor |
| Brand | eneo |

Additional Productinformation

Related products

Recommendations

| jetrics jetcon HDMI Kabel 1 |  |
| --- | --- |
| jetrics jetcon HDMI Kabel 3 |  |
| jetrics jetcon HDMI Kabel 5 |  |

Services