Specifications text

Article number

239392

Type

eneo VM-FHD24PFWA

Short description

24" (61cm) LCD Monitor, white FHD, 1920x1080, LED, DisplayPort, HDMI, external Power Supply

Main Features

Professional 24" (61cm) LCD monitor

Maximum resolution Full HD 1920x1080 pixels

Designed for continuous operation 24x7x365

LED backlight

Wide viewing angle 178° (H/V)

Optimum display thanks to 3D filtering and noise suppression

Short response time of 5ms

Versatile inputs DisplayPort, HDMI

Integrated speakers

White plastic housing

Integrated height-adjustable stand

Other Features (Specifications)

| General |  |
| --- | --- |
| Applications | 24/7 |
| Dimensions (WxHxD) (extended) | 541,4x321,8x44,3mm |
| Fortification | VESA Standard 100x100mm |
| Weight | 3.5 kg |
| Energy efficiency class | D |
| Color (Housing) | White |
| Housing material | Plastic |
| Power consumption | 25 W |
| Power supply | 12VDC |
| Display |  |
| Viewing Angle Horizontal | 178 ° |
| Viewing Angle Vertical | 178 ° |
| Screen diagonal (cm) | 61 cm |
| Screen diagonal (inches) | 24 " |
| Display | Color TFT-LED, Hard coating (3H) |
| Pixel error class | ISO 13406-2, Class 2 |
| Brightness (typical) | 240 cd/m² |
| Backlight | LED |
| Contrast ratio (typical) | 1000:1 |
| Response time (typical) | 5 ms |
| Aspect ratio | 16:9 |
| Functions |  |
| Comb filter | Yes |
| Menu languages | English, French, German, Italian, Spanish |
| Notes |  |
| Remark | NDAA compliance under Section 889 |
| Interfaces / Inputs/ Outputs |  |
| Video Inputs | DisplayPort, HDMI |
| Parts supplied |  |
| Scope of delivery | manual,power cord,Power supply unit 100 ~ 240VAC / 12VDC,50/60Hz,feet,HDMI cable |
| Video |  |
| Maximum Image Resolution (H x V) | 1920x1080 |
| Resolution (extended) | 800x600,1024x768,1280x1024,1600x900,1920x1080 |
| Product labeling: |  |
| EAN/GTIN | 4260229841105 |
| Manufacturer order number | 239392 |
| Series | eneo Monitor |
| Brand | eneo |

Additional Productinformation

Related products

| jetrics VM-UCWM20 | Compact slim tilt wall mount LCD Monitors, 23" to 42", VESA support up to 25Kg max. |
| --- | --- |
| jetrics VM-UMA10 | Aluminium gas spring monitor arm for wall mount, LCD Monitors, max. 27" support up to 7 kg max. |
| jetrics VM-UTM10 | Universal Table Mount for LCD Monitors, up to 45", VESA support up to 13Kg max. |
| jetrics VM-UWM10 | Universal Wall Mount for LCD Monitors, up to max. 27", VESA support up to 15 kg max. |
| jetrics VM-UWM30 | Universal Wall Mount for LCD Monitors, up to 43", VESA support up to 20Kg max. |

Recommendations

| jetrics jetcon HDMI Kabel 1 |  |
| --- | --- |
| jetrics jetcon HDMI Kabel 3 |  |
| jetrics jetcon HDMI Kabel 5 |  |

Services