Specifications text

Article number

74309

Type

VB43ATF

Short description

Passive Twisted Pair Transmitter, Receiver, Video, Data, Power, BNC, RJ45

Main Features

Video, Data (RS-422) and Camera Power Transmission

Bandwidth: DC to 10MHz

4-pair of Unshielded CAT5 (min) Cable Required

Up-to 350m Video Transmission to Passive Receiver

Up-to 1000m Video Transmission to TR560 Receiver

Up-to 500m to Active Receiver (TR510)

Up-to 100m in TIA/EIA 568 Applications

No Adjustment Necessary

Immunity to Noise and Interference

BNC Plug Cable / RJ45 Jack

Other Features (Specifications)

| Design | Mini module |
| --- | --- |
| Mode | Passive |
| Manufacturer order number | VB43ATF |
| Reach | 350 km |
| Protection | no information |
| Signalling | Data, Video, Voltage |
| Brand | Nitek |

Additional Productinformation

Related products

Recommendations

Services