Specifications text

Article number

74322

Type

VH439

Short description

Passive 4-channel Video TP Transmitter, BNC Inputs

Main Features

4-channel Twisted-pair Video & PTZ Transmission

Video and Data (up-to-coax) Transmission

Bandwidth: DC to 10MHz

Unshielded CAT2 (min) Cable Required

Up-to 350m to Passive Receiver (unshielded cable)

Up-to 1000m to Active Receiver (TR560)

No Adjustment Necessary

Immunity to Noise and Interference

Protection from Power Surge and Transients

Video, AC, Telephone, Computer Signals via 1 Cable

BNC Plug / Terminal Block Connectors or RJ-45

Highly Compact, Wall-mounted Unit

Other Features (Specifications)

| Brand | Nitek |
| --- | --- |
| Model | module |
| Type of unit | transmitter |
| Range | 350 m |
| Signal transmission | data, video, voltage |
| Operation mode | passive |
| Manufacturer order number | VH439 |

Additional Productinformation

<p><span style="color: rgb(254, 0, 0);"><strong>Bitte beachten Sie, dass es bei unserem US-Hersteller Nitek aufgrund der nachlassenden Verfügbarkeit älterer Bauteile, insbesondere analoger Video-Komponenten, zu Lieferverzögerungen kommen kann. Wir bitten um Ihr Verständnis.</strong></span></p>

Related products

| RK439 | Rack Panel, for multiple Nitek VH439, 19", 1U |
| --- | --- |

Recommendations

Services