Specifications text

Article number

74323

Type

VH839

Short description

Passive 8-channel Video TP Transmitter, BNC Inputs

Main Features

8-channel Twisted-pair Video Transmission

Bandwidth: DC to 10MHz

Unshielded CAT2 (min) Cable Required

Up-to 350m to Passive Receiver (unshielded cable)

Up-to 1000m to Active Receiver (TR560)

No Adjustment Necessary

Immunity to Noise and Interference

Built-in Surge Arresting

Video, AC, Telephone, Computer Signals via 1 Cable

BNC Plug / Terminal Block Connectors

Highly Compact, One Rack Unit in Height

Other Features (Specifications)

| Model | housing |
| --- | --- |
| Type of unit | transmitter |
| Range | 350 m |
| Signal transmission | video |
| Protection rating | n/a |
| Operation mode | passive |
| Manufacturer order number | VH839 |
| Brand | Nitek |

Additional Productinformation

<p><span style="color: rgb(254, 0, 0);"><strong>Bitte beachten Sie, dass es bei unserem US-Hersteller Nitek aufgrund der nachlassenden Verfügbarkeit älterer Bauteile, insbesondere analoger Video-Komponenten, zu Lieferverzögerungen kommen kann. Wir bitten um Ihr Verständnis.</strong></span></p>

Related products

Recommendations

Services