Specifications text

Article number

240897

Type

eneo MPC-52A0030M0A

Short description

1/2.8" HD Camera, 1920x1080, 30x AF Zoom 4,6-136mm, T-WDR, HD-SDI, EX-SDI, TVI, Composite

Main Features

1/2.8” 2.0 megapixel Sony CMOS sensor

Resolution max. 1920x1080, 1080p@60/50fps

Optical 30x auto focus zoom, 4.6~136mm

Multisignal outputs HD-SDI, EX-SDI, TVI, Composite

WDR 120dB, DNR, motion detector

Digital noise reduction (2D+3D)

Image stabilization, privacy masking, defog

Slow speed shutter

Compact design

12VDC power supply

Other Features (Specifications)

| General |  |
| --- | --- |
| Lighting | No |
| Dimensions (WxHxD) (extended) | 74,4x67,4x130,1mm |
| Weight | 0.57 kg |
| Text overlay | available |
| Color (Housing) | silvermetallic,black |
| Housing type | Inside |
| Housing material | Aluminium |
| Power consumption | 6 W |
| Power supply | 12VDC |
| Temperature Range (Operating) | -10°C ~ +50°C |
| Wide Dynamic Range (WDR) | WDR 120dB |
| Camera |  |
| Backlight compensation | Yes |
| Low Speed Shutter | Yes |
| Recording sensor | CMOS |
| Chip size | 1/2,8" |
| Flickerless function | Yes |
| Camera shape | Zoom cameras |
| Signal-to-noise ratio | 50 dB |
| Day/night mode | Automatic,Day,Night,extern (Sensor) |
| White balance | AUTO,One push,Manual,Indoor,Outdoor |
| Photosensitivity | 0,5 Lux bei F1,6,50 IRE |
| System | True Day&Night |
| Day/night switching | Switchable IR Filter (ICR) |
| Display |  |
| Image resolution max. | 1920x1080 |
| DORI |  |
| Watch (62px/m) | 27,8 - 800 m |
| Detect (125px/m) | 12,4 - 400 m |
| Identify (250px/m) | 5,6 - 190 m |
| Perceiving (25px/m) | 70,3 - 1800 m |
| Functions |  |
| Analysis | Motion Detection |
| Motion detector | Yes |
| Digital Noise Reduction (DNR) | Yes |
| Menu languages | Chinese, English, Japanese |
| Interfaces / Inputs/ Outputs |  |
| Number of alarm outputs | 1 |
| External connections | 1x SDI Signal (BNC),1x HD/SD Analog Signal (BNC),12VDC power input,RS-485,external day&night switching,Alarm output |
| Interface Protocols | Pelco D, Pelco P, UTC (HD Signal Format) |
| Control Interfaces | Coaxitron (HD Protocol), RS-485 |
| Video Outputs | Composite (FBAS) |
| Parts supplied |  |
| Scope of delivery | manual |
| Lens |  |
| Horizontal angle of view | 65,4 - 2,2° |
| Vertical angle of view | 38,3 - 1,3° |
| Focal length | 4,6 - 136 mm |
| Aperture Range (F) | F1.63-F4.85 |
| Lens Type | Auto Focus Zoom |
| Zoomfactor (optical) | 30 |
| Video |  |
| Resolution (extended) | 1920x1080,1280x720 |
| Signal Format | Composite (FBAS), EX-SDI @ 2MP, HD-SDI @ 2MP, HD-TVI @ 2MP |
| Horizontal resolution | 700 TVL |
| Product labeling: |  |
| EAN/GTIN | 4260229841839 |
| Manufacturer order number | 240897 |
| Brand | eneo |

Additional Productinformation

Related products

| eneo WD-30 | Wall Mount Bracket with Ball Joint, 1/4" Camera Fixing Screw, Pantone Cool Grey 1C |
| --- | --- |

Recommendations

| eneo MAM-5DC1004M0A | Distributor, 1x BNC Input HD-TVI, CVI, AHD, CVBS, 4x BNC Output |
| --- | --- |
| eneo MAM-5MM1001M0A | Converter, HD-CVI, HD-TVI, AHD, Composite to HDMI, VGA, Composite, Indoor |
| eneo TVA-1201TRA | UTP Transmitter+Receiver, UTP, BNC, 250m, Indoor, HD TVI CVI, AHD, CVBS, Active, 12V |
| eneo TVA-1201TRP | UTP Transmitter+Receiver, UTP, BNC, 150m, Indoor, HD TVI CVI, AHD, CVBS, Passive |
| jetrics cable BNC BS-BS-1 | Videokabel mit 2 x BNC-Stecker 1m, 75 Ohm |
| jetrics cable BNC BS-BS-2 | Videokabel mit 2 x BNC-Stecker 2m, 75 Ohm |
| jetrics cable BNC BS-BS-3 | Videokabel mit 2 x BNC-Stecker 5m, 75 Ohm |
| jetrics cable BNC BS-BS-3 | Videokabel mit 2 x BNC-Stecker 3m, 75 Ohm |

Services

| PAINTING SERVICE basic costs | once per RAL color and delivery lot |
| --- | --- |
| PAINTING SERVICE bullet camera | in RAL color |
| Vorkonfiguration analoge Kamera | Preconfiguration 1x analog camera Function test, FW update |