Specifications text

Article number

241967

Type

MPC-52A0010M0B

Short description

1/3" HD Camera, 1920x1080, 10x AF Zoom 5,1-51mm, T-WDR, HD-SDI, EX-SDI, TVI, Composite

Main Features

1/2,8" 2,0 Megapixel Sony CMOS-Sensor

Resolution max. 1920x1080, 1080p@60/50fps

Optical 10x Auto Focus Zoom, 5,1~51mm

Multisignal Output HD-SDI, EX-SDI, TVI, Composite

WDR 120dB, DNR, motion detector

Digital noise reduction (2D+3D)

Image stabilization, privacy masking, defog

Privacy Masking, Defog

Slow Speed Shutter

Compact Housing

Power Supply 12VDC

Other Features (Specifications)

| Brand | eneo |
| --- | --- |
| Dimensions (HxWxD) | 62,2x59,2x99,6mm |
| Resolution Pixels | 1280x720, 1920x1080 |
| Horizontal resolution | 700 TVL |
| System | True Day&Night |
| Sensor size | 1/2.8" |
| Imager | CMOS |
| Day&Night switch | ICR |
| Sensitivity | 0,003 Lux |
| Digital Noise Reduction | yes |
| Focal Length | 5,1 - 51 mm |
| Iris Control | DC |
| Zoom Factor | 10x |
| Digital Zoom | yes |
| Horizontal Angle of View | 57 - 6,2° |
| Vertical Angle of View | 34 - 3,5° |
| MOD (Mininum Object Distance) | 0,1 m |
| llumination | no |
| Housing | Indoor |
| Color (housing) | black, silbermetallic |
| Hosuing material | Aluminium |
| Alarm outputs | 1 |
| Serial interfaces | Coaxitron (HD Protocol), RS-485 |
| Analysis functions | Motion detection |
| Supply voltage | 12VDC |
| Power consumption | 3W |
| Vandal proof | no |
| Operating temperature | -10°C ~ +50°C, @ Humidity 90% or less (non-condensing) |
| Low Speed Shutter | yes |
| Backlight Compensation | BLC, HLC |
| Wide Dynamic Range (WDR) | WDR 120dB |
| Motion Detector | yes |
| Signal format | Composite (FBAS), EX-SDI (270 Mbps), HD-SDI (1.485 Gbps), HD-TVI @ 2MP |
| Camera type | Zoom Camera |
| Frame rate | 25/30fps @ 1280x720p, 25/30fps @ 1920x1080p, 50/60fps @ 1280x720p, 50/60fps @ 1920x1080p |
| Aperture range (F) | F1.6~F1.8 |
| Weight | 360 g |
| Manufacturer order number | 241967 |

Additional Productinformation

Related products

| MAM-5DC1004M0A | Distributor, 1x BNC Input HD-TVI, CVI, AHD, CVBS, 4x BNC Output |
| --- | --- |
| MAM-5MM1001M0A | Converter, HD-CVI, HD-TVI, AHD, Composite to HDMI, VGA, Composite, Indoor |
| TVA-1201TRA | UTP Transmitter+Receiver, UTP, BNC, 250m, Indoor, HD TVI CVI, AHD, CVBS, Active, 12V |
| TVA-1201TRP | UTP Transmitter+Receiver, UTP, BNC, 150m, Indoor, HD TVI CVI, AHD, CVBS, Passive |
| WD-30 | Wall Mount Bracket with Ball Joint, 1/4" Camera Fixing Screw, Pantone Cool Grey 1C |

Recommendations

| LACKIERUNG Bulletkamera | Extra charge for special paint bullet camera in RAL color |
| --- | --- |

Services

| LACKIERUNG Bulletkamera | Extra charge for special paint bullet camera in RAL color |
| --- | --- |
| LACKIERUNG Grundkosten | Painting basic costs once per RAL color and delivery lot |
| Vorkonfiguration analoge Kamera | Preconfiguration 1x analog camera Function test, FW update |