Specifications text

Article number

239241

Type

ISM-42F0036MCB

Short description

1/2.8" Mini Network Tube Camera, 1920x1080, Day&Night, 3DNR, 3.6mm, 12VDC, PoE, IP66

Main Features

1/2.8" Sony STARVIS CMOS image sensor

Resolution max. 1920x1080 (Full HD)

Video Compression Format: H.264/MJPEG

Motion Detection, Low Speed Shutter

Built in SD micro card slot

Backlight compensation 3DNR and D-WDR

Supports privacy zones and corridor mode

Lens 3.6mm (further lenses upon request)

Power Supply 12VDC (1.8W) or PoE (IEEE802.3af)

Connection cable (RJ45) for PoE and power supply

Operation Temperature -20°C - +50°C, IP66

Supports ONVIF Profile S

Other Features (Specifications)

| Brand | eneo |
| --- | --- |
| Dimensions (HxWxD) | 21,5x94,5 |
| Series | eneo ISM-Serie |
| Camera Type | Board Camera, Nadelöhr/Mini, Zylinderkamera |
| Resolution Standard | HD 1080p |
| System | electronic Day&Night |
| Sensor size | 1/2.8" |
| Imager | CMOS |
| Video compression | MJPEG |
| Image resolution max. | 1920x1080 |
| Image Transmission Rate | 30 fps |
| Day&Night switch | Electronic |
| Digital Noise Reduction | yes |
| Focal Length | 3,6 mm |
| Iris Control | no |
| Horizontal Angle of View | 89,6° |
| Vertical Angle of View | 47,8° |
| MOD (Mininum Object Distance) | 0,3 m |
| Housing | Indoor, Outdoor |
| Color (housing) | black |
| Hosuing material | Metal |
| Heater | no |
| Blower | no |
| Alarm inputs | nein |
| Alarm outputs | no |
| Audio support | no |
| Ethernet interface | 100Base-TX, 10Base-T |
| Internal storage | microSD-Kartenslot |
| Analysis functions | Motion detection |
| Mounting | Ceiling mount, Wall mount |
| Supply voltage | 12VDC, PoE |
| Power consumption | 1,4W at 12VDC, 2,5W at PoE |
| Power over Ethernet | IEEE 802.3af |
| Vandal proof | no |
| Operating temperature | -20°C ~ +50°C |
| ONVIF | ONVIF Profile S |
| Protection rating | IP67 |
| Low Speed Shutter | yes |
| Backlight Compensation | BLC |
| Wide Dynamic Range (WDR) | D-WDR |
| Motion Detector | yes |
| Camera type | Modul, Platine |
| Remote configuration | Browser |
| Weight | 95g |
| Manufacturer order number | 239241 |

Additional Productinformation

<p style='margin:0cm;font-size:15px;font-family:"Calibri",sans-serif;'>Kleine und kompakte IP-Video-Sicherheitskameras sind vielseitig einsetzbar und können in verschiedenen Umgebungen zur Überwachung und Sicherheit genutzt werden.</p><p style='margin:0cm;font-size:15px;font-family:"Calibri",sans-serif;'>Aufgrund Ihrer kompakten Baugröße können diese Kameras überall dort eingesetzt werden, wo nur geringer Platz zur Verfügung steht oder eine möglichst diskrete Überwachung stattfinden soll.</p><p style='margin:0cm;font-size:15px;font-family:"Calibri",sans-serif;'> </p><p style='margin:0cm;font-size:15px;font-family:"Calibri",sans-serif;'>Hier sind einige mögliche Anwendungsbereiche:</p><p style='margin:0cm;font-size:15px;font-family:"Calibri",sans-serif;'>Industrielle Überwachung von Maschinen und Produktionsprozessen</p><p style='margin:0cm;font-size:15px;font-family:"Calibri",sans-serif;'>Abwasser Kanalüberwachung</p><p style='margin:0cm;font-size:15px;font-family:"Calibri",sans-serif;'>Platzsparende Überwachung von Geschäftsräumen und oder Privathaushalten</p><p style='margin:0cm;font-size:15px;font-family:"Calibri",sans-serif;'>Einsatz in Mobilen Anwendungen wie z. B. Fahrzeugen, LKW´s, Busse & Bahnen</p><p style='margin:0cm;font-size:15px;font-family:"Calibri",sans-serif;'>Kundenspezifische Anpassungen für spezielle Anwendungen möglich</p>

Related products

| eneo Site Manager | Management Software for eneo IP products |
| --- | --- |

Recommendations

Services

| Vorkonfiguration IP-Kamera | Preconfiguration 1x IP camera Function test, FW update |
| --- | --- |