Specifications text

Article number

239241

Type

eneo ISM-42F0036MCB

Short description

1/2.8" Mini Network Tube Camera, 1920x1080, Day&Night, 3DNR, 3.6mm, 12VDC, PoE, IP66

Main Features

1/2.8" Sony STARVIS CMOS image sensor

Resolution max. 1920x1080 (Full HD)

Video Compression Format: H.264/MJPEG

Motion Detection, Low Speed Shutter

Built in SD micro card slot

Backlight compensation 3DNR and D-WDR

Supports privacy zones and corridor mode

Lens 3.6mm (further lenses upon request)

Power Supply 12VDC (1.8W) or PoE (IEEE802.3af)

Connection cable (RJ45) for PoE and power supply

Operation Temperature -20°C - +50°C, IP66

Supports ONVIF Profile S

Other Features (Specifications)

| General |  |
| --- | --- |
| Heating | No |
| Dimensions (WxHxD) (extended) | 21,5x94,5 |
| Weight | 0.095 kg |
| Text overlay | available |
| Color (Housing) | black |
| Housing type | Inside, Outside |
| Housing material | Metal |
| Camera Type | Board Camera, Cylinder camera |
| Power consumption | 2.5 W |
| Mounting type | Ceiling, Wall |
| ONVIF | ONVIF Profile S |
| Power over Ethernet | IEEE 802.3af |
| Protection | IP67 |
| Power supply | 12VDC, Poe |
| Temperature Range (Operating) | -20°C ~ +50°C |
| Wide Dynamic Range (WDR) | D-WDR |
| Camera |  |
| Backlight compensation | Yes |
| Low Speed Shutter | Yes |
| Resolution standard | 2 MP |
| Recording sensor | CMOS |
| Chip size | 1/2,8" |
| Camera shape | Module, Printed circuit board |
| Day/night mode | Automatic,Day,Night,Time |
| White balance | Automatic,Manual |
| System | Electronic Day&Night |
| Day/night switching | Electronic |
| Display |  |
| Image resolution max. | 1920x1080 |
| DORI |  |
| Watch (62px/m) | 22,6 m |
| Detect (125px/m) | 10,8 m |
| Identify (250px/m) | 5,2 m |
| Perceiving (25px/m) | 56 m |
| Functions |  |
| Analysis | Motion Detection |
| Audio support | No |
| Motion detector | Yes |
| Digital Noise Reduction (DNR) | Yes |
| Browser | Edge, Firefox, Google Chrome, Opera |
| Menu languages | English, German, Japanese, Korean |
| Password | Administrator and User |
| Internal memory | microSD card slot |
| Notes |  |
| Remark | This model is also available with a resolution of 4/5MP on request. |
| Interfaces / Inputs/ Outputs |  |
| External connections | RJ-45,12VDC power input,connection cable approx.15cm,Micro SD card slot 512 GB |
| Parts supplied |  |
| Scope of delivery | Quick installation guide,mounting bracket,3x mounting screws for the bracket,1x Allen wrench,1x waterproof cover for the RJ45 connector |
| Lens |  |
| Horizontal angle of view | 89,6° |
| Vertical angle of view | 47,8° |
| Iris Control | No |
| Focal length | 3,6 mm |
| MOD (Minimum Object Distance) | 0.3 m |
| Lens Type | Prime lens |
| Video |  |
| Image transfer rate max. | 30 fps |
| Resolution (extended) | 1920x1080 (16:9),1280x720 (16:9),960x540 (16:9),640x480 (4:3),320x240 (4:3) |
| Data stream types | 2x H.264,1x H.264 oder MJPEG simultan |
| Video Compression | MJPEG |
| Product labeling: |  |
| Manufacturer order number | 239241 |
| Series | eneo ISM-Serie |
| Brand | eneo |

Additional Productinformation

Related products

| eneo Site Manager | Management Software for eneo IP products |
| --- | --- |

Recommendations

| eneo NSMI-4004M0A | Gigabit Switch Industrial, Managed, 4x PoE, 2xSFP, 30W each Port, -40°C~75°C, 44~57 VDC, w/o power supply |
| --- | --- |
| eneo NSMI-4008M0A | Gigabit Switch Industrial, Managed, 8x PoE, 2xSFP, 30W each Port, -40°C~75°C, 44~57 VDC, w/o power supply |
| eneo NSMR-2008M0A | Gigabit Switch Rack, Managed, 8x PoE, 2xSFP, max. 30W each Port, internal Power Supply max. 150W |
| eneo NSMR-2016M0A | Gigabit Switch Rack, Managed, 16x PoE, 2xSFP, max. 30W each Port, integrated Power Suppply max. 250W |
| eneo NSMR-2024M0A | Gigabit Switch Rack, Managed, 24x PoE, 2xSFP, max. 30W each Port, integrated Power Supply max. 400W |
| eneo NSUI-3004M0A | Gigabit Switch Industrial, Unmanaged, 4x PoE, 1xRJ45, 1xSFP, 30W each Port, -40°C~75°C, 44~57 VDC, w/o power supply |
| eneo NSUI-3008M0A | Gigabit Switch Industrial, Unmanaged, 8x PoE, 2xSFP, 30W each Port, -40°C~75°C, 44~57 VDC, w/o power supply |
| eneo NSUR-1008M0A | Gigabit Switch Rack, Unmanaged, 8x PoE, 2xSFP, max. 30W each Port, integrated Power Supply max. 150W |
| eneo NSUR-1016M0A | Gigabit Switch Rack, Unmanaged, 16x PoE, 2xSFP, max. 30W each Port, integrated Power Supply max. 250W |
| eneo NSUR-1024M0A | Gigabit Switch Rack, Unmanaged, 24x PoE, 2xSFP, max. 30W each Port, integrated Power Supply max. 400W |
| Jetrics jetcat6SFTP-GN-0,5/1 |  |
| Jetrics jetcat6SFTP-GN-1/1 |  |
| Jetrics jetcat6SFTP-GN-2/1 |  |
| Jetrics jetcat6SFTP-GN-3/1 |  |
| Jetrics jetcat6SFTP-GN-7,5/1 |  |
| Jetrics jetpower-PSU-XS-11C Steckernetzteil |  |

Services

| Vorkonfiguration IP-Kamera | Preconfiguration 1x IP camera Function test, FW update |
| --- | --- |