Specifications text

Article number

239240

Type

ISM-42F0036MMB

Short description

1/2,8" Mini Network Modul Camera, 1920x1080, Day&Night, 3DNR, 3,6mm, 12VDC, PoE, Indoor

Main Features

1/2.8" Sony STARVIS CMOS image sensor

Resolution max. 1920x1080 (Full HD)

Video Compression Format: H.264/MJPEG

Motion Detection, Low Speed Shutter

Built in SD micro card slot

Backlight compensation 3DNR and D-WDR

Supports privacy zones and corridor mode

Lens 3.6mm (further lenses upon request)

Power Supply 12VDC (1.8W) or PoE (IEEE802.3af)

Connection cable (RJ45) for PoE and power supply

Operation Temperature -20°C - +50°C

Supports ONVIF Profile S

Other Features (Specifications)

| Dimensions (HxWxD) | 36x36x14mm |
| --- | --- |
| Series | eneo ISM-Serie |
| Camera Type | Board Camera, Nadelöhr/Mini |
| Resolution Standard | HD 1080p |
| System | electronic Day&Night |
| Sensor size | 1/2.8" |
| Imager | CMOS |
| Video compression | H.264, MJPEG |
| Image resolution max. | 1920x1080 |
| Image Transmission Rate | 30 fps |
| Day&Night switch | Electronic |
| Low Speed Shutter | yes |
| Digital Noise Reduction | yes |
| Backlight Compensation | BLC |
| Wide Dynamic Range (WDR) | D-WDR |
| Motion Detector | yes |
| Focal Length | 3,6 mm |
| Iris Control | no |
| Horizontal Angle of View | 89,6° |
| Vertical Angle of View | 47,8° |
| MOD (Mininum Object Distance) | 0,3 m |
| Housing | Indoor |
| Color (housing) | black |
| Hosuing material | Metal |
| Heater | no |
| Blower | no |
| Alarm inputs | nein |
| Alarm outputs | no |
| Audio support | no |
| Ethernet interface | 100Base-TX, 10Base-T |
| Internal storage | microSD-Kartenslot |
| Analysis functions | Motion detection |
| Mounting | Ceiling mount, Wall mount |
| Supply voltage | 12VDC, PoE |
| Power consumption | 1.4W |
| Power over Ethernet | IEEE 802.3af |
| Vandal proof | no |
| Operating temperature | -20°C ~ +50°C |
| ONVIF | ONVIF Profile S |
| Camera type | Modul, Platine |
| Remote configuration | Browser |
| Weight | 45g |
| Manufacturer order number | 239240 |
| Brand | eneo |

Additional Productinformation

Kleine und kompakte IP-Video-Sicherheitskameras sind vielseitig einsetzbar und können in verschiedenen Umgebungen zur Überwachung und Sicherheit genutzt werden.Aufgrund Ihrer kompakten Baugröße können diese Kameras überall dort eingesetzt werden, wo nur geringer Platz zur Verfügung steht oder eine möglichst diskrete Überwachung stattfinden soll. Hier sind einige mögliche Anwendungsbereiche:Industrielle Überwachung von Maschinen und ProduktionsprozessenAbwasser KanalüberwachungPlatzsparende Überwachung von Geschäftsräumen und oder PrivathaushaltenEinsatz in Mobilen Anwendungen wie z. B. Fahrzeugen, LKW´s, Busse & BahnenKundenspezifische Anpassungen für spezielle Anwendungen möglich

Related products

| eneo Site Manager | Management Software for eneo IP products |
| --- | --- |

Recommendations

Services

| Vorkonfiguration IP-Kamera | Preconfiguration 1x IP camera Function test, FW update |
| --- | --- |