Specifications text

Article number

241589

Type

INM-45F0036MMA

Short description

Mini Network Camera, 2880x1620, TDay/Night, WDR, AI, 3,6mm, 3D DNR, 12VDC, PoE

Main Features

1/2.7" CMOS sensor, maximum resolution 5 MP/30fps

AI analysis functions including object classification

WDR 120dB, 3D DNR noise reduction

Lens with fixed focal length 3.6mm (HFoV 91°)

Smart codec H.265/H.264, multi-streaming

Browser-independent (IE, Edge, Safari, Firefox, Chrome)

Supports ONVIF profiles S/G/T/M

Multilingual menu navigation (OSD)

Supports audio

12VDC or PoE power supply

NDAA compliance

Very compact design

Other Features (Specifications)

| EAN/GTIN | 4260229840696 |
| --- | --- |
| Audio support | Yes |
| Motion detector | Yes |
| Digital Noise Reduction (DNR) | Yes |
| Multicast | Yes |
| Integrated wall mount | Yes |
| Backlight compensation | Yes |
| Resolution standard | 5 MP |
| Recording sensor | CMOS, Sensor SC500AI |
| Lighting | no |
| Image resolution max. | 2880x1620 |
| Image transfer rate max. | 30 fps |
| Horizontal angle of view | 91° |
| Focal length | 3,6 mm |
| Chip size | 1/2,7" |
| Dimensions (WxHxD) (extended) | 44,0x36,4x53,3mm |
| Alarm processing | FTP,E-Mail,Cloud Upload |
| Audio | 2 Way Audio |
| Resolution (extended) | 2880x1620 (16:9),2560x1440 (16:9),2304x1296 (16:9),1920x1080 (16:9),1280x720 (16:9),640x480 (4:3),320x240 (4:3) |
| Watch (62px/m) | 23.1 m |
| Picture size | 16:9, 4:3, VerticalView |
| Aperture Range (F) | F1.6 |
| Browser | Edge, Firefox, Google Chrome, Safari |
| Data stream types | Triple-Stream(Main/Sub/Mobile) |
| Detect (125px/m) | 10.6 m |
| ESC | 1/5 ~ 1/20.000sec. |
| Flickerless function | Yes |
| Identify (250px/m) | 4.5 m |
| Camera shape | Module |
| Scope of delivery | bag of screws,manual |
| Manual Electric Shutter (MES) | 1-5~1-20,000sec. |
| Menu languages | English, French, German, Italian, Japanese, Portuguese, Russian, Spanish, Turkish |
| Password | Administrator and User |
| Privacy Zone | Areas-ON/OFF switchable |
| SDK | on request |
| Signal-to-noise ratio | 39 dB |
| Day/night mode | Automatic,Day,Night,Image,Time |
| Text overlay | Switchable on/off |
| Perceiving (25px/m) | 59.8 m |
| White balance | Automatic,Manual |
| Color (Housing) | black |
| Housing type | Inside |
| Housing material | Metal |
| Internal memory | no |
| Camera Type | Board Camera |
| Photosensitivity | 0,004 Lux bei F1,6 |
| MOD (Minimum Object Distance) | 2 m |
| Mounting type | Built-in module |
| Network Protocols | DDNS, DHCP, DNA, FTP, HTTP, HTTPS, NTP, RTP, RTSP, SMTP, TCP-IP, UPnP |
| Lens Type | Prime lens |
| ONVIF | ONVIF Profile G, ONVIF Profile M, ONVIF Profile S, ONVIF Profile T |
| Manufacturer order number | 241589 |
| Power over Ethernet | IEEE 802.3af |
| Series | eneo IN-Serie |
| Control Interfaces | Ethernet |
| Power supply | 12VDC, Poe |
| System | Electronic Day&Night |
| Day/night switching | Electronic |
| Temperature Range (Operating) | -35°C ~ +60°C |
| Video Compression | H.264, H.264+, H.265, H.265+, M-JPEG |
| Wide Dynamic Range (WDR) | WDR 120dB |
| Zoom Factor (Digital) | 1 |
| Brand | eneo |

Additional Productinformation

Related products

| eneo INsight Installation Tool | Supports adjustments for cameras with AI features for IN-series |
| --- | --- |
| eneo INsight mobile | Mobile App for recorder IN- and MN-series |
| eneo Site Manager | Management Software for eneo IP products |

Recommendations

| eneo INsight mobile | Mobile App for recorder IN- and MN-series |
| --- | --- |
| Vorkonfiguration IP-Kamera | Preconfiguration 1x IP camera Function test, FW update |

Services

| Vorkonfiguration IP-Kamera | Preconfiguration 1x IP camera Function test, FW update |
| --- | --- |