Specifications text

Article number

241589

Type

eneo INM-45F0036MMA

Short description

Mini Network Camera, 2880x1620, TDay/Night, WDR, AI, 3,6mm, 3D DNR, 12VDC, PoE

Main Features

1/2.7" CMOS sensor, maximum resolution 5 MP/30fps

AI analysis functions including object classification

WDR 120dB, 3D DNR noise reduction

Lens with fixed focal length 3.6mm (HFoV 91°)

Smart codec H.265/H.264, multi-streaming

Browser-independent (IE, Edge, Safari, Firefox, Chrome)

Supports ONVIF profiles S/G/T/M

Multilingual menu navigation (OSD)

Supports audio

12VDC or PoE power supply

NDAA compliance

Very compact design

Other Features (Specifications)

| General |  |
| --- | --- |
| Integrated wall mount | Yes |
| Audio compression | G.711 |
| Lighting | No |
| Dimensions (WxHxD) (extended) | 44,0x36,4x53,3mm |
| ESC | 1/5 ~ 1/20.000sec. |
| Weight | 0.06 kg |
| SDK | on request |
| Text overlay | Switchable on/off |
| Color (Housing) | black |
| Housing type | Inside |
| Housing material | Metal |
| Camera Type | Board Camera |
| Power consumption | 3.62 W |
| Mounting type | Built-in module |
| ONVIF | ONVIF Profile G, ONVIF Profile M, ONVIF Profile S, ONVIF Profile T |
| Power over Ethernet | IEEE 802.3af |
| Power supply | 12VDC, Poe |
| Temperature Range (Operating) | -35°C ~ +60°C |
| Wide Dynamic Range (WDR) | WDR 120dB |
| Camera |  |
| Backlight compensation | Yes |
| Low Speed Shutter | No |
| Resolution standard | 5 MP |
| Recording sensor | Chipset SSC30KQ, CMOS, Sensor SC500AI |
| Chip size | 1/2,7" |
| Flickerless function | Yes |
| Camera shape | Module |
| Manual Electric Shutter (MES) | 1-5~1-20,000sec. |
| Signal-to-noise ratio | 39 dB |
| Day/night mode | Automatic,Day,Night,Image,Time |
| White balance | Automatic,Manual |
| Photosensitivity | 0,004 Lux bei F1,6 |
| System | Electronic Day&Night |
| Day/night switching | Electronic |
| Display |  |
| Image resolution max. | 2880x1620 |
| DORI |  |
| Watch (62px/m) | 25,3 m |
| Detect (125px/m) | 12 m |
| Identify (250px/m) | 5,6 m |
| Perceiving (25px/m) | 64 m |
| Functions |  |
| Analysis | Abandoned and Removed Objects (SOD), Attendance (FA), Crowds (CD), Enter Region, Face Capture (FD), Face Recognition (FR), Heat Map (HM), Intrusion, Intrusion (PID), Leave Region, License plate recognition (LPD), License plate recognition (LPR), Line crossing (LCD), Manipulation (TD), Object Classification Human / Vehicle (PD&VD), Passage count (CC), Queues (QD), Rare Sounds (RSD) |
| Audio support | Yes |
| Motion detector | Yes |
| Digital Noise Reduction (DNR) | Yes |
| Multicast | Yes |
| Alarm processing | FTP,E-Mail,Cloud Upload |
| Audio | 2 Way Audio |
| Browser | Edge, Firefox, Google Chrome, Safari |
| Menu languages | English, French, German, Italian, Japanese, Portuguese, Russian, Spanish, Turkish |
| Password | Administrator and User |
| Privacy Zone | Areas-ON/OFF switchable |
| Interfaces / Inputs/ Outputs |  |
| Ethernet interface | 100Base-TX, 10Base-T |
| Control Interfaces | Ethernet |
| Parts supplied |  |
| Scope of delivery | Quick installation guide,1x wall plug with screw |
| Network |  |
| Network Protocols | DDNS, DHCP, DNA, FTP, HTTP, HTTPS, IEE 802.1, NTP, RTP, RTSP, SMTP, TCP-IP, UPnP |
| Lens |  |
| Horizontal angle of view | 91° |
| Vertical angle of view | 46° |
| Iris Control | No |
| Focal length | 3,6 mm |
| Aperture Range (F) | F1.6 |
| MOD (Minimum Object Distance) | 2 m |
| Lens Type | Prime lens |
| Video |  |
| Image transfer rate max. | 30 fps |
| Resolution (extended) | 2880x1620 (16:9),2560x1440 (16:9),2304x1296 (16:9),1920x1080 (16:9),1280x720 (16:9),640x480 (4:3),320x240 (4:3) |
| Picture size | 16:9, 4:3, VerticalView |
| Data stream types | Triple-Stream(Main/Sub/Mobile) |
| Video Compression | H.264, H.264+, H.265, H.265+, M-JPEG |
| Product labeling: |  |
| EAN/GTIN | 4260229840696 |
| Manufacturer order number | 241589 |
| Series | eneo IN-Serie |
| Brand | eneo |

Additional Productinformation

Related products

| eneo INsight | Control software for recorder IN- and MN-series |
| --- | --- |
| eneo INsight Installation Tool | Supports adjustments for cameras with AI features for IN-series |
| eneo INsight mobile | Mobile App for recorder IN- and MN-series |
| eneo Site Manager | Management Software for eneo IP products |

Recommendations

| eneo NSMI-4004M0A | Gigabit Switch Industrial, Managed, 4x PoE, 2xSFP, 30W each Port, -40°C~75°C, 44~57 VDC, w/o power supply |
| --- | --- |
| eneo NSMI-4008M0A | Gigabit Switch Industrial, Managed, 8x PoE, 2xSFP, 30W each Port, -40°C~75°C, 44~57 VDC, w/o power supply |
| eneo NSMR-2008M0A | Gigabit Switch Rack, Managed, 8x PoE, 2xSFP, max. 30W each Port, internal Power Supply max. 150W |
| eneo NSMR-2016M0A | Gigabit Switch Rack, Managed, 16x PoE, 2xSFP, max. 30W each Port, integrated Power Suppply max. 250W |
| eneo NSMR-2024M0A | Gigabit Switch Rack, Managed, 24x PoE, 2xSFP, max. 30W each Port, integrated Power Supply max. 400W |
| eneo NSUI-3004M0A | Gigabit Switch Industrial, Unmanaged, 4x PoE, 1xRJ45, 1xSFP, 30W each Port, -40°C~75°C, 44~57 VDC, w/o power supply |
| eneo NSUI-3008M0A | Gigabit Switch Industrial, Unmanaged, 8x PoE, 2xSFP, 30W each Port, -40°C~75°C, 44~57 VDC, w/o power supply |
| eneo NSUR-1008M0A | Gigabit Switch Rack, Unmanaged, 8x PoE, 2xSFP, max. 30W each Port, integrated Power Supply max. 150W |
| eneo NSUR-1016M0A | Gigabit Switch Rack, Unmanaged, 16x PoE, 2xSFP, max. 30W each Port, integrated Power Supply max. 250W |
| eneo NSUR-1024M0A | Gigabit Switch Rack, Unmanaged, 24x PoE, 2xSFP, max. 30W each Port, integrated Power Supply max. 400W |
| Jetrics jetcat6SFTP-GN-0,5/1 |  |
| Jetrics jetcat6SFTP-GN-1/1 |  |
| Jetrics jetcat6SFTP-GN-2/1 |  |
| Jetrics jetcat6SFTP-GN-3/1 |  |
| Jetrics jetcat6SFTP-GN-7,5/1 |  |

Services

| Vorkonfiguration IP-Kamera | Preconfiguration 1x IP camera Function test, FW update |
| --- | --- |