Specifications text

Article number

238644

Type

IND-58F0002MTPA

Short description

Network Camera Turret, Panorama 180°, 2x Sensor, Lens 2.8mm, max. 3840x2160, AI, DWDR, IR, WL, PoE, 12VDC, IP67

Main Features

Multisensor 2x 1/2.7’ CMOS sensor, panorama

Maximum resolution 5120x1440 pixels each, 15fps

Panorama ultra wide-angle camera 180° HFoV, 32:9

AI analysis functions including object classification

WDR, BLC, HLC and 3D-DNR noise reduction

Lenses with 2.8mm fixed focal length

Smart Codec H.265/H.264, dual streaming

Browser-independent (Edge, Safari, Firefox, Chrome)

Supports ONVIF profiles S/T/G/M and audio

Local storage via micro SD card

NDAA compliance

Integrated IR and white light LED illumination, IP67

Other Features (Specifications)

| EAN/GTIN | 4260229840726 |
| --- | --- |
| Audio support | Yes |
| Resolution standard | 8 MP |
| Recording sensor | CMOS |
| Lighting | Infrared, White |
| Motion detector | No |
| Image resolution max. | 5120x1440 |
| Image transfer rate max. | 15 fps |
| Horizontal angle of view | 180° |
| Focal length | 2,8 mm |
| Chip size | 1/2,7" |
| Digital Noise Reduction (DNR) | Yes |
| Dimensions (WxHxD) (extended) | 112,4x135mm |
| Alarm processing | recording internal storage (SD card),E-Mail,HTTP,FTP,TCP,UDP,Red & bule flashing lights,Siren,Cloud Upload |
| Audio | 2 Way Audio, Built-in microphone, Built-in speaker |
| Resolution (extended) | 5120x1440,4608x1296,4096x1152,3840x1080,3072x864,1536x432,1024x288 |
| Remark | To guarantee you an optimum user experience,we recommend that you only use components of the same brand in a system. If several brands are used in combination,eneo cannot guarantee functionality. If combined use is necessary,we offer a compatibility check |
| Watch (62px/m) | 40.1 m |
| Browser | Edge, Firefox, Google Chrome, Safari |
| Data stream types | Dual Streaming |
| Detect (125px/m) | 10.7 m |
| External connections | RJ-45,12VDC input (DC plug),connection cable approx.30cm,micro SD card slot 256 GB |
| Flickerless function | Yes |
| Identify (250px/m) | 10.7 m |
| Camera shape | Minsters |
| Scope of delivery | manual,bag of screws |
| Menu languages | English, French, German, Italian, Japanese, Portuguese, Russian, Spanish, Turkish |
| Multicast | Yes |
| Password | Administrator and User |
| SDK | upon request |
| Sunshade canopy | No |
| Day/night mode | Day,Night,Automatic,Time |
| Perceiving (25px/m) | 108.3 m |
| Integrated wall mount | No |
| White balance | Automatic,Manual |
| Backlight compensation | No |
| Housing type | Outside |
| Housing material | Metal |
| Heating | Yes |
| Internal memory | microSD card slot |
| Camera Type | Multi-sensor camera |
| Photosensitivity | 0,001 Lux bei F1,6 |
| Low Speed Shutter | No |
| MOD (Minimum Object Distance) | 2 m |
| Mounting type | Ceiling, Wall |
| Lens Type | Prime lens |
| ONVIF | ONVIF Profile G, ONVIF Profile M, ONVIF Profile S, ONVIF Profile T |
| Manufacturer order number | 238644 |
| Power over Ethernet | IEEE 802.3af |
| Protection | IP67 |
| Series | eneo IN-Serie |
| Control Interfaces | Ethernet |
| Power supply | 12VDC, Poe |
| System | 24h Color, True Day&Night |
| Day/night switching | Electronic |
| Temperature Range (Operating) | -30°C ~ +60°C,@ Luftfeuchtigkeit 90% oder weniger (nicht kondensierend) |
| Vandalism-resistant | No |
| Video Compression | H.264, H.264+, H.265, H.265+, M-JPEG |
| Wide Dynamic Range (WDR) | D-WDR, D-WDR, D-WDR, D-WDR, D-WDR, D-WDR, D-WDR |
| Wiper | No |
| Brand | eneo |

Additional Productinformation

Related products

| AK-20 | Camera connection box for Wall and ceiling mounting, IN Series Bullet Cameras |
| --- | --- |
| AK-21 | Camera connection box for Wall and ceiling mounting, IN Series Bullet Cameras |
| AK-24 | Dual camera connection box for selected Bullet or Fix Dome IN-Series cameras |
| DOME-WB8 | Wall bracket for PTZ dome INP-54M2812M0A and IND-5 Dome Series |
| eneo INsight Installation Tool | Supports adjustments for cameras with AI features for IN-series |
| eneo Site Manager | Management Software for eneo IP products |
| KBD-3NET | Network keyboard with joystick, supports ONVIF protocol, RJ45, 12VDC |

Recommendations

| jetpower-PSU-XS-11C Steckernetzteil | Universal Netzteil 12V/1,5A ID 2.1 x OD 5.5, 11mm, inlusive Adapter von 11 auf 9mm Länge |
| --- | --- |
| LACKIERUNG Fix-Dome-Kamera | Extra charge for special paint fixed dome in RAL color |

Services

| LACKIERUNG Fix-Dome-Kamera | Extra charge for special paint fixed dome in RAL color |
| --- | --- |
| LACKIERUNG Grundkosten | Painting basic costs once per RAL color and delivery lot |
| Vorkonfiguration IP-Kamera | Preconfiguration 1x IP camera Function test, FW update |