Specifications text

Article number

238644

Type

IND-58F0002MTPA

Short description

Network Dome, Turret, 2x Sensor, 180° Panorama, 3840x2160, 2.8mm, AI, DWDR, PoE, 12VDC, IP67

Main Features

Multisensor 2x 1/2.7? CMOS sensor, panorama

Maximum resolution 5120x1440 pixels each, 15fps

Panorama ultra wide-angle camera 180° HFoV, 32:9

AI analysis functions including object classification

WDR, BLC, HLC and 3D-DNR noise reduction

Lenses with 2.8mm fixed focal length

Smart Codec H.265/H.264, dual streaming

Browser-independent (Edge, Safari, Firefox, Chrome)

Supports ONVIF profiles S/T/G and audio

Local storage via micro SD card

NDAA compliance

Integrated IR and white light LED illumination, IP67

Other Features (Specifications)

| EAN/GTIN | 4260229840726 |
| --- | --- |
| Dimensions (HxWxD) | 112,4x135mm |
| Series | eneo IN-Series |
| Camera Type | Multisensor Camera |
| Resolution Standard | 8 Megapixel |
| System | 24h color, True Day&Night |
| Sensor size | 1/2.7" |
| Imager | CMOS |
| Video compression | H.264, H.264+ (Smart Stream), H.265, H.265+ (Smart Stream), M-JPEG |
| Image resolution max. | 5120x1440 |
| Image Transmission Rate | 15 fps |
| Day&Night switch | Electronic |
| Sensitivity | 0,001 Lux at F1,6 |
| Low Speed Shutter | yes |
| Digital Noise Reduction | yes |
| Backlight Compensation | BLC, HLC |
| Wide Dynamic Range (WDR) | D-WDR |
| Motion Detector | yes |
| Focal Length | 2,8 mm |
| Iris Control | DC |
| Digital Zoom | yes |
| Horizontal Angle of View | 180° |
| MOD (Mininum Object Distance) | 2 m |
| llumination | Infrared, White light |
| Housing | Outdoor |
| Color (housing) | Pantone P 179-1, RAL9003 |
| Hosuing material | Aluminium |
| Heater | no |
| Blower | no |
| Alarm inputs | nein |
| Alarm outputs | no |
| Audio support | yes |
| Ethernet interface | 100Base-TX, 10Base-T |
| Serial interfaces | Ethernet |
| Internal storage | microSD-Kartenslot |
| Analysis functions | Cross Counting (CC), Crowd Density Detection (CD), Face Recogntion (FR), Heat Map (HM), Human&Vehicle Detection (PD&VD), License Plate Recognition (LPR), Line Crossing Detection (LCD), Perimeter Intrusion Detection (PID), Queue Lenght Detection (QD), Rare Sound Detection (RSD), Region Entrance, Region Exiting |
| Mounting | Ceiling mount, Wall mount |
| Supply voltage | 12VDC, PoE |
| Power consumption | max. 8,88W |
| Power over Ethernet | IEEE 802.3af |
| Vandal proof | no |
| Operating temperature | -30°C ~ +60°C, @ Humidity 90% or less (non-condensing) |
| ONVIF | ONVIF Profile G, ONVIF Profile S, ONVIF Profile T |
| Protection rating | IP67 |
| Camera type | Dome |
| Audio | 2 Way Audio, built-in loudspeaker, built-in microphone |
| Remote configuration | Browser |
| Illumination range | approx. 30m, Warm light approx. 15m |
| Weight | 765g |
| Manufacturer order number | 238644 |
| Brand | eneo |

Additional Productinformation

Related products

| eneo INsight Installation Tool | Supports adjustments for cameras with AI features for IN-series |
| --- | --- |
| eneo Site Manager | Management Software for eneo IP products |
| KBD-3NET | Network keyboard with joystick, supports ONVIF protocol, RJ45, 12VDC |

Recommendations

| jetpower-PSU-XS-11C Steckernetzteil | &nbsp; |
| --- | --- |
| LACKIERUNG Fix-Dome-Kamera | Extra charge for special paint fixed dome in RAL color |

Services

| Vorkonfiguration IP-Kamera | Preconfiguration 1x IP camera Function test, FW update |
| --- | --- |