Specifications text

Article number

238644

Type

eneo IND-58F0002MTDA

Short description

Network Dome Camera Fix, Turret, Panorama 180°, 2x 5120x1440, Day/Night, DWDR, AI, 2x 2,8mm, IR, White Light, 12VDC, PoE, IP67

Main Features

Multisensor 2x 1/2.7’ CMOS sensor, panorama

Maximum resolution 5120x1440 pixels each, 15fps

Panorama ultra wide-angle camera 180° HFoV, 32:9

AI analysis functions including object classification

WDR, BLC, HLC and 3D-DNR noise reduction

Lenses with 2.8mm fixed focal length

Smart Codec H.265/H.264, dual streaming

Browser-independent (Edge, Safari, Firefox, Chrome)

Supports ONVIF profiles S/T/G/M and audio

Local storage via micro SD card

NDAA compliance

Integrated IR and white light LED illumination, IP67

Other Features (Specifications)

| General |  |
| --- | --- |
| Heating | No |
| Lighting | Infrared, White |
| Dimensions (WxHxD) (extended) | 112,4x135mm |
| Illumination range | Approx. 30 m ¦ Approx. 15 m Warm light |
| ESC | 1/5 ~ 1/20.000sec. |
| Weight | 0.765 kg |
| SDK | upon request |
| Text overlay | Switchable on/off |
| Color (Housing) | RAL9003,Pantone P 179-1 / Other colors available upon request |
| Housing type | Outside |
| Housing material | Aluminium |
| Camera Type | Panoramic camera |
| Power consumption | 8.88 W |
| Mounting type | Ceiling, Wall |
| ONVIF | ONVIF Profile G, ONVIF Profile M, ONVIF Profile S, ONVIF Profile T |
| Power over Ethernet | IEEE 802.3af |
| Protection | IP67, NEMA 4X |
| Power supply | 12VDC, Poe |
| Temperature Range (Operating) | -30°C ~ +60°C,@ Luftfeuchtigkeit 90% oder weniger (nicht kondensierend) |
| Wide Dynamic Range (WDR) | D-WDR |
| Camera |  |
| Backlight compensation | Yes |
| Low Speed Shutter | Yes |
| Resolution standard | 8 MP |
| Recording sensor | CMOS |
| Chip size | 1/2,7" |
| Flickerless function | Yes |
| Camera shape | Minsters |
| Manual Electric Shutter (MES) | 1-5~1-20,000sec. |
| Day/night mode | Day,Night,Automatic,Time |
| White balance | Automatic,Manual |
| Photosensitivity | 0,001 Lux bei F1,6 |
| System | 24h Color, True Day&Night |
| Day/night switching | Electronic |
| Display |  |
| Image resolution max. | 5120x1440 |
| DORI |  |
| Watch (62px/m) | 23,8 m |
| Detect (125px/m) | 11,7 m |
| Identify (250px/m) | 5,5 m |
| Perceiving (25px/m) | 60,1 m |
| Functions |  |
| Analysis | Abandoned and Removed Objects (SOD), Crowds (CD), Enter Region, Heat Map (HM), Intrusion, Intrusion (PID), Leave Region, Line crossing (LCD), Manipulation (TD), Object Classification Human / Vehicle (PD&VD), Passage count (CC), Queues (QD), Rare Sounds (RSD) |
| Audio support | Yes |
| Motion detector | Yes |
| Digital Noise Reduction (DNR) | Yes |
| Multicast | Yes |
| Alarm processing | recording internal storage (SD card),E-Mail,HTTP,FTP,TCP,UDP,Red & bule flashing lights,Siren,Cloud Upload |
| Audio | 2 Way Audio, Built-in microphone, Built-in speaker |
| Browser | Edge, Firefox, Google Chrome, Safari |
| Menu languages | English, French, German, Italian, Japanese, Portuguese, Russian, Spanish, Turkish |
| Password | Administrator and User |
| Privacy Zone | Areas-ON/OFF switchable |
| Internal memory | microSD card slot |
| Notes |  |
| Remark | Face and license plate recognition require an SD card or an eneo IN recorder. For optimal results,use only eneo components. For mixed brands,we offer a compatibility check for a fee (info: item #229235 at VIDEOR). |
| Interfaces / Inputs/ Outputs |  |
| External connections | RJ-45,12VDC input (DC plug),connection cable approx.30cm,micro SD card slot 256 GB |
| Control Interfaces | Ethernet |
| Parts supplied |  |
| Scope of delivery | manual,bag of screws |
| Network |  |
| Network Protocols | DDNS, DHCP, DNA, FTP, HTTP, HTTPS, IEE 802.1, IPv4, IPv6, NTP, PPPoE, RTP, RTSP, SMTP, TCP-IP, UDP, UPnP |
| Lens |  |
| Horizontal angle of view | 180° |
| Focal length | 2,8 mm |
| Aperture Range (F) | F1,2 |
| MOD (Minimum Object Distance) | 2 m |
| Lens Type | Prime lens |
| Video |  |
| Image transfer rate max. | 15 fps |
| Resolution (extended) | 5120x1440,4608x1296,4096x1152,3840x1080,3072x864,1536x432,1024x288 |
| Picture size | 32:9 |
| Data stream types | Dual Streaming |
| Video Compression | H.264, H.264+, H.265, H.265+, M-JPEG |
| Product labeling: |  |
| EAN/GTIN | 4260229840726 |
| Manufacturer order number | 238644 |
| Series | eneo IN-Serie |
| Brand | eneo |

Additional Productinformation

Related products

| eneo AK-20 | Camera connection box for Wall and ceiling mounting, IN Series Bullet Cameras |
| --- | --- |
| eneo AK-21 | Camera connection box for Wall and ceiling mounting, Bullet and Dome Cameras IN series |
| eneo AK-24 | Dual camera connection box for selected Bullet or Fix Dome IN-Series cameras |
| eneo DOME-WB8 | Wall bracket for PTZ dome INP-54M2812M0A and IND-5 Dome Series |
| eneo INsight | Control software for recorder IN- and MN-series |
| eneo INsight Installation Tool | Supports adjustments for cameras with AI features for IN-series |
| eneo INsight mobile | Mobile App for recorder IN- and MN-series |
| eneo Site Manager | Management Software for eneo IP products |

Recommendations

| eneo NSMI-4004M0A | Gigabit Switch Industrial, Managed, 4x PoE, 2xSFP, 30W each Port, -40°C~75°C, 44~57 VDC, w/o power supply |
| --- | --- |
| eneo NSMI-4008M0A | Gigabit Switch Industrial, Managed, 8x PoE, 2xSFP, 30W each Port, -40°C~75°C, 44~57 VDC, w/o power supply |
| eneo NSMR-2008M0A | Gigabit Switch Rack, Managed, 8x PoE, 2xSFP, max. 30W each Port, internal Power Supply max. 150W |
| eneo NSMR-2016M0A | Gigabit Switch Rack, Managed, 16x PoE, 2xSFP, max. 30W each Port, integrated Power Suppply max. 250W |
| eneo NSMR-2024M0A | Gigabit Switch Rack, Managed, 24x PoE, 2xSFP, max. 30W each Port, integrated Power Supply max. 400W |
| eneo NSUI-3004M0A | Gigabit Switch Industrial, Unmanaged, 4x PoE, 1xRJ45, 1xSFP, 30W each Port, -40°C~75°C, 44~57 VDC, w/o power supply |
| eneo NSUI-3008M0A | Gigabit Switch Industrial, Unmanaged, 8x PoE, 2xSFP, 30W each Port, -40°C~75°C, 44~57 VDC, w/o power supply |
| eneo NSUR-1008M0A | Gigabit Switch Rack, Unmanaged, 8x PoE, 2xSFP, max. 30W each Port, integrated Power Supply max. 150W |
| eneo NSUR-1016M0A | Gigabit Switch Rack, Unmanaged, 16x PoE, 2xSFP, max. 30W each Port, integrated Power Supply max. 250W |
| eneo NSUR-1024M0A | Gigabit Switch Rack, Unmanaged, 24x PoE, 2xSFP, max. 30W each Port, integrated Power Supply max. 400W |
| jetrics jetcat6SFTP-GN-0,5/1 |  |
| jetrics jetcat6SFTP-GN-1/1 |  |
| jetrics jetcat6SFTP-GN-2/1 |  |
| jetrics jetcat6SFTP-GN-3/1 |  |
| jetrics jetcat6SFTP-GN-7,5/1 |  |
| jetrics jetpower-PSU-XS-11C Steckernetzteil |  |

Services

| PAINTING SERVICE basic costs | once per RAL color and delivery lot |
| --- | --- |
| PAINTING SERVICE fix dome camera | in RAL color |
| Vorkonfiguration IP-Kamera | Preconfiguration 1x IP camera Function test, FW update |