Specifications text

Article number

241615

Type

INB-58F0308MLA

Short description

Network Bullet Camera, 8MP Ultra Low Light, 3-8mm MVF, AI, WDR, PoE, 12V, IP67

Main Features

1/1.8" Sony CMOS sensor, maximum resolution 8 MP/30fps

Very high light sensitivity

Colour images in the dark thanks to integrated white light

AI analysis functions including object classification

WDR and 3D DNR noise reduction

Motorised varifocal lens F1.0/3.0-8.0mm

Smart Codec H.265 /H.264, multi-streaming

Browser-independent (Edge, Safari, Firefox, Chrome)

Supports ONVIF profiles S/T/G/M

Local storage via micro SD card

Supports audio, built-in speaker and microphone

Integrated red/blue light deterrent, NDAA compliance

Other Features (Specifications)

| EAN/GTIN | 4260229840689 |
| --- | --- |
| Audio support | Yes |
| Motion detector | Yes |
| Digital Noise Reduction (DNR) | Yes |
| Multicast | Yes |
| Sunshade canopy | Yes |
| Integrated wall mount | Yes |
| Backlight compensation | Yes |
| Low Speed Shutter | Yes |
| Alarm inputs | 1 |
| Resolution standard | 8 MP |
| Recording sensor | CMOS |
| Lighting | White |
| Image resolution max. | 3840x2160 |
| Image transfer rate max. | 30 fps |
| Horizontal angle of view | 105 - 56° |
| Focal length | 3 - 8 mm |
| Chip size | 1/1,8" |
| Dimensions (WxHxD) (extended) | 199.82 x 128.98 x 90.7mm |
| Alarm processing | recording internal storage (SD card),E-Mail,HTTP,FTP,TCP,UDP,Red & bule flashing lights,Siren |
| Audio | Built-in microphone, Built-in speaker |
| Resolution (extended) | 3840x2160 (16:9),2592x1944 (4:3),3072x1728 (16:9),2592x1520 (16:9),2304x1296 (16:9),1920x1080 (16:9),640x480 (VGA),320x240 (QVGA) |
| Illumination range | Approx 30m |
| Remark | NDAA compliance under Section 889,The face detection and license plate recognition functions are only available with an inserted SD card or in conjunction with a recorder from the eneo IN series.,To guarantee you an optimum user experience,we recommend th |
| Watch (62px/m) | 25.4 m |
| Picture size | 16:9, 4:3 |
| Browser | Edge, Firefox, Google Chrome, Safari |
| Data stream types | Triple-Stream(Main/Sub/Mobile) |
| Detect (125px/m) | 11.7 m |
| ESC | 1/5 ~ 1/20.000sec. |
| External connections | RJ-45,12VDC input (DC socket),connection cable approx. 30cm,micro SD card slot 256 GB |
| Flickerless function | Yes |
| Identify (250px/m) | 4.4 m |
| Camera shape | Bullet |
| Scope of delivery | manual,bag of screws |
| Manual Electric Shutter (MES) | 1-5~1-20,000sec. |
| Menu languages | English, French, German, Italian, Portuguese, Russian, Spanish, Turkish |
| Password | Administrator and User |
| Privacy Zone | Areas-ON/OFF switchable |
| SDK | upon request |
| Day/night mode | Yes |
| Text overlay | Switchable on/off |
| Perceiving (25px/m) | 65 m |
| White balance | Automatic,Manual |
| Color (Housing) | RAL9003,Pantone P 179-1 |
| Housing type | Outside |
| Housing material | Aluminium |
| Internal memory | SDHC Card, SDXC Card |
| Camera Type | Face, License plate recognition, People counting, Retail Intelligence |
| Photosensitivity | 0,0005 Lux bei F1.0 |
| MOD (Minimum Object Distance) | 3 m |
| Mounting type | Ceiling, Wall |
| Network Protocols | DDNS, DHCP, DNA, FTP, HTTP, HTTPS, NTP, RTP, RTSP, SMTP, TCP-IP, UPnP |
| Lens Type | Varifocal |
| ONVIF | ONVIF Profile G, ONVIF Profile M, ONVIF Profile S, ONVIF Profile T |
| Manufacturer order number | 241615 |
| Power over Ethernet | IEEE 802.3af |
| Protection | IP67 |
| Series | eneo IN-Serie |
| Control Interfaces | Ethernet |
| Power supply | 12VDC, Poe |
| System | 24h Color |
| Day/night switching | no |
| Temperature Range (Operating) | -35°C ~ +60°C,@ Luftfeuchtigkeit 90% oder weniger (nicht kondensierend) |
| Video Compression | H.264, H.264+, H.265, H.265+, M-JPEG |
| Wide Dynamic Range (WDR) | WDR |
| Zoomfactor (optical) | 2 |
| Brand | eneo |

Additional Productinformation

Related products

| AK-20 | Camera connection box for Wall and ceiling mounting, IN Series Bullet Cameras |
| --- | --- |
| AK-21 | Camera connection box for Wall and ceiling mounting, IN Series Bullet Cameras |
| AK-24 | Dual camera connection box for selected Bullet or Fix Dome IN-Series cameras |
| eneo INsight Installation Tool | Supports adjustments for cameras with AI features for IN-series |
| eneo INsight mobile | Mobile App for recorder IN- and MN-series |
| eneo Site Manager | Management Software for eneo IP products |
| PM-21 | Pole mounting adapter for camera connection box AK-20 and AK-21 |

Recommendations

| jetpower-PSU-XS-11C Steckernetzteil |  |
| --- | --- |
| NSMI-4004M0A | Gigabit Switch Industrial, Managed, 4x PoE, 2xSFP, 30W each Port, -40°C~75°C, 44~57 VDC |
| NSMI-4008M0A | Gigabit Switch Industrial, Managed, 8x PoE, 2xSFP, 30W each Port, -40°C~75°C, 44~57 VDC |
| NSMR-2008M0A | Gigabit Switch Rack, Managed, 8x PoE, 2xSFP, max. 30W each Port, internal Power Supply max. 150W |
| NSMR-2016M0A | Gigabit Switch Rack, Managed, 16x PoE, 2xSFP, max. 30W each Port, integrated Power Suppply max. 250W |
| NSMR-2024M0A | Gigabit Switch Rack, Managed, 24x PoE, 2xSFP, max. 30W each Port, integrated Power Supply max. 400W |
| NSUI-3004M0A | Gigabit Switch Industrial, Unmanaged, 4x PoE, 1xRJ45, 1xSFP, 30W each Port, -40°C~75°C, 44~57 VDC |
| NSUI-3008M0A | Gigabit Switch Industrial, Unmanaged, 8x PoE, 2xSFP, 30W each Port, -40°C~75°C, 44~57 VDC |
| NSUR-1008M0A | Gigabit Switch Rack, Unmanaged, 8x PoE, 2xSFP, max. 30W each Port, integrated Power Supply max. 150W |
| NSUR-1016M0A | Gigabit Switch Rack, Unmanaged, 16x PoE, 2xSFP, max. 30W each Port, integrated Power Supply max. 250W |
| NSUR-1024M0A | Gigabit Switch Rack, Unmanaged, 24x PoE, 2xSFP, max. 30W each Port, integrated Power Supply max. 400W |

Services

| LACKIERUNG Bulletkamera | Extra charge for special paint bullet camera in RAL color |
| --- | --- |
| LACKIERUNG Grundkosten | Painting basic costs once per RAL color and delivery lot |
| Vorkonfiguration IP-Kamera | Preconfiguration 1x IP camera Function test, FW update |