Specifications text

Article number

243653

Type

eneo MSM-42F0028M0B

Short description

1/2.8" HD Board Camera, Colour, 1920x1080, 2,8mm, WDR, Housing, 12VDC

Main Features

1/2,8" 2 Megapixel SONY STARVIS 2 CMOS-Sensor

Super low light performance by Sony STARVIS 2 technology

Board Camera with rugged metal cabinet

Resolution max. 1920x1080 (Full HD)

Separate outputs for digital and multi-signal

Multisignal: HD-TVI, AHD, CVI and Composite selectable

Digital signal: EX-SDI, HD-SDI selectable

Fixed 2.8mm

Electronic Day/Night function

ROI (Region of Interest) WDR mode

Integrated motion detector, defog, privacy zones

Other Features (Specifications)

| General |  |
| --- | --- |
| Dimensions (WxHxD) (extended) | 37x37,8x27mm |
| Weight | 0.11 kg |
| Text overlay | available |
| Color (Housing) | black |
| Housing type | Inside |
| Housing material | Aluminium |
| Camera Type | Board Camera |
| Power consumption | 1.5 W |
| Mounting type | Ceiling, Wall |
| Power supply | 12VDC |
| Temperature Range (Operating) | -10°C ~ +50°C,@ Luftfeuchtigkeit 20%-80% (nicht kondensierend) |
| Wide Dynamic Range (WDR) | D-WDR |
| Camera |  |
| Backlight compensation | Yes |
| Low Speed Shutter | Yes |
| Recording sensor | CMOS |
| Chip size | 1/2,8" |
| Flickerless function | Yes |
| Camera shape | Module |
| Signal-to-noise ratio | 50 dB |
| Day/night mode | Automatic,Day,Night,extern (Sensor) |
| White balance | Automatic,Preset,Manual,AUTOext |
| Photosensitivity | 0,2Lux@F2,3 |
| System | Color |
| Day/night switching | Switchable IR Filter (ICR) |
| Display |  |
| Image resolution max. | 1920x1080 |
| DORI |  |
| Watch (62px/m) | 18 m |
| Detect (125px/m) | 8,8 m |
| Identify (250px/m) | 4,2 m |
| Perceiving (25px/m) | 45,9 m |
| Functions |  |
| Motion detector | Yes |
| Digital Noise Reduction (DNR) | Yes |
| Menu languages | Chinese, English, French, German, Japanese, Korean |
| Notes |  |
| Remark | NDAA compliance under Section 889 |
| Interfaces / Inputs/ Outputs |  |
| External connections | voltage input,1x SDI Signal (BNC),1x HD/SD Analog Signal (BNC) |
| Interface Protocols | UTC (HD Signal Format) |
| Control Interfaces | Coaxitron (HD Protocol) |
| Video Outputs | AHD, Composite (FBAS) |
| Parts supplied |  |
| Scope of delivery | Quick Installation Guide,2x mounting screws,OSD menu control board,video sub-output cable |
| Lens |  |
| Horizontal angle of view | 97° |
| Vertical angle of view | 48° |
| Focal length | 2,8 mm |
| Lens Type | Prime lens |
| Video |  |
| Resolution (extended) | 1920x1080,1280x720,720x576,720x480 |
| Signal Format | AHD @ 2MP, Composite (FBAS), EX-SDI @ 2MP, HD-SDI @ 2MP |
| Product labeling: |  |
| EAN/GTIN | 4260229841358 |
| Manufacturer order number | 243653 |
| Series | eneo COAXIZE |
| Brand | eneo |

Additional Productinformation

Related products

Recommendations

| eneo MAM-5DC1004M0A | Distributor, 1x BNC Input HD-TVI, CVI, AHD, CVBS, 4x BNC Output |
| --- | --- |
| eneo MAM-5MM1001M0A | Converter, HD-CVI, HD-TVI, AHD, Composite to HDMI, VGA, Composite, Indoor |
| eneo TVA-1201TRA | UTP Transmitter+Receiver, UTP, BNC, 250m, Indoor, HD TVI CVI, AHD, CVBS, Active, 12V |
| eneo TVA-1201TRP | UTP Transmitter+Receiver, UTP, BNC, 150m, Indoor, HD TVI CVI, AHD, CVBS, Passive |
| Jetrics jetcon-BS-BS-1 | Videokabel mit 2 x BNC-Stecker 1m, 75 Ohm |
| Jetrics jetcon-BS-BS-2 | Videokabel mit 2 x BNC-Stecker 2m, 75 Ohm |
| Jetrics jetcon-BS-BS-3 | Videokabel mit 2 x BNC-Stecker 3m, 75 Ohm |
| Jetrics jetcon-BS-BS-5 | Videokabel mit 2 x BNC-Stecker 5m, 75 Ohm |
| Jetrics jetpower-PSU-XS-11C Steckernetzteil |  |

Services

| Vorkonfiguration analoge Kamera | Preconfiguration 1x analog camera Function test, FW update |
| --- | --- |