Specifications text

Article number

243654

Type

MSM-42F0037MPB

Short description

1/2.8" HD Board Camera, Colour, 1920x1080, 3.7mm, Pinhole, WDR, Housing, 12VDC

Main Features

1/2,8" 2 Megapixel SONY STARVIS 2 CMOS-Sensor

Super low light performance by Sony STARVIS 2 technology

Board Camera with rugged metal cabinet

Resolution max. 1920x1080 (Full HD)

Separate outputs for digital and multi-signal

Multisignal: HD-TVI, AHD, CVI and Composite selectable

Digital signal: EX-SDI, HD-SDI selectable

Pinhole lens 3.7mm

Electronic Day/Night function

ROI (Region of Interest) WDR mode

Integrated motion detector, defog, privacy zones

Other Features (Specifications)

| Dimensions (HxWxD) | 37x37,8x21,5mm |
| --- | --- |
| Series | eneo COAXIZE |
| Camera Type | Board Camera |
| Resolution Pixels | 1280x720, 1920x1080, 704x576, 720x480 |
| System | True Day&Night |
| Sensor size | 1/2.8" |
| Imager | CMOS |
| Day&Night switch | ICR |
| Sensitivity | 0,2 Lux bei F2,3 |
| Low Speed Shutter | yes |
| Digital Noise Reduction | 3D-NR, yes |
| Backlight Compensation | BLC, HLC |
| Wide Dynamic Range (WDR) | Clear HDR, D-WDR, DOL, Inter Frame |
| Motion Detector | yes |
| Focal Length | 3,7 mm |
| Iris Control | no |
| Digital Zoom | yes |
| Horizontal Angle of View | 72° |
| Vertical Angle of View | 37° |
| MOD (Mininum Object Distance) | 0,5 m |
| Housing | Indoor |
| Color (housing) | black |
| Hosuing material | Aluminium |
| Alarm inputs | nein |
| Alarm outputs | no |
| Serial interfaces | Coaxitron (HD Protocol) |
| Analysis functions | Defog |
| Mounting | Ceiling mount, Wall mount |
| Supply voltage | 12VDC |
| Power consumption | 1.5W |
| Operating temperature | -10°C ~ +50°C, @ Humidity 20%-80% (non-condensing) |
| Signal format | AHD @ 2MP, CHD @ 2MP, Composite (FBAS), EX-SDI (270 Mbps), HD-SDI (1.485 Gbps), HD-TVI @ 2MP |
| Camera type | Modul |
| Frame rate | 25/30fps @ 1920x1080p, 50/60fps @ 1280x720p |
| Weight | 110g |
| Manufacturer order number | 243654 |
| Brand | eneo |

Additional Productinformation

Sony STARVIS 2 Improved low-light performance (especially with HDR)STARVIS 2 offers significantly improved performance in low light, especially with high dynamic range (HDR). It has been specially optimised for backlit situations and night scenes with high contrast. Higher dynamic range (High Dynamic Range)STARVIS 2 achieves up to 88 dB real dynamic range (Single Exposure) according to Sony - that's more than the original STARVIS. This helps to better display details in dark AND bright areas at the same time - without ‘burning out’ or ‘drowning out’ the image areas. Uniform brightness across the image fieldSTARVIS 2 reduces the so-called vignetting effect (edge darkening) that can occur with BSI sensors. This means a more even image brightness, even in poorly lit conditions. Better colour reproduction in low lightSTARVIS 2 offers a more natural colour reproduction at night - colours appear less washed out or monochromatic. Particularly relevant for forensic applications (e.g. clothing, vehicles, skin tones).

Related products

| MAM-5DC1004M0A | Distributor, 1x BNC Input HD-TVI, CVI, AHD, CVBS, 4x BNC Output |
| --- | --- |
| MAM-5MM1001M0A | Converter, HD-CVI, HD-TVI, AHD, Composite to HDMI, VGA, Composite, Indoor |
| MAM-5MM2203M0A | Converter, 4K HD-TVI, HD-CVI, AHD, auf HDMI, Indoor 12VDC |
| TVA-1201TRA | UTP Transmitter+Receiver, UTP, BNC, 250m, Indoor, HD TVI CVI, AHD, CVBS, Active, 12V |
| TVA-1201TRP | UTP Transmitter+Receiver, UTP, BNC, 150m, Indoor, HD TVI CVI, AHD, CVBS, Passive |

Recommendations

Services

| Vorkonfiguration analoge Kamera | Preconfiguration 1x analog camera Function test, FW update |
| --- | --- |