Specifications text

Article number

243652

Type

MPD-62M2713P0B

Short description

1/2.8" HD Dome Fixed, WDR, Day&Night, 2.7-13,5mm, Infrare 1920x1080, 12VDC, Indoor

Main Features

1/2.8" 2 Megapixel SONY STARVIS 2 CMOS sensor

Resolution max. 1920x1080 pixels (Full HD)

Super low light performance by Sony STARVIS 2 technology

Multisignal: HD-TVI, AHD, CVI and Composite selectable

Digital signal: EX-SDI, HD-SDI selectable

Motorized vari-focal lens 2.7~13.5mm

Removable infrared cut filter

ROI (Region of Interest) WDR mode

Integrated motion detector, defog, privacy zones

Wedge adapter included (10° inclination)

Scratch resistant hard coated bubble

Easy installation, 3-Axis gimbal

Other Features (Specifications)

| Dimensions (HxWxD) | 94x116mm |
| --- | --- |
| Series | eneo Callisto Indoor, eneo KX-Series |
| Resolution Pixels | 1280x720, 1920x1080, 720x576 |
| System | True Day&Night |
| Sensor size | 1/2.8" |
| Imager | CMOS |
| Day&Night switch | ICR |
| Sensitivity | 0.005Lux(Day mode), 0.0005Lux(Night mode) |
| Low Speed Shutter | yes |
| Digital Noise Reduction | 3D-NR, yes |
| Backlight Compensation | BLC, HLC |
| Wide Dynamic Range (WDR) | Clear HDR, D-WDR, DOL, Inter Frame |
| Motion Detector | yes |
| Focal Length | 2,7 - 13,5 mm |
| Iris Control | DC |
| Zoom Factor | 4x |
| Digital Zoom | yes |
| Horizontal Angle of View | 101 - 32° |
| Vertical Angle of View | 54 - 18° |
| MOD (Mininum Object Distance) | 0,5 m |
| llumination | Infrared |
| Housing | Indoor |
| Color (housing) | white |
| Hosuing material | Plastic |
| Serial interfaces | Coaxitron (HD Protocol) |
| Analysis functions | Defog |
| Mounting | Ceiling mount, Wall mount |
| Supply voltage | 12VDC |
| Power consumption | 6W |
| Vandal proof | no |
| Operating temperature | -10°C ~ +50°C |
| Signal format | AHD @ 2MP, CHD @ 2MP, Composite (FBAS), EX-SDI 1.0@270Mbps, EX-SDI 2.0@135Mbps, HD-SDI@1,485Gbps, non compressed, HD-TVI @ 2MP |
| Camera type | Dome |
| Frame rate | 25/30fps @ 1920x1080p, 50/60fps @ 1280x720p |
| Aperture range (F) | F1.6 |
| Illumination range | approx. 30m |
| Weight | 260g |
| Manufacturer order number | 243652 |
| Brand | eneo |

Additional Productinformation

Sony STARVIS 2 Improved low-light performance (especially with HDR)STARVIS 2 offers significantly improved performance in low light, especially with high dynamic range (HDR). It has been specially optimised for backlit situations and night scenes with high contrast. Higher dynamic range (High Dynamic Range)STARVIS 2 achieves up to 88 dB real dynamic range (Single Exposure) according to Sony - that's more than the original STARVIS. This helps to better display details in dark AND bright areas at the same time - without ‘burning out’ or ‘drowning out’ the image areas. Uniform brightness across the image fieldSTARVIS 2 reduces the so-called vignetting effect (edge darkening) that can occur with BSI sensors. This means a more even image brightness, even in poorly lit conditions. Better colour reproduction in low lightSTARVIS 2 offers a more natural colour reproduction at night - colours appear less washed out or monochromatic. Particularly relevant for forensic applications (e.g. clothing, vehicles, skin tones).

Related products

| MAM-5DC1004M0A | Distributor, 1x BNC Input HD-TVI, CVI, AHD, CVBS, 4x BNC Output |
| --- | --- |
| MAM-5MM1001M0A | Converter, HD-CVI, HD-TVI, AHD, Composite to HDMI, VGA, Composite, Indoor |
| VWA-01 | Aluminiumschiene zur Montage von Video Walls, Länge 2m |
| VWM-02 | Wandhalterung, ausziehbar, für Monitore 42"-65", bis 80kg, für Video Walls geeignet |

Recommendations

| LACKIERUNG Fix-Dome-Kamera | Extra charge for special paint fixed dome in RAL color |
| --- | --- |
| SC-04MHD | Quad Splitter, 1920x1080, Composite, AHD, TVI, CVI Input Composite, HDMI, VGA Output |
| SC-IPM07PRO | 7" (17,8cm) Test Monitor, Touch, IP, HD/3G/EX-SDI, TVI, CVI, AHD, Composite, HDMI |
| SC-MFM07HD | Test Monitor, Multiformat, for Service, HD-TVI/CVI/AHD, HD-SDI, EX-SDI, FBAS, HDMI |

Services

| LACKIERUNG Fix-Dome-Kamera | Extra charge for special paint fixed dome in RAL color |
| --- | --- |
| LACKIERUNG Grundkosten | Painting basic costs once per RAL color and delivery lot |
| Vorkonfiguration analoge Kamera | Preconfiguration 1x analog camera Function test, FW update |