Specifications text

Article number

243652

Type

MPD-62M2713P0B

Short description

1/2.8" HD Dome Fixed, WDR, Day&Night, 2.7-13,5mm, Infrare 1920x1080, 12VDC, Indoor

Main Features

1/2.8" 2 Megapixel SONY STARVIS CMOS sensor

Resolution max. 1920x1080 pixels (Full HD)

Separate outputs for digital and multi-signal

Multisignal: HD-TVI, AHD, CVI and Composite selectable

Digital signal: EX-SDI, HD-SDI selectable

Motorized vari-focal lens 2.7~13.5mm

Removable infrared cut filter

ROI (Region of Interest) WDR mode

Integrated motion detector, defog, privacy zones

Wedge adapter included (10° inclination)

Scratch resistant hard coated bubble

Easy installation, 3-Axis gimbal

Other Features (Specifications)

| Dimensions (HxWxD) | 94x116mm |
| --- | --- |
| Series | eneo Callisto Indoor, eneo KX-Series |
| Resolution Pixels | 1280x720, 1920x1080, 720x576 |
| System | True Day&Night |
| Sensor size | 1/2.8" |
| Imager | CMOS |
| Day&Night switch | ICR |
| Low Speed Shutter | yes |
| Digital Noise Reduction | yes |
| Backlight Compensation | BLC, HLC |
| Wide Dynamic Range (WDR) | Clear HDR, D-WDR, DOL, Inter Frame |
| Motion Detector | yes |
| Focal Length | 2,7 - 13,5 mm |
| Iris Control | DC |
| Zoom Factor | 4x |
| Digital Zoom | yes |
| Horizontal Angle of View | 101 - 32° |
| Vertical Angle of View | 54 - 18° |
| MOD (Mininum Object Distance) | 0,5 m |
| llumination | Infrared |
| Housing | Indoor |
| Color (housing) | white |
| Hosuing material | Plastic |
| Serial interfaces | Coaxitron (HD Protocol) |
| Analysis functions | Defog |
| Mounting | Ceiling mount, Wall mount |
| Supply voltage | 12VDC |
| Power consumption | 6W |
| Vandal proof | no |
| Operating temperature | -10°C ~ +50°C |
| Signal format | AHD @ 2MP, CHD @ 2MP, Composite (FBAS), EX-SDI 1.0@270Mbps, EX-SDI 2.0@135Mbps, HD-SDI@1,485Gbps, non compressed, HD-TVI @ 2MP |
| Camera type | Dome |
| Frame rate | 25/30fps @ 1920x1080p, 50/60fps @ 1280x720p |
| Aperture range (F) | F1.6 |
| Illumination range | approx. 30m |
| Weight | 260g |
| Manufacturer order number | 243652 |
| Brand | eneo |

Additional Productinformation

Related products

| MAM-5DC1004M0A | Distributor, 1x BNC Input HD-TVI, CVI, AHD, CVBS, 4x BNC Output |
| --- | --- |
| MAM-5MM1001M0A | Converter, HD-CVI, HD-TVI, AHD, Composite to HDMI, VGA, Composite, Indoor |
| VWA-01 | Aluminiumschiene zur Montage von Video Walls, Länge 2m |
| VWM-02 | Wandhalterung, ausziehbar, für Monitore 42"-65", bis 80kg, für Video Walls geeignet |

Recommendations

| LACKIERUNG Fix-Dome-Kamera | Extra charge for special paint fixed dome in RAL color |
| --- | --- |
| SC-04MHD | Quad Splitter, 1920x1080, Composite, AHD, TVI, CVI Input Composite, HDMI, VGA Output |
| SC-IPM07PRO | 7" (17,8cm) Test Monitor, Touch, IP, HD/3G/EX-SDI, TVI, CVI, AHD, Composite, HDMI |
| SC-MFM07HD | Test Monitor, Multiformat, for Service, HD-TVI/CVI/AHD, HD-SDI, EX-SDI, FBAS, HDMI |

Services

| LACKIERUNG Fix-Dome-Kamera | Extra charge for special paint fixed dome in RAL color |
| --- | --- |
| LACKIERUNG Grundkosten | Painting basic costs once per RAL color and delivery lot |
| Vorkonfiguration analoge Kamera | Preconfiguration 1x analog camera Function test, FW update |