Specifications text

Article number

243655

Type

MPC-52C0000M0B

Short description

1/2.8" HD Camera, Day&Night, 1920x1080, WDR, C/CS, 12/24V

Main Features

1/2.8" 2 Megapixel SONY STARVIS 2 CMOS sensor

Super low light performance by Sony STARVIS 2 technology

Resolution max. 1920x1080 pixels (Full HD)

Separate outputs for digital and multi-signal

Multisignal: HD-TVI, AHD, CVI and Composite selectable

Digital signal: EX-SDI, HD-SDI selectable

Removable infrared cut filter

ROI (Region of Interest) WDR mode

Integrated motion detector, defog, privacy zones

C/CS mount, LSC (Lens shading compensation)

Other Features (Specifications)

| Audio support | No |
| --- | --- |
| Recording sensor | CMOS |
| Motion detector | No |
| Chip size | 1/2,8" |
| Digital Noise Reduction (DNR) | Yes |
| Dimensions (WxHxD) (extended) | 72x74x89mm |
| Remark | NDAA compliance under Section 889 |
| External connections | voltage input,1x SDI Signal (BNC),1x HD/SD Analog Signal (BNC) |
| Flickerless function | Yes |
| Camera shape | Box |
| Scope of delivery | manual,C/CS adapter |
| Menu languages | Chinese, English, French, German, Japanese, Korean |
| Multicast | No |
| Interface Protocols | UTC (HD Signal Format), UTC (HD Signal Format) |
| Signal-to-noise ratio | 50 dB |
| Signal Format | AHD @ 2MP, AHD @ 2MP, Composite (FBAS), EX-SDI @ 2MP, HD-SDI @ 2MP, HD-TVI @ 2MP, HD-TVI @ 2MP, HD-TVI @ 2MP, HD-TVI @ 2MP |
| Sunshade canopy | No |
| Day/night mode | Automatic,Day,Night,extern (Sensor) |
| Text overlay | available |
| Integrated wall mount | No |
| White balance | Automatic,Preset,Manual,AUTOext |
| Backlight compensation | No |
| Housing type | Inside |
| Housing material | Aluminium |
| Heating | Yes |
| Photosensitivity | 0.005Lux @F1,6 (Day mode),0.0005Lux(Night mode) |
| Low Speed Shutter | No |
| Fan | No |
| Manufacturer order number | 243655 |
| Series | eneo KX-Serie,eneo COAXIZE |
| Control Interfaces | Coaxitron (HD Protocol) |
| Power supply | 12VDC |
| System | True Day&Night |
| Day/night switching | Switchable IR Filter (ICR) |
| Temperature Range (Operating) | -10°C ~ +50°C,@ Luftfeuchtigkeit 20%-80% (nicht kondensierend) |
| Vandalism-resistant | No |
| Wide Dynamic Range (WDR) | D-WDR, D-WDR, D-WDR, D-WDR, D-WDR, D-WDR, D-WDR |
| Brand | eneo |

Additional Productinformation

Related products

| H04Z03NFDC-MP | F1,5/4-13mm, Megapixel DC-Lens, variable Focal Lenght, 1/1.8" CS-Mount |
| --- | --- |

Recommendations

| LACKIERUNG Bulletkamera | Extra charge for special paint bullet camera in RAL color |
| --- | --- |

Services

| LACKIERUNG Bulletkamera | Extra charge for special paint bullet camera in RAL color |
| --- | --- |
| LACKIERUNG Grundkosten | Painting basic costs once per RAL color and delivery lot |
| Vorkonfiguration analoge Kamera | Preconfiguration 1x analog camera Function test, FW update |