Specifications text

Article number

243655

Type

MPC-52C0000M0B

Short description

1/2.8" HD Camera, Day&Night, 1920x1080, WDR, C/CS, 12/24V

Main Features

1/2.8" 2 Megapixel SONY STARVIS CMOS sensor

Resolution max. 1920x1080 pixels (Full HD)

Separate outputs for digital and multi-signal

Multisignal: HD-TVI, AHD, CVI and Composite selectable

Digital signal: EX-SDI, HD-SDI selectable

Removable infrared cut filter

ROI (Region of Interest) WDR mode

Integrated motion detector, defog, privacy zones

C/CS mount, LSC (Lens shading compensation)

Other Features (Specifications)

| Dimensions (HxWxD) | 72x74x89mm |
| --- | --- |
| Series | eneo COAXIZE, eneo KX-Series |
| Resolution Pixels | 1280x720, 1920x1080, 720x480, 720x576 |
| System | True Day&Night |
| Sensor size | 1/2.8" |
| Imager | CMOS |
| Day&Night switch | ICR |
| Sensitivity | 0,005 Lux at F1,6 |
| Low Speed Shutter | yes |
| Digital Noise Reduction | yes |
| Wide Dynamic Range (WDR) | Clear HDR, D-WDR, DOL, Inter Frame |
| Motion Detector | yes |
| Iris Control | DC |
| Housing | Indoor |
| Color (housing) | Pantone Cool Gray 1C |
| Hosuing material | Aluminium |
| Alarm inputs | nein |
| Alarm outputs | no |
| Serial interfaces | Coaxitron (HD Protocol) |
| Analysis functions | Defog |
| Supply voltage | 12VDC |
| Power consumption | 3.5W |
| Vandal proof | no |
| Operating temperature | -10°C ~ +50°C, @ Humidity 20%-80% (non-condensing) |
| Signal format | AHD @ 2MP, CHD @ 2MP, Composite (FBAS), EX-SDI (270 Mbps), HD-SDI (1.485 Gbps), HD-TVI @ 2MP |
| Camera type | Box |
| Frame rate | 25/30fps @ 1920x1080p, 50/60fps @ 1280x720p |
| Weight | 290g |
| Manufacturer order number | 243655 |
| Brand | eneo |

Additional Productinformation

Related products

| H04Z03NFDC-MP | F1,5/4-13mm, Megapixel DC-Lens, variable Focal Lenght, 1/1.8" CS-Mount |
| --- | --- |

Recommendations

| LACKIERUNG Bulletkamera | Extra charge for special paint bullet camera in RAL color |
| --- | --- |

Services

| LACKIERUNG Bulletkamera | Extra charge for special paint bullet camera in RAL color |
| --- | --- |
| LACKIERUNG Grundkosten | Painting basic costs once per RAL color and delivery lot |
| Vorkonfiguration analoge Kamera | Preconfiguration 1x analog camera Function test, FW update |