Specifications text

Article number

243649

Type

MPB-72M0550M0B

Short description

1/2,8" HD Camera, Day&Night, 1920x1080, Infrared, WDR, 5-50mm, 12/24V, IP67

Main Features

1/2,8" 2 Megapixel SONY STARVIS 2 CMOS-Sensor

Resolution max. 1920x1080 (Full HD)

Super low light performance by Sony STARVIS 2 technology

Multisignal: HD-TVI, AHD, CVI and Composite selectable

Digital signal: EX-SDI, HD-SDI selectable

Motorized lens 5 ~ 50mm

Removable infrared cut filter

ROI (Region of Interest) WDR mode

Integrated motion detector, defog, privacy zones

IP67, built heater

Easy installation, central locking screw

IR illumination switchable via OSD menu

Other Features (Specifications)

| Digital Noise Reduction (DNR) | Yes |
| --- | --- |
| Sunshade canopy | Yes |
| Integrated wall mount | Yes |
| Recording sensor | CMOS |
| Lighting | Infrared |
| Horizontal angle of view | 47 - 10° |
| Focal length | 5 - 50 mm |
| Chip size | 1/2,8" |
| Dimensions (WxHxD) (extended) | 100x108x267mm |
| Illumination range | ca. 60m |
| Remark | NDAA compliance under Section 889 |
| Watch (62px/m) | 39 m |
| Aperture Range (F) | F1.6 |
| External connections | voltage input,1x SDI Signal (BNC),1x HD/SD Analog Signal (BNC),connection cable approx.100cm |
| Flickerless function | Yes |
| Identify (250px/m) | 9 m |
| Camera shape | Bullet |
| Scope of delivery | manual,drilling template,bag of screws |
| Menu languages | Chinese, English, French, German, Japanese, Korean |
| Interface Protocols | UTC (HD Signal Format) |
| Signal-to-noise ratio | 50 dB |
| Signal Format | AHD @ 2MP, Composite (FBAS), EX-SDI @ 2MP, HD-SDI @ 2MP, HD-TVI @ 2MP |
| Day/night mode | Automatic,Day,Night,extern (Sensor) |
| Text overlay | available |
| Perceiving (25px/m) | 100 m |
| White balance | Automatic,Preset,Manual,AUTOext |
| Housing type | Outside |
| Housing material | Aluminium |
| Photosensitivity | 0.005Lux(Day mode),0.0005Lux(Night mode) |
| Mounting type | Ceiling, Wall |
| Lens Type | Varifocal (motorized) |
| Manufacturer order number | 243649 |
| Protection | IP67 |
| Series | eneo KX-Serie,eneo COAXIZE,eneo Candid Medium |
| Control Interfaces | Coaxitron (HD Protocol) |
| Power supply | 12VDC |
| System | True Day&Night |
| Day/night switching | Switchable IR Filter (ICR) |
| Temperature Range (Operating) | -30°C ~ +50°C,@ Luftfeuchtigkeit 20%-80% (nicht kondensierend) |
| Wide Dynamic Range (WDR) | D-WDR |
| Zoomfactor (optical) | 10 |
| Brand | eneo |

Additional Productinformation

Related products

| AK-4 | Camera junction box, for Candid Small ,Candid Medium, Series |
| --- | --- |
| MAM-5DC1004M0A | Distributor, 1x BNC Input HD-TVI, CVI, AHD, CVBS, 4x BNC Output |
| MAM-5MM1001M0A | Converter, HD-CVI, HD-TVI, AHD, Composite to HDMI, VGA, Composite, Indoor |
| MAM-5MM2203M0A | Converter, 4K HD-TVI, HD-CVI, AHD, auf HDMI, Indoor |
| PM-42A | Pole mount adaptor for eneo Candid, Candid S, Candid M, WB1, AK-5, AK-4 |
| TVA-1201TRA | UTP Transmitter+Receiver, UTP, BNC, 250m, Indoor, HD TVI CVI, AHD, CVBS, Active, 12V |
| TVA-1201TRP | UTP Transmitter+Receiver, UTP, BNC, 150m, Indoor, HD TVI CVI, AHD, CVBS, Passive |

Recommendations

| LACKIERUNG Bulletkamera | Extra charge for special paint bullet camera in RAL color |
| --- | --- |
| SC-04MHD | Quad Splitter, 1920x1080, Composite, AHD, TVI, CVI Input Composite, HDMI, VGA Output |
| SC-MFM07HD | Test Monitor, Multiformat, for Service, HD-TVI/CVI/AHD, HD-SDI, EX-SDI, FBAS, HDMI |

Services

| LACKIERUNG Bulletkamera | Extra charge for special paint bullet camera in RAL color |
| --- | --- |
| LACKIERUNG Grundkosten | Painting basic costs once per RAL color and delivery lot |
| Vorkonfiguration analoge Kamera | Preconfiguration 1x analog camera Function test, FW update |