Specifications text

Article number

243649

Type

eneo MPB-72M0550M0B

Short description

HD Analogue Camera Bullet, Day&Night, 1920x1080, Infrared, WDR, 5-50mm, 12/24V, IP67

Main Features

1/2,8" 2 Megapixel SONY STARVIS 2 CMOS-Sensor

Resolution max. 1920x1080 (Full HD)

Super low light performance by Sony STARVIS 2 technology

Multisignal: HD-TVI, AHD, CVI and Composite selectable

Digital signal: EX-SDI, HD-SDI selectable

Motorized lens 5 ~ 50mm

Removable infrared cut filter

ROI (Region of Interest) WDR mode

Integrated motion detector, defog, privacy zones

IP67

Easy installation, central locking screw

IR illumination switchable via OSD menu

Other Features (Specifications)

| General |  |
| --- | --- |
| Heating | Yes |
| Sunshade canopy | Yes |
| Integrated wall mount | Yes |
| Lighting | Infrared |
| Dimensions (WxHxD) (extended) | 100x108x267mm |
| Illumination range | ca. 60m |
| Weight | 1.35 kg |
| Text overlay | available |
| Color (Housing) | Pantone Cool Gray 1C |
| Housing type | Outside |
| Housing material | Aluminium |
| Power consumption | 9 W |
| Mounting type | Ceiling, Wall |
| Protection | IP67 |
| Power supply | 12VDC |
| Temperature Range (Operating) | -30°C ~ +50°C,@ Luftfeuchtigkeit 20%-80% (nicht kondensierend) |
| Wide Dynamic Range (WDR) | D-WDR |
| Camera |  |
| Backlight compensation | Yes |
| Low Speed Shutter | Yes |
| Recording sensor | CMOS |
| Chip size | 1/2,8" |
| Flickerless function | Yes |
| Camera shape | Bullet |
| Signal-to-noise ratio | 50 dB |
| Day/night mode | Automatic,Day,Night,extern (Sensor) |
| White balance | Automatic,Preset,Manual,AUTOext |
| Photosensitivity | 0.005Lux(Day mode),0.0005Lux(Night mode) |
| System | True Day&Night |
| Day/night switching | Switchable IR Filter (ICR) |
| Display |  |
| Image resolution max. | 1920x1080 |
| DORI |  |
| Watch (62px/m) | 39 - 253 m |
| Detect (125px/m) | 19 - 125,5 m |
| Identify (250px/m) | 9 - 62,8 m |
| Perceiving (25px/m) | 94,6 - 629 m |
| Functions |  |
| Motion detector | Yes |
| Digital Noise Reduction (DNR) | Yes |
| Menu languages | Chinese, English, French, German, Japanese, Korean |
| Notes |  |
| Remark | NDAA compliance under Section 889 |
| Interfaces / Inputs/ Outputs |  |
| External connections | voltage input,1x SDI Signal (BNC),1x HD/SD Analog Signal (BNC),connection cable approx.100cm |
| Interface Protocols | UTC (HD Signal Format) |
| Control Interfaces | Coaxitron (HD Protocol) |
| Video Outputs | AHD, Composite (FBAS) |
| Parts supplied |  |
| Scope of delivery | Quick installation guide,drilling template,Easy Bracket,4x wall plugs with screws,video sub-output cable,4x M4x15mm screws,1x Allen wrench |
| Lens |  |
| Horizontal angle of view | 47 - 10° |
| Vertical angle of view | 25 - 5° |
| Focal length | 5 - 50 mm |
| Aperture Range (F) | F1.6 |
| Lens Type | Varifocal (motorized) |
| Zoomfactor (optical) | 10 |
| Video |  |
| Resolution (extended) | 1920x1080,1280x720,720x576,720x480 |
| Signal Format | AHD @ 2MP, Composite (FBAS), EX-SDI @ 2MP, HD-SDI @ 2MP, HD-TVI @ 2MP |
| Product labeling: |  |
| Manufacturer order number | 243649 |
| Series | eneo KX-Serie,eneo COAXIZE,eneo Candid Medium |
| Brand | eneo |

Additional Productinformation

Related products

| eneo AK-4 | Camera junction box, for Candid Small ,Candid Medium, Series |
| --- | --- |
| eneo PM-42A | Pole mount adaptor for eneo Candid, Candid S, Candid M, WB1, AK-5, AK-4 |

Recommendations

| eneo MAM-5DC1004M0A | Distributor, 1x BNC Input HD-TVI, CVI, AHD, CVBS, 4x BNC Output |
| --- | --- |
| eneo MAM-5MM1001M0A | Converter, HD-CVI, HD-TVI, AHD, Composite to HDMI, VGA, Composite, Indoor |
| eneo TVA-1201TRA | UTP Transmitter+Receiver, UTP, BNC, 250m, Indoor, HD TVI CVI, AHD, CVBS, Active, 12V |
| eneo TVA-1201TRP | UTP Transmitter+Receiver, UTP, BNC, 150m, Indoor, HD TVI CVI, AHD, CVBS, Passive |
| Jetrics jetcon-BS-BS-1 | Videokabel mit 2 x BNC-Stecker 1m, 75 Ohm |
| Jetrics jetcon-BS-BS-2 | Videokabel mit 2 x BNC-Stecker 2m, 75 Ohm |
| Jetrics jetcon-BS-BS-3 | Videokabel mit 2 x BNC-Stecker 3m, 75 Ohm |
| Jetrics jetcon-BS-BS-5 | Videokabel mit 2 x BNC-Stecker 5m, 75 Ohm |
| Jetrics jetpower-PSU-XS-11C Steckernetzteil |  |
| SC-04MHD | Quad Splitter, 1920x1080, Composite, AHD, TVI, CVI Input Composite, HDMI, VGA Output |
| SC-MFM07HD | Test Monitor, Multiformat, for Service, HD-TVI/CVI/AHD, HD-SDI, EX-SDI, FBAS, HDMI |

Services

| PAINTING SERVICE basic costs | once per RAL color and delivery lot |
| --- | --- |
| PAINTING SERVICE bullet camera | in RAL color |
| Vorkonfiguration analoge Kamera | Preconfiguration 1x analog camera Function test, FW update |