Specifications text

Article number

243176

Type

MPP-62A0020M0C

Short description

1/2,8" HD Analogue Dome, PTZ, Day&Night, 20x, 4,7-94mm, 1920x1080, WDR, IP66

Main Features

1/2,8" Sony STARVIS CMOS-Sensor

20x Auto Focus Zoom, 4.7 ~ 94mm / 16x Digital

Resolution max. 1920x1080 (Full HD)

Separate outputs for HD signals and composite

Signal formats: HD-TVI, AHD selectable, composite

Real time WDR (Wide Dynamic Range) 25 / 30fps

Compact design for in- and outdoor applications

3D DNR noise reduction, auto-calibration

Protected areas, Defog function

Alarm inputs and outputs, 12VDC/24VAC, IK10, IP66

Other Features (Specifications)

| Dimensions (HxWxD) | 150x154mm |
| --- | --- |
| Series | eneo COAXIZE |
| Resolution Pixels | 1280x720, 1920x1080, 720x576 |
| System | True Day&Night |
| Sensor size | 1/2.8" |
| Imager | CMOS |
| Day&Night switch | ICR |
| Sensitivity | 0,35 Lux at F1,6, 50IRE |
| Low Speed Shutter | yes |
| Digital Noise Reduction | yes |
| Backlight Compensation | BLC, HLC |
| Wide Dynamic Range (WDR) | WDR 120dB |
| Motion Detector | yes |
| Focal Length | 4,7 - 94 mm |
| Zoom Factor | 20x |
| Digital Zoom | 16x |
| Horizontal Angle of View | 55,5 - 3° |
| Vertical Angle of View | 32,4 - 1,7° |
| MOD (Mininum Object Distance) | 1 m |
| llumination | no |
| Housing | Outdoor |
| Color (housing) | white |
| Color (bubble) | clear |
| Hosuing material | Aluminium |
| Window heater | no |
| Blower | no |
| Rotation range | 360° |
| Speed (manual) | 380°/Sek. |
| Speed (preset) | 380°/Sek. |
| Alarm inputs | 4 |
| Alarm outputs | 2 |
| Serial interfaces | Coaxitron (HD Protocol), RS-485 |
| Analysis functions | Defog, DIS (digital image stabilisation) |
| Mounting | Ceiling mount, Wall mount |
| Supply voltage | 12VDC, 24VAC |
| Power consumption | 12W |
| Vandal proof | yes |
| Operating temperature | -10°C ~ +50°C, @ Humidity 10%-90% (non-condensing) |
| Certifications | IK10 |
| Protection rating | IP66 |
| Signal format | AHD @ 2MP, Composite (FBAS), HD-TVI @ 2MP |
| Camera type | PTZ |
| Frame rate | 25/30fps @ 1920x1080p, 50/60fps @ 1280x720p |
| Aperture range (F) | F1.6~F3.0 |
| Weight | 1.6 kg |
| Manufacturer order number | 243176 |
| Brand | eneo |

Additional Productinformation

Related products

| DOME-WB2-W | Wall bracket for IED-63xx Metal series and IEP-63Mxx, MEP-62Mxx |
| --- | --- |
| MAM-5DC1004M0A | Distributor, 1x BNC Input HD-TVI, CVI, AHD, CVBS, 4x BNC Output |
| MAM-5MM1001M0A | Converter, HD-CVI, HD-TVI, AHD, Composite to HDMI, VGA, Composite, Indoor |
| MAM-5MM2203M0A | Converter, 4K HD-TVI, HD-CVI, AHD, auf HDMI, Indoor 12VDC |
| TVA-1201TRA | UTP Transmitter+Receiver, UTP, BNC, 250m, Indoor, HD TVI CVI, AHD, CVBS, Active, 12V |
| TVA-1201TRP | UTP Transmitter+Receiver, UTP, BNC, 150m, Indoor, HD TVI CVI, AHD, CVBS, Passive |

Recommendations

Services

| LACKIERUNG Grundkosten | Painting basic costs once per RAL color and delivery lot |
| --- | --- |
| LACKIERUNG PTZ oder Panoramakamera | Extra charge for special paint PTZ dome or panoramic camera in RAL color |
| Vorkonfiguration analoge Kamera | Preconfiguration 1x analog camera Function test, FW update |