Specifications text

Article number

242363

Type

eneo NSUR-1008M0A

Short description

Gigabit Switch Rack, Unmanaged, 8x PoE, 2xSFP, max. 30W each Port, integrated Power Supply max. 150W

Main Features

8 RJ45 ports with 10/100/1000 Mbps

2 Gigabit SFP fiber ports for uplink

8 PoE+ ports, up to 30W per port

Total PoE power budget: 130 Watts

Supports IEEE 802.3af/at standard

Store-and-forward switching mode

MAC address auto-learning & aging

Switching capacity: 20 Gbps backplane

Forwarding rate up to 14.88 Mpps

Jumbo frame support up to 10K bytes

Sturdy metal case, 271×181×44mm

Operating temperature: 0-45°C

Other Features (Specifications)

| General |  |
| --- | --- |
| Fan | No |
| Management | No |
| Number of PoE ports | 8 |
| Design | Desktop |
| Dimensions (WxHxD) (extended) | 44.5x271x181.5mm |
| Application environment | inside |
| ESD protection | IEC61000-4-2, IEC61000-4-5 |
| Weight | 0.6 kg |
| Type | Unmanaged |
| Power consumption | 150 W |
| Mounting type | 19" Rack Installation, Desktop |
| PoE Budget (Total) | 130 W |
| Power over Ethernet | IEEE 802.3af, IEEE 802.3at |
| Power supply | 100-240VAC |
| Temperature Range (Operating) | 0°C ~ +45°C,@ Luftfeuchtigkeit 10%-90% (nicht kondensierend) |
| Interfaces / Inputs/ Outputs |  |
| Ethernet interface | 1000Base-T, 100Base-TX, 10Base-T |
| Ports | 1000base-T |
| Parts supplied |  |
| Scope of delivery | manual,19\" rack mount angle,power cord |
| Network |  |
| Eneo _netzwerkstandards | IEEE 802.3 10BaseT, IEEE 802.3u 100BaseTX |
| Internal Switching Capacity (Gbit) | 20 Gbps |
| Internal Switching Capacity (MP per second) | 14.88 Mpps |
| Product labeling: |  |
| Manufacturer order number | 242363 |
| Series | eneo Switch |
| Brand | eneo |

Additional Productinformation

Related products

| eneo NSFP-20000DLC | SFP Module, 1.25 Gbps, 20KM, Duplex LC, Tx/Rx at 1310 nm, hot-plug, SFP MSA-compliant, Digital Monitoring |
| --- | --- |
| eneo NSFP-20000SLC | SFP Modul Pair, 1.25 Gbps, 20 km, Simplex LC, Tx/Rx at 1310/1550nm, SFP MSA-compliant, Digital Monitoring |
| eneo NSFP-20000SSC | SFP Modul Pair, 1,25Gbps, 20KM, Simplex SC, Tx/Rx at 1310/1550nm, SFP MSA-compliant, Digital Monitoring |
| eneo NSFP-3000SLC | SFP Modul Pair, 1,25Gbps, 3KM, Simplex LC, Tx/Rx at 1310/1550nm, SFP MSA-compliant, Digital Monitoring |
| eneo NSFP-3000SSC | SFP Modul Pair, 1,25Gbps, 3KM, Simplex SC, Tx/Rx at 1310/1550nm, SFP MSA-compliant, Digital Monitoring |
| eneo SFP-0850M10-10G | SFP Modul, 10Gbps, 300m, Multimode, 850nm, Duplex LC Connector, -5°~+70°C |
| eneo SFP-0850M40 | SFP Modul, 1,25Gbps, 550m, Multimode, 850nm, Duplex LC Connector, -40° ~ +70°C |
| eneo SFP-0850MM10 | SFP Modul, 155Mbps , 2km, Multimode, 850nm, Duplex LC Connector, 0°~+70°C |
| eneo SFP-1310M40 | SFP Modul, 1.25Gbps, 550m, Multimode, 1310nm, Duplex LC Connector, -40°~+75°C |
| eneo SFP-1310S10 | SFP Modul, 1.25Gbps, 20km, Singlemode, 1310nm, Duplex LC Connector, 0°~+70°C |
| eneo SFP-1310S10-10G | SFP Modul, 10Gbps, 10km, Singlemode, 1310nm, Duplex LC Connector, -5°~+70°C |

Recommendations

Services