Specifications text

Article number

242364

Type

eneo NSUR-1016M0A

Short description

Gigabit Switch Rack, Unmanaged, 16x PoE, 2xSFP, max. 30W each Port, integrated Power Supply max. 250W

Main Features

16 RJ45 ports with 10/100/1000 Mbps

2 Gigabit SFP ports for fiber uplink

16 PoE+ ports, up to 30W per port

Total PoE budget: up to 225 Watts

Supports IEEE 802.3af/at/az standards

Non-blocking wire-speed on all ports

8K MAC address table supported

Forwarding rate: up to 26.784 Mpps

Switching capacity: 36 Gbps backplane

Supports Energy Efficient Ethernet (802.3az)

Rugged 19" metal case, 440×190×44 mm

Operating temperature: 0–45°C

Other Features (Specifications)

| General |  |
| --- | --- |
| Fan | Yes |
| Number of PoE ports | 16 |
| Design | Desktop |
| Dimensions (WxHxD) (extended) | 44,5x440x190mm |
| ESD protection | IEC61000-4-2, IEC61000-4-5 |
| Weight | 1.55 kg |
| Type | Unmanaged |
| Mounting type | 19" Rack Installation, Desktop |
| PoE Budget (Total) | 225 W |
| Power over Ethernet | IEEE 802.3af, IEEE 802.3at |
| Temperature Range (Operating) | 0°C ~ +45°C,@ Luftfeuchtigkeit 10%-90% (nicht kondensierend) |
| Interfaces / Inputs/ Outputs |  |
| Ethernet interface | 1000Base-T, 100Base-TX, 10Base-T |
| Parts supplied |  |
| Scope of delivery | manual,power cord,19\" rack mount angle |
| Network |  |
| Eneo _netzwerkstandards | IEEE 802.3 10BaseT, IEEE 802.3u 100BaseTX, IEEE 802.3x Flow Control |
| Internal Switching Capacity (Gbit) | 36 Gbps |
| Product labeling: |  |
| EAN/GTIN | 4260229841563 |
| Manufacturer order number | 242364 |
| Series | eneo Switch |
| Brand | eneo |

Additional Productinformation

Related products

| eneo NSFP-00100RJ45 | SFP module, transceiver, RJ45 connection, Cat.6A, supports 10G/5G/2.5 G/1G Ethernet up to 50m range, CAT6a, hot-pluggable |
| --- | --- |
| eneo NSFP-00300DLC-Multimode | SFP module, transceiver, Multi Mode, 1250Mbps, up to 550m distance, Duplex LC optical connector |
| eneo NSFP-20000DLC | SFP Module, 1.25 Gbps, 20KM, Duplex LC, Tx/Rx at 1310 nm, hot-plug, SFP MSA-compliant, Digital Monitoring |
| eneo NSFP-20000SLC | SFP Modul Pair, 1.25 Gbps, 20 km, Simplex LC, Tx/Rx at 1310/1550nm, SFP MSA-compliant, Digital Monitoring |
| eneo NSFP-20000SSC | SFP Modul Pair, 1,25Gbps, 20KM, Simplex SC, Tx/Rx at 1310/1550nm, SFP MSA-compliant, Digital Monitoring |
| eneo NSFP-3000SLC | SFP Modul Pair, 1,25Gbps, 3KM, Simplex LC, Tx/Rx at 1310/1550nm, SFP MSA-compliant, Digital Monitoring |
| eneo NSFP-3000SSC | SFP Modul Pair, 1,25Gbps, 3KM, Simplex SC, Tx/Rx at 1310/1550nm, SFP MSA-compliant, Digital Monitoring |
| eneo SFP-0850M10-10G | SFP Modul, 10Gbps, 300m, Multimode, 850nm, Duplex LC Connector, -5°~+70°C |
| eneo SFP-0850M40 | SFP Modul, 1,25Gbps, 550m, Multimode, 850nm, Duplex LC Connector, -40° ~ +70°C |
| eneo SFP-0850MM10 | SFP Modul, 155Mbps , 2km, Multimode, 850nm, Duplex LC Connector, 0°~+70°C |
| eneo SFP-1310M40 | SFP Modul, 1.25Gbps, 550m, Multimode, 1310nm, Duplex LC Connector, -40°~+75°C |
| eneo SFP-1310S10-10G | SFP Modul, 10Gbps, 10km, Singlemode, 1310nm, Duplex LC Connector, -5°~+70°C |

Recommendations

Services