Specifications text

Article number

242365

Type

NSUR-1024M0A

Short description

Gigabit Switch Rack, Unmanaged, 24x PoE, 2xSFP, max. 30W each Port, integrated Power Supply max. 400W

Main Features

24 RJ45 ports with 10/100/1000 Mbps

2 Gigabit SFP ports for fiber uplink

24 PoE+ ports, up to 30W per port

Total PoE power budget: 400 Watts

Supports IEEE 802.3af/at/az standards

Non-blocking wire-speed on all ports

8K MAC address table supported

Forwarding rate: up to 38.688 Mpps

Switching capacity: 52 Gbps backplane

Supports Energy Efficient Ethernet (802.3az)

Rugged 19" metal case, 440×290×44 mm

Operating temperature: 0–45 °C

Other Features (Specifications)

| Dimensions (HxWxD) | 44,5x440x290mm |
| --- | --- |
| Type | Unmanaged |
| Series | eneo Switch |
| Mounting | 19" rack, desktop |
| Blower | no |
| Ethernet interface | 1000Base-T, 100Base-TX, 10Base-T |
| VLAN | no |
| Ports | 24x 10/100/1000Base-TX, 2x 1000Mbps SFP |
| PoE ports (number) | 24 |
| Management | no |
| Model | Desktop |
| Supply voltage | 100-240VAC |
| Power over Ethernet | IEEE 802.3af, IEEE 802.3at |
| PoE budget | 380 W |
| Operating temperature | 0°C ~ +45°C, @ Humidity 10%-90% (non-condensing) |
| Weight | 3.5kg |
| Manufacturer order number | 242365 |
| Brand | eneo |

Additional Productinformation

Related products

| NSFP-20000DLC | SFP Module, 1.25 Gbps, 20KM, Duplex LC, Tx/Rx at 1310 nm, hot-plug, SFP MSA-compliant, Digital Monitoring |
| --- | --- |
| NSFP-20000SLC | SFP Modul Pair, 1.25 Gbps, 20 km, Duplex LC, Tx/Rx at 1310/1550nm, SFP MSA-compliant, Digital Monitoring |
| NSFP-20000SSC | SFP Modul Pair, 1,25Gbps, 20KM, Simplex SC, Tx/Rx at 1310/1550nm, SFP MSA-compliant, Digital Monitoring |
| NSFP-3000SLC | SFP Modul Pair, 1,25Gbps, 3KM, Duplex LC, Tx/Rx at 1310/1550nm, SFP MSA-compliant, Digital Monitoring |
| NSFP-3000SSC | SFP Modul Pair, 1,25Gbps, 3KM, Simplex SC, Tx/Rx at 1310/1550nm, SFP MSA-compliant, Digital Monitoring |
| SFP-0850M10 | SFP Modul, 1.25Gbps, 550m, Multimode, 850nm, Duplex LC Connector, 0°~+70°C |
| SFP-0850M10-10G | SFP Modul, 10Gbps, 300m, Multimode, 850nm, Duplex LC Connector, -5°~+70°C |
| SFP-0850M40 | SFP Modul, 1,25Gbps, 550m, Multimode, 850nm, Duplex LC Connector, -40° ~ +70°C |
| SFP-0850MM10 | SFP Modul, 155Mbps , 2km, Multimode, 850nm, Duplex LC Connector, 0°~+70°C |
| SFP-1310M40 | SFP Modul, 1.25Gbps, 550m, Multimode, 1310nm, Duplex LC Connector, -40°~+75°C |
| SFP-1310S10 | SFP Modul, 1.25Gbps, 20km, Singlemode, 1310nm, Duplex LC Connector, 0°~+70°C |
| SFP-1310S10-10G | SFP Modul, 10Gbps, 10km, Singlemode, 1310nm, Duplex LC Connector, -5°~+70°C |
| SFP-1310S40 | SFP Modul, 1.25Gbps, 20km, Singlemode, 1310nm, Duplex LC Connector -40°~+75°C |

Recommendations

Services