Specifications text

Article number

242366

Type

NSMR-2008M0A

Short description

Gigabit Switch Rack, Managed, 8x PoE, 2xSFP, max. 30W each Port, internal Power Supply max. 150W

Main Features

8 RJ45 ports with 10/100/1000 Mbps

2 Gigabit SFP ports for fiber optic uplink

8 PoE+ ports, each with up to 30W power

Total PoE power: up to 130 watts

Supports IEEE 802.3af/at/az standards

Supports MAC address auto-learning and auto-aging

Supports QoS, port mirroring, and link aggregation protocol

PoE watchdog automatically detects and restarts PD devices

Supports energy-efficient Ethernet (802.3az)

Robust 19" metal housing, 271×182×45 mm

Operating temperature: 0-45°C

Other Features (Specifications)

| Multicast | Yes |
| --- | --- |
| Fan | No |
| Management | Yes |
| VLAN | Yes |
| Number of PoE ports | 8 |
| Design | Desktop |
| Dimensions (WxHxD) (extended) | 44,5x271x181,5mm |
| ESD protection | IEC61000-4-2, IEC61000-4-5 |
| Weight | 1.35 kg |
| Scope of delivery | manual,power cord |
| Eneo _netzwerkstandards | IEEE 802.1p QoS, IEEE 802.3 10BaseT, IEEE 802.3u 100BaseTX, IEEE 802.3x Flow Control |
| QoS | DSCP Priority, QoS, WRR |
| Security | Password, Port Isolation |
| Internal Switching Capacity (Gbit) | 20 Gbps |
| Internal Switching Capacity (MP per second) | 14.88 Mpps |
| Ethernet interface | 1000Base-T, 100Base-TX, 10Base-T |
| Type | Smart Switch |
| Mounting type | 19" Rack Installation, Desktop |
| Manufacturer order number | 242366 |
| PoE Budget (Total) | 130 W |
| Power over Ethernet | IEEE 802.3af, IEEE 802.3at |
| Series | eneo Switch |
| Temperature Range (Operating) | 0°C ~ +45°C,@ Luftfeuchtigkeit 10%-90% (nicht kondensierend) |
| Brand | eneo |

Additional Productinformation

Related products

| NSFP-20000DLC | SFP Module, 1.25 Gbps, 20KM, Duplex LC, Tx/Rx at 1310 nm, hot-plug, SFP MSA-compliant, Digital Monitoring |
| --- | --- |
| NSFP-20000SLC | SFP Modul Pair, 1.25 Gbps, 20 km, Simplex LC, Tx/Rx at 1310/1550nm, SFP MSA-compliant, Digital Monitoring |
| NSFP-20000SSC | SFP Modul Pair, 1,25Gbps, 20KM, Simplex SC, Tx/Rx at 1310/1550nm, SFP MSA-compliant, Digital Monitoring |
| NSFP-3000SLC | SFP Modul Pair, 1,25Gbps, 3KM, Simplex LC, Tx/Rx at 1310/1550nm, SFP MSA-compliant, Digital Monitoring |
| NSFP-3000SSC | SFP Modul Pair, 1,25Gbps, 3KM, Simplex SC, Tx/Rx at 1310/1550nm, SFP MSA-compliant, Digital Monitoring |
| SFP-0850M10 | SFP Modul, 1.25Gbps, 550m, Multimode, 850nm, Duplex LC Connector, 0°~+70°C |
| SFP-0850M10-10G | SFP Modul, 10Gbps, 300m, Multimode, 850nm, Duplex LC Connector, -5°~+70°C |
| SFP-0850M40 | SFP Modul, 1,25Gbps, 550m, Multimode, 850nm, Duplex LC Connector, -40° ~ +70°C |
| SFP-0850MM10 | SFP Modul, 155Mbps , 2km, Multimode, 850nm, Duplex LC Connector, 0°~+70°C |
| SFP-1310M40 | SFP Modul, 1.25Gbps, 550m, Multimode, 1310nm, Duplex LC Connector, -40°~+75°C |
| SFP-1310S10 | SFP Modul, 1.25Gbps, 20km, Singlemode, 1310nm, Duplex LC Connector, 0°~+70°C |
| SFP-1310S10-10G | SFP Modul, 10Gbps, 10km, Singlemode, 1310nm, Duplex LC Connector, -5°~+70°C |
| SFP-1310S40 | SFP Modul, 1.25Gbps, 20km, Singlemode, 1310nm, Duplex LC Connector -40°~+75°C |

Recommendations

Services