Specifications text

Article number

242368

Type

NSMR-2024M0A

Short description

Gigabit Switch Rack, Managed, 24x PoE, 2xSFP, max. 30W each Port, integrated Power Supply max. 400W

Main Features

24 RJ45 ports with 10/100/1000 Mbps

2 Gigabit SFP ports for fiber optic uplink

24 PoE+ ports, each with up to 30W power

Total PoE power: up to 400 watts

Supports IEEE 802.3af/at/az standards

Noiseless operation thanks to fanless design

Supports automatic MAC address learning and aging

Supports QoS, port mirroring, and link aggregation protocol

PoE watchdog automatically detects and starts PD devices

Supports energy-efficient Ethernet (802.3az)

Robust 19" metal housing, 440×190×45 mm

Operating temperature range: 0-45°C

Other Features (Specifications)

| Dimensions (HxWxD) | 44,5x440x290mm |
| --- | --- |
| Type | Smart Switch |
| Series | eneo Switch |
| Mounting | 19" rack, desktop |
| Blower | no |
| Ethernet interface | 1000Base-T, 100Base-TX, 10Base-T |
| VLAN | yes |
| Ports | 24x 10/100/1000Mbps, 2x 1000Mbps SFP |
| PoE ports (number) | 24 |
| Management | yes |
| Model | Desktop |
| Supply voltage | 100-240VAC |
| Power over Ethernet | IEEE 802.3af, IEEE 802.3at |
| PoE budget | 400 W |
| Operating temperature | 0°C ~ +45°C, @ Humidity 10%-90% (non-condensing) |
| Weight | 3,15 kg |
| Manufacturer order number | 242368 |
| Brand | eneo |

Additional Productinformation

Related products

| NSFP-20000DLC | SFP Module, 1.25 Gbps, 20KM, Duplex LC, Tx/Rx at 1310 nm, hot-plug, SFP MSA-compliant, Digital Monitoring |
| --- | --- |
| NSFP-20000SLC | SFP Modul Pair, 1.25 Gbps, 20 km, Simplex LC, Tx/Rx at 1310/1550nm, SFP MSA-compliant, Digital Monitoring |
| NSFP-20000SSC | SFP Modul Pair, 1,25Gbps, 20KM, Simplex SC, Tx/Rx at 1310/1550nm, SFP MSA-compliant, Digital Monitoring |
| NSFP-3000SLC | SFP Modul Pair, 1,25Gbps, 3KM, Simplex LC, Tx/Rx at 1310/1550nm, SFP MSA-compliant, Digital Monitoring |
| NSFP-3000SSC | SFP Modul Pair, 1,25Gbps, 3KM, Simplex SC, Tx/Rx at 1310/1550nm, SFP MSA-compliant, Digital Monitoring |
| SFP-0850M10 | SFP Modul, 1.25Gbps, 550m, Multimode, 850nm, Duplex LC Connector, 0°~+70°C |
| SFP-0850M10-10G | SFP Modul, 10Gbps, 300m, Multimode, 850nm, Duplex LC Connector, -5°~+70°C |
| SFP-0850M40 | SFP Modul, 1,25Gbps, 550m, Multimode, 850nm, Duplex LC Connector, -40° ~ +70°C |
| SFP-0850MM10 | SFP Modul, 155Mbps , 2km, Multimode, 850nm, Duplex LC Connector, 0°~+70°C |
| SFP-1310M40 | SFP Modul, 1.25Gbps, 550m, Multimode, 1310nm, Duplex LC Connector, -40°~+75°C |
| SFP-1310S10 | SFP Modul, 1.25Gbps, 20km, Singlemode, 1310nm, Duplex LC Connector, 0°~+70°C |
| SFP-1310S10-10G | SFP Modul, 10Gbps, 10km, Singlemode, 1310nm, Duplex LC Connector, -5°~+70°C |
| SFP-1310S40 | SFP Modul, 1.25Gbps, 20km, Singlemode, 1310nm, Duplex LC Connector -40°~+75°C |

Recommendations

Services