Specifications text

Article number

242370

Type

eneo NSUI-3008M0A

Short description

Gigabit Switch Industrial, Unmanaged, 8x PoE, 2xSFP, 30W each Port, -40°C~75°C, 44~57 VDC, w/o power supply

Main Features

8 RJ45 ports with 10/100/1000 Mbps

2 Gigabit SFP port for fiber optic uplink

8 PoE+ ports, each with up to 30W power

Total PoE power: up to 222 watts,

depending on the connection

Supports IEEE 802.3af/at standards

High EMC protection level

Plug-and-play, no configuration, easy to use.

Redundant power supply 44-57VDC

Robust IP30 housing, DIN-RAIL or wall mounting

Operating temperature range: -40-75°C

Power supply not included

Other Features (Specifications)

| General |  |
| --- | --- |
| Fan | No |
| Number of PoE ports | 8 |
| Design | DIN Rail Module |
| Dimensions (WxHxD) (extended) | 50x149,2x122mm |
| ESD protection | IEC61000-4-2, IEC61000-4-5 |
| Weight | 1.168 kg |
| Type | Unmanaged |
| Power consumption | 18 W |
| Mounting type | DIN Rail, Wall |
| PoE Budget (Total) | 222 W |
| Power over Ethernet | IEEE 802.3af, IEEE 802.3at |
| Power supply | 44-57VDC |
| Temperature Range (Operating) | -40°C ~ +75°C,@ Luftfeuchtigkeit 10%-90% (nicht kondensierend) |
| Notes |  |
| Remark | No power supply included. A suitable,optionally available power supply must be used to achieve the full specified PoE budget value. |
| Interfaces / Inputs/ Outputs |  |
| Ethernet interface | 1000Base-T, 100Base-TX, 10Base-T |
| Parts supplied |  |
| Scope of delivery | manual,1x DIN Rail |
| Network |  |
| Eneo _netzwerkstandards | IEEE 802.3 10BaseT, IEEE 802.3u 100BaseTX, IEEE 802.3x Flow Control |
| Internal Switching Capacity (Gbit) | 20 Gbps |
| Internal Switching Capacity (MP per second) | 14.88 Mpps |
| Product labeling: |  |
| EAN/GTIN | 4260229841624 |
| Manufacturer order number | 242370 |
| Series | eneo Switch |
| Brand | eneo |

Additional Productinformation

Related products

| eneo NSFP-00100RJ45 | SFP module, transceiver, RJ45 connection, Cat.6A, supports 10G/5G/2.5 G/1G Ethernet up to 50m range, CAT6a, hot-pluggable |
| --- | --- |
| eneo NSFP-00300DLC-Multimode | SFP module, transceiver, Multi Mode, 1250Mbps, up to 550m distance, Duplex LC optical connector |
| eneo NSFP-20000DLC | SFP Module, 1.25 Gbps, 20KM, Duplex LC, Tx/Rx at 1310 nm, hot-plug, SFP MSA-compliant, Digital Monitoring |
| eneo NSFP-20000SLC | SFP Modul Pair, 1.25 Gbps, 20 km, Simplex LC, Tx/Rx at 1310/1550nm, SFP MSA-compliant, Digital Monitoring |
| eneo NSFP-20000SSC | SFP Modul Pair, 1,25Gbps, 20KM, Simplex SC, Tx/Rx at 1310/1550nm, SFP MSA-compliant, Digital Monitoring |
| eneo NSFP-3000SLC | SFP Modul Pair, 1,25Gbps, 3KM, Simplex LC, Tx/Rx at 1310/1550nm, SFP MSA-compliant, Digital Monitoring |
| eneo NSFP-3000SSC | SFP Modul Pair, 1,25Gbps, 3KM, Simplex SC, Tx/Rx at 1310/1550nm, SFP MSA-compliant, Digital Monitoring |
| eneo SFP-0850M10-10G | SFP Modul, 10Gbps, 300m, Multimode, 850nm, Duplex LC Connector, -5°~+70°C |
| eneo SFP-0850M40 | SFP Modul, 1,25Gbps, 550m, Multimode, 850nm, Duplex LC Connector, -40° ~ +70°C |
| eneo SFP-0850MM10 | SFP Modul, 155Mbps , 2km, Multimode, 850nm, Duplex LC Connector, 0°~+70°C |
| eneo SFP-1310M40 | SFP Modul, 1.25Gbps, 550m, Multimode, 1310nm, Duplex LC Connector, -40°~+75°C |
| eneo SFP-1310S10-10G | SFP Modul, 10Gbps, 10km, Singlemode, 1310nm, Duplex LC Connector, -5°~+70°C |

Recommendations

| jetrics NDR-240-48 DIN RAIL | Power Supply, 230VAC / 48VDC, max. 240W, for DIN rail mounting |
| --- | --- |
| jetrics WDR-480-48 Industrial Power Supply | Input 230VAC, Output 48VDC, for DIN RAIL TS-35/7.5 or 15 Operating temperature from -30 °C to +70 °C |

Services